3GPP TSG-SA WG1 Meeting #108	S1-244033

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	KDDI, NTT DOCOMO

pCR Title:	Use case on efficient USIM authentication

Draft Spec:	TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Kenichi Yamamoto <kc-yamamoto@kddi-std.jp>
Yusuke Nakano <you-nakano@kddi.com>

Abstract: This document provides a new use case on efficient USIM authentication.

1. Introduction

This contribution proposes a new use case on efficient USIM authentication.

2. Reason for Change

TR 22.870 v0.0.0 does not yet contain any use case that covers efficient USIM authentication.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

# ***1st Changes ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 33.203: "Access security for IP-based services".

# ***2nd Changes (All new texts)***

5.x	Use case on efficient USIM authentication

5.x.1	Description

If a UE subscribes IMS service, it sends a USIM authentication request to EPC/5GC. Following that, the UE sends a USIM authentication request to the IMS. This means the HSS/UDM must process same authentication twice, leading extra signaling.

When a country deploying the communication service is broken down due to IMS-related / IMS-affected failures, massive subscribers de-register network and then attempt to re-register to the network in cycles. This can cause unexpected overloads, network congestion and accelerating the signalling storm in the network, ultimately take a longer time to recover the communication services.

To mitigate IMS-related / IMS-affected failures, signaling optimization is crucial.

In this use case, it is assumed that an efficient USIM authentication mechanism for IMS and 6G system that minimizes signalling overload can be provided.

5.x.2	Pre-conditions

User A and Use B subscribe the communication services of Operator A.

Operator A deploys an efficient USIM authentication mechanism.

5.x.3	Service Flows

- User A turns on his mobile phone.

- User A’s mobile phone sends a registration request to Operator A’s network.

- Operator A’s network activates a single USIM authentication mechanism for 6G network and IMS.

- User A’s mobile phone completes the USIM authentication for 6G network and IMS service.

- User A makes a phone call to user B and start to communicate.

5.x.4	Post-conditions

User A enjoys a communication service of Operator A.

5.x.5	Existing features partly or fully covering the use case functionality

GPRS-IMS-Bundled Authentication (GIBA) has been specified in the annex of TS 33.203 [x]. However, the annex documents an interim security solution for early IMS implementations that are not fully compliant with the IMS security architecture specified in the main body of TS 33.203 [y].

5.x.6	Potential New Requirements needed to support the use case

[PR.5.x.6-1] The 6G system shall support an efficient means for USIM authentication for IMS and 6G system that minimize signalling overload.

# *** End of Changes ***