3GPP TSG-SA WG1 Meeting #108 	S1-244039

Orlando, Florida, USA, 18-22 November 2024

Source:	SK Telecom

pCR Title:	Use case on Robust and resilient Core network

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	DongJin Lee <dongjin@sk.com>

SeongJun Lee <seoul.lee@sk.com>

Abstract: This document provides a new use case on Robust and resilient Core.

1. Introduction

This contribution proposes a new use case on Robust and resilient Core network using in-depth tracing and logging of abnormal packets.

2. Reason for Change

TR 22.870 v0.0.0 does not yet contain any use case that covers Robust and resilient Core network using in-depth tracing and logging of abnormal packets.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

# *** First Changes ***

5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

5.x	Use case on In-depth Tracing/Logging for Abnormal Packets

5.x.1	Description

To simplify network systems and enhance efficiency, combined function nodes are introduced, performing multiple roles within a single system to support multiple generations or similar functionalities (e.g., AUSF/UDM, MME/AMF, SMF/SGW-C/PGW-C).

In virtualized cloud-native systems, identifying the actual logical or physical path through which packets are processed, including specific services or sub-functions within network functions (NFs), is challenging. Furthermore, based on the outcomes and decisions of intra- or inter-procedures within NFs, pinpointing the sub-functions responsible for abnormal behaviors or system failures is difficult, leading to significant time consumption in troubleshooting.

Consequently, it is imperative to implement comprehensive tracing and logging mechanisms for abnormal packets that may result in network malfunctions. A near real-time monitoring and analysis, including in-network computing and in-band network telemetry, is crucial for assessing packet delivery status and events related to both intra- and inter-procedures of network functions. AI-assisted functions, either new or existing, will be able to leverage these tracing and logging capabilities to ensure a robust and resilient network.

5.x.2	Pre-conditions

User A subscribe the communication services of Operator A.

System A is used to provide network services of Operator A.

5.x.3	Service Flows

- User A performs registration to use the services of Operator A.

- User A performs additional call processing procedures and sends packets (signalling/data) to Operator A's network system.

- System A tracks and stores packets per unit function within and outside the system for a certain period.

- System A detects abnormal operations within and outside the system during call processing (e.g., rebooting).

- System A sends previously transmitted packets to a separate collector for analysis.

5.x.4	Post-conditions

It is possible to accurately infer the points of failure within and outside each network system A.

5.x.5	Existing features partly or fully covering the use case functionality

Currently, subscriber-level trace is supported. However, there is no method to verify the message exchanges between (sub)functional units within the system.

5.x.6	Potential New Requirements needed to support the use case

[PR.5.x.6-1] The 6G system shall support monitoring mechanism of NF's intra- and inter-procedures within each system to minimize performance degradation of key operations.

[PR.5.x.6-2] The 6G system shall support in-depth tracing and logging mechanisms for control and data plane packets that cause abnormal operations (e.g., reinitialization of sub-functional units).

# *** End of Changes ***