3GPP TSG SA WG 1 Meeting #108 	S1-244084

Orlando, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	NTT DOCOMO

pCR Title:	Pseudo-CR on Fault analysis and reporting for NTN communications

Draft Spec:	3GPP TR 22.870 v0.0.0

Agenda item:	8.1.3

Document for:	Approval

Contact:	azem@docomolab-euro.com

Abstract: <provide a short description of the content>

1. Introduction

pCR on Fault Analysis and Reporting for NTN communications.

2. Reason for Change

MNOs use various monitoring mechanisms to track the network activities to diagnose and analyse faults/errors in their system.

Such mechanism can be used in the 3GPP system that supports and provides satellite access. This will help to enhance the performance by monitoring the communication/traffic which involve the NTN system, and report the communications failures/errors to the MNOs and to external monitoring (third party) systems.

3. Conclusions

Define the requirements for 6G system to support fault analysis and reporting for NTN communications.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 22.261: " Service requirements for the 5G system; Stage 1".

* * * Next Change * * * *

# 7	Ubiquitous Connectivity

Editor's Note: "Ubiquitous Connectivity" is intended to enhance connectivity to bridge the digital divide and enhance user experience, to address presently uncovered or scarcely covered areas, particularly rural, remote and sparsely populated areas, and indoor connectivity

## 7.x	Use Case x: Fault Analysis and Reporting for NTN communications

### 7.x.1	Description

The MNOs use various mechanisms to track network activity for network surveillance and troubleshooting to diagnose and analyse faults in their system. Such fault analysis mechanisms can be also used for monitoring network activities involving NTN system, where this monitoring system is fully under the control of the MNOs.

In scenarios when communication failure occurs, the failure related information that held at the UE can be utilized by MNOs to analyse the cause of communication failure and accordingly enhance the system performance and improve user experience. For communicating such information between UE and core network over the satellites, new capabilities will be required in 6G system to exchange the data with minimum impact on the real-time traffic and consider the security aspects in case the reports sent towards the external (third party) monitoring system.

### 7.x.2	Pre-conditions

- Operator A’s network supports satellite access and provide automated satellite fault detection and analysis capabilities.

- a user subscribed to operator A’s network and uses UE1, which supports satellite access and supports satellite fault information handling.

### 7.x.3	Service flows

1. UE1 connects to the operator A network via Sat A.

2. a communication failure/error occurs, and the UE1 stores this information locally.

3. UE1 sends the fault information to the Sat A when the connection is available again.

4. Sat A has the capability of collecting, storing and reporting the failure communication information to the core network, it forwards the received fault information to the core network.

5. upon the reception of the information at the core network, it can be used for further analysis and can be forwarded and used by a trusted third party or authorized users.

Figure 7.x.1-1: Example of fault analysis for NTN

### 7.x.4	Post conditions

The operator A can optimize the performance by using the reported failure information.

### 7.x.5	Existing features partly or fully covering the use cases functionality

clause 6.46.2 in TS 22.261[x]:

Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite.

### 7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-001] 	Subject to operator’s policies, a 6G system with satellite access shall be able to support satellite fault analysis operations on-board for collecting, storing, and reporting of communication failure information received form a UE registered to the network.

[PR 7.x.6-002] 	A 6G system with satellite access shall be able to indicate to the UE whether the satellite fault analysis operations are supported for the communication.

[PR 7.x.6-003] 	A UE supporting satellite access shall indicate to the network the support of satellite fault analysis operations.

[PR 7.x.6-004] 	A 6G system with satellite access shall be able to indicate to an authorised third party whether satellite fault analysis operations are supported and applied for communication with the UE, and shall be able to provide the related communication failure information to that third party.

## [PR 7.x.6-005] 	A 6G system with satellite access shall be able to provide a mechanism to an authorised third party to configure the requested information.

* * * End of Change * * * *