3GPP TSG-SA WG1 Meeting #108 	S1-244086

Orlando, USA, 18-22 November 2024

Title:	New use case on Immersive Service Coordination

Agenda Item:	8.1.4

Source:	Nokia

Contact:	Hideaki Takahashi; hideaki dot takahashi at nokia dot com

Abstract: This paper proposes a new use case on immersive service coordination to offload one or more service processing from UE to the network.

---------- Use Case template ----------

## 8.X	Use case on Immersive Service Coordination

### 8.X.1	Description

In immersive communications, a typical use case is to optimally locate one or more software modules comprised of the whole application service into computing instances present over the network including user devices (i.e., UE). Depending on dynamic compute and network capabilities and availability, a particular software module could run in the UE or be “offloaded” to a compute instance on the network. The motivations of such a use case are as follows:

-	By offloading a software module to the computing instance located in the network, the UE could better reduce its resource consumption (e.g., energy and local processing).

-	As the computing resource in the network has more resources than UE typically, the offloaded software module could run more efficiently, resulting in enhanced Quality of Service.

For instance, some XR applications use different modules such as ‘scene manager’ module, ‘object detection’ module, ‘rendering’ module, etc. With the consideration of the imaging technologies of live service migration, these modules could be designed to support relocation to a target server that is also designed to support offload. In this regard, the UE would be able to reduce its consumption by offloading a module to a compute site as long as the required QoS is retained.

Figure x.1.1 illustrates a conceptual workflow on how the software module is relocated and offloaded according to the dynamic statistics, e.g., UE power consumption, network latency, etc.

Figure x.1.1:	Immersive Service Coordination

Potential sustainability impacts of the use case

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11 UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) | - Reducing device energy consumption, by offloading one or more modules to compute sites in the network - Reducing the network energy consumption too by taking metrics of energy consumption into account. | - success of such use case may trigger more low cost end-user devices and hence as rebound effect generate more energy consumption overall if an additional personal IoT device and not a replacement device of the smartphone for end users |
|  | Material resources (UN SDG 11, 12) | - - Reducing complexity of end-user devices (and thus e-waste) by leveraging network/edge compute capabilities | - - success of such use case may trigger more low cost end-user devices and hence as rebound effect generate more material extraction to build these devices, as well as globally additional e-waste |
|  | Emissions (UN SDG 6, 7, 11, 12, 13, 14, 15) | - Enabling CO2 emission reduction in the overall system by means of reduced energy consumption, which sources of energy are reduced (network instead of UEs) and can be better managed towards low carbon. | - success of such use case may trigger more low cost end-user devices and hence as rebound effect generate more ghg emissions |
| Socio-economic sustainability aspects (UN SDGs 2, 3, 4, 5, 8, 9, 10, 11, 16 & 17 and indirectly 12) UN GDC “Closing Digital Divides and Accelerating SDG Progress” & “Expanding Digital Economy Inclusion” & “Fostering an Inclusive, Safe Digital Space”) |  |  |  |
|  | Health (UN SDG 3) | - - Offload enable lightweight devices that can thus be worn for a longer period | - effects of massive usage of wearables may have negative impact on health (EMF etc) or safety risks (be isolated from surrounding environment) |
|  | Inclusion & Equality (UN SDGs 11, 10, 4, 5 and indirectly 3, 16 & 17) | - offload enables low cost end devices more largely affordable by the population - Wearable devices – eg handsfree, lightweight – can also be used by a larger population base beyond some disabilities |  |
|  | Trustworthiness (UN SDGs 11 and indirectly 3 & 17) | - Offload to the network may provide various technical alternatives to local UE computing thus increasing the availability and resilience of the system towards service continuity | - privacy of data being offloaded (eg image/video of people’s face) has to be taken into consideration when offloading |
|  | Work & income (UN SDG 8 and indirectly 12) | - offload can enable longer battery time in operations, in particular for remote workers, thus increasing productivity |  |
|  | Infrastructure (UN SDG 9) | - - offload can benefit remote workers for many infrastructure sectors with longer battery times and lightweight, more affordable devices |  |
|  | TCO reduction (UN SDGs 8, 9 and 12) | - offload can benefit several sectors with more affordable devices, thus making some business cases of remote workers commercially viable | - extra cost for investment in end-user devices and subscriptions to access offload capabilities, which should be traded-off with productivity gains |

### 8.X.2	Pre-conditions

A Computing Service Provider (CoSP) is a provider to offer their computing resources, e.g. for immersive communications. MNO or the 3rd party can be a CoSP. The CoSP deploys one or more split rendering service instances (e.g., XR rendering, etc.) at various physical sites.

A user subscribes immersive application services (e.g., XR applications, etc.) provided by the MNO and/or the ASP by leveraging CoSP’s split rendering service instances at their physical sites.

In the initial configuration, some modules are processed at the UE (e.g., module #A and #B as in Fig. x.1.1), whereas the other is at a (central) computing site (module #C as in Fig. x.1.1), which is hosted by the CoSP.

In CoSP’s platform, while a split rendering service infrastructure is active, its computing resource status (e.g., CPU/memory load of the immersive service) and its energy consumption is measured, which is to be used as a metric of computing performance and (energy) cost. The MNO network is also capable of measuring network performance (e.g., latency between a UE and a computing instance) when requested for a given UE.

### 8.X.3	Service Flows

The following service flows are exemplified according to the conceptual workflow in Fig. x.1.1.

1.	The UE processing software modules #A and #B recognises that power consumption is significantly soared.

2.	The immersive application client at the UE requests MNO’s or ASP’s application server to offload these software modules from the UE, possibly indicating information related to the associated power consumption of these module at UE side.

3.	The ASP or MNO requests the CoSP to provide the network & energy metrics per split rendering service instance or site where the software modules #A and #B may be relocated.

4.	User’s application or ASP collects from CoSP, the computing resource status and energy consumption of the split rendering service instance or site as well as the network performance in relation to user’s UE. In case MNO plays a role of CoSP, the network performance and energy consumption is obtained from 6G network, whereas compute performance is obtained via a different layer than the one where 6G network encompasses.

5.	User’s application or ASP selects an optimal split rendering service instance to process the software module #B, taking into account the computing, energy and network metrics for each split rendering service instance or site obtained from CoSP . Based on the same information, and on the information provided by the UE, User’s application or ASP decides that software module #A should remain on the UE.

6.	The software module #B is relocated from the UE to the selected computing instance (the central site in Fig. x.1.1).

7.	Later, the application client or server recognises that RTT becomes too long between UE and the split rendering service instance processing software modules #B and #C, which hinders the processing of software module #B from satisfying QoS.

8.	The application client or server again collects the network performance and energy consumption of available computing instances and select an optimal split rendering service instance to process the software module #B fulfilling both computing, energy and network metrics.

9.	The software module #B is again relocated to the selected computing instance (the edge site in Fig. x.1.1).

### 8.X.4	Post-conditions

The immersive application client or server can relocate and offload one or more software modules in order to retain optimal QoS and user experience over dynamic compute, energy and network characteristics.

### 8.X.5	Existing features partly or fully covering the use case functionality

None.

### 8.X.6	Potential New Requirements needed to support the use case

[PR 8.X.6-1] Subject to operator’s policy and an authorized 3rd party and/or user consent, the 6G system shall be able to assist immersive applications in selecting a split rendering service instance or site which is optimal, including based on network, energy and compute metrics, and application’s networking and compute requirements.

[PR 8.X.6-2] The 6G network shall be able to receive information about split rendering service instances and sites for which its selection assistance applies (e.g., identification, addressing and capability information of the split rendering service instances and sites).

NOTE:	The capability information of the split rendering service instances and sites include information on which metrics exposure or determination methods (e.g., measurement result of metrics) the split rendering service instances and compute sites support towards the 6G Network.

[PR 8.X.6-3] The 6G network shall be able to determine, e.g., by measurements using the measurement results of metrics supported by the split rendering service instances and sites, the network metrics (e.g., latency) between a UE and each potential split rendering service instance or site.

[PR 8.X.6-4] The 6G network shall be able to expose network metrics (e.g., latency) between a UE and each potential split rendering service instance or site to an authorized 3rd party.