3GPP TSG SA WG 1 Meeting #108 	S1-244130

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	CATT

pCR Title:	Use Case on Coordinating Computing and Communication for XR Rendering

Draft Spec:	3GPP TR 22.870 v0.0.0

Agenda item:	8.1.7

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes a use case about 3GPP 6G system leading the collaboration of the computing and communication for XR rendering.

1. Introduction

The 5G system has already introduced specific technial solutions to support some AR/XR use cases, but limit to the 5G capabilities, the large-scale XR applications requesting large amount of data transmission and heavy computing workload are not well discussed and can be considered in 6G. This contribution proposes a use case about 6G system deeply involved in the coordination of the computing and communication for XR rendering.

2. Reason for Change

Introduce a new use case for the application of 6G system in XR application.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.870 v0.1.0

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ".

(https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf)

* * * Next Change * * * *

W	Other Use Cases

W. x	Use case on coordinating computing and communication for XR rendering

W.x.1	Description

The exploration of ultimate user experience in consumer products, and the digital and intelligent transformation in the industrial fields have promoted the widely use of computing-intensive applications such as AR/XR, cyber-physical systems, industry robots and etc. However, the balance between computing capabilities and the size and cost of the devices raises a big challenge for the deployment of the entire applications. The ITU-R report [x] points out several emerging technologies are being envisioned to address the challenges. One trend is to process data at the network edge close to the data source for real-time response, low data transport costs and energy efficiency by edge computing technologies, while another trend is to scale out device computing capability beyond its physical limitations by splitting computing workload over reachable computing resources. With the development of each technology trend, the independent control, management and orchestration of communication and the computing resources has been identified to bring out significant negative impact on the performance of the end-to-end solution.

The 6G system is expected to change the situation by coordinating the communication service and computing resource utilization in the stage of system architecture design. In this way, the 6G network can make the ubiquitous single-point computing resource connected and participate in the scheduling of various types of computing resources such as UE, MEC server, cloud server. For example, the 6G network can select appropriate computing resource based on service characteristics to achieve optimal resource usage. On the other hand, the data transmission for the computation can be more flexible and efficient with additional information (e.g. workload, available capability) about the selected computing resources.

The real-time rendering of 3D scenes in large-scale XR application is a typical case of computing-intensive application, which requires large amount of calculation to handle the complicated model and texture mapping. Sometime limited to the capability of hardware, it’s impossible for a single device to render the scene individually, and the distribution of the computing workload to other computing capable nodes can solve the issue. Sometimes the duplicated rendering of the same XR scene which is being requested by multiple users can be avoided if choose to execute the rendering in the cloud server. However, the XR application can have little information on the status of computing resources so that it’s difficult for the planned data transmission and computation to work together efficiently.

This use case illustrates how the 6G system enables better user experience of split XR rendering via the coordination of computing resources and communication resources.

Figure W.x.1-1: Coordinating Computing and Communication for XR rendering

W.x.2	Pre-conditions

Several computing resource nodes (e.g. MEC servers, cloud computing center) for XR services are deployed distributedly in different locations and have been enrolled in 6G core network. MEC server A, B and C are provisioned as a part of rendering pipelines to execute render engine, engine adaption, rendering acceleration for the rendering task. Cloud computing center is capable of all the functions for XR rendering.

XR Application Platform provides the render services to various XR applications.

Task Management Server is responsible for task analyse, sub-task planning, graphic composition and etc.

The UE supporting XR application is the subscriber of the 6G network. It has registered to 6G network and been authorized for the computing coordination service.

W.x.3	Service Flows

- Regarding XR application’s need, UE sends a request of XR rendering to XR Application Platform.

- XR application platform discomposes UE’s request to different tasks, and dispatches the render task to Task Management Server through IP network.

- Task Management Server analyzes the render task and plan the computing task based on the capability of computing resources. Then it sends the requests of computing coordination service and XR communication service to 6G core network, including the information of computing coordination service QoS(e.g. requested computing capabilities, response time) and communication service QoS (e.g. latency).

- 6G core network selects MEC server B and cloud computing center based on the requested QoS of computing coordination service, and the status information of all enrolled computing resources, and then feedback Task Management Server  about the coordination result (e.g. selected computing resources). Meanwhile, 6G core network establishes the communication paths between UE and MEC server B and cloud computing center to transmit data for the rendering based on the requested QoS.

- Task Management Server distributes the necessary data and associated information to all the selected computing resources via IP network.

- MEC server B and cloud computing center executes individual rendering sub-tasks, sync-up with Task Management Server about the progress of its sub-tasks and sync-up the status information (e.g. computing workload, congestion status) with 6G core network.

- The 6G core network detects the overload of MEC server B, and informs Task Management Server to replace MEC server B with MEC server C.

- Task Management Server collects the rendering graphics from MEC server B, MEC server C and Cloud center after the sub-tasks are finished and composes the XR image for XR Application Platform. Then XR Application Platform responds the rendered XR image to UE.

W.x.4	Post-conditions

The render task is efficiently completed with the collaboration of several edge computing servers under the coordination of 6G core network.

The rendered XR image is successfully provided to UE as expected.

W.x.5	Existing features partly or fully covering the use case functionality

None

W.x.6	Potential New Requirements needed to support the use case

[PR-W.x.6-1] The 6G network shall support mechanisms to manage computing resources e.g. enroll computing capability, maintain the computing information and etc.

[PR-W.x.6-2] The 6G network shall support mechanisms to collect the status information (e.g. computing workload, congestion information, available capability) of trusted computing resources on-demand or periodically.

[PR-W.x.6-3] The 6G network shall provide mechanisms to expose to trusted 3rd party the information of computing resources (e.g. computing capability, the location, allowed service type, status).

[PR-W.x.6-4] Subject to operator’s policy, application needs or both, the 6G network shall support the selection of computing resource(s) e.g. based on the request of computing capabilities, and support routing of data traffic between a UE and selected computing resource(s) based on required QoS.

[PR-W.x.6-5] Subject to operator’s policy, the 6G network shall support an efficient communication path, and modifying the path as needed e.g. based on the status of computing resources.

NOTE: the computing resources can be an edge server, a cloud server or a group of computing servers.