3GPP TSG- SA1 Meeting # 108	S1-244211

# Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

# Source:	China Telecom

# Title:	New WID on Multiple Access Technologies Supporting Different IMS Services

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Multiple Access Technologies Supporting Different IMS Services

Acronym:	MA_IMS

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

The types of IMS services nowadays are becoming increasingly diverse, with bunches of data services via IMS Data Channel in addition to audio and video, which may have differentiated requirements on QoS and data transmission. When network conditions change, some user traffic related to dedicated IMS services can be offloaded from 3GPP access to non-3GPP access path, while other types of IMS services remain on the 3GPP access path.

For instance, traffic of video and IMS data channel services can be offloaded to the Wi-Fi path, leaving audio traffic on 3GPP access network. Another similar case is also applicable to IMS data channel services (e.g. offloading IMS DC file transfer service to the Wi-Fi path with keeping the IMS DC real-time translation service on the 3GPP access path).

The existing requirement on multiple access technologies supporting traffic offloading does not explicitly cover different types of IMS services and/or different types of services over IMS Data Channel, e.g., audio, video, text and IMS DC file transfer, etc.

4	Objective

The objective of this WID is to clarify the requirement for supporting traffic offloading among different access technologies based on different types of IMS services and/or different types of services over IMS Data Channel.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Clarify the requirement for supporting multiple access technologies based on different types of IMS services and/or different types of services over IMS Data Channel. | TSG#106 (Dec. 2024) |  |

6	Work item Rapporteur(s)

Yuying Zhang, China Telecom, zhangyy45@chinatelecom.cn

7	Work item leadership

SA WG1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom