3GPP SA WG1 Meeting #108 	S1-244322

Orlando, FL, USA, 18 – 22nd  November 2024	(revision of S1-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable Priority Messaging Service for authorized third party user. |
| --- | --- |
| Source to WG: | Google, AT&T, SyncTechno Inc., III |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy (TBD) |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on miniWID of Priority Messaging Service in R20, which proposes to enable PMS (includign SMS and NIDD-based messaging) for third party user. All requirements specified in TS 22.153 related to MPS for messaging for authorized Service User using a UE or IoT device are assumed to be applicable to PMS for authorized third-party user, unless otherwise indicated in this miniWID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the following: - Enable PMS prioirty treatment for authorized third party user using UEs or IoT devices for SMS or NIDD-based messaging. - Add charging aspects for authrozied third party user via third party servcie provider. - Enable UAC for PMS for third party user using SMS, or NIDD-based messaging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No PMS support and UAC enhancement for authorithed third party user using SMS or NIDD-based Messaging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.22.2.1, 6.22.2.2, 6.22.2.3, 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** The first Change *************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G LAN-VN	5G LAN-Virtual Network

A/S	Actuator/Sensor

CPN	Customer Premises Network

eFMSS	Enhancement to Flexible Mobile Service Steering

eRG	Evolved Residential Gateway

eV2X	Enhanced V2X

FL	Federated Learning

FMSS	Flexible Mobile Service Steering

GEO	Geostationary satellite Earth Orbit

ICP	Internet Content Provider

ID	Identification

IMU	Inertial Measurement Unit

IOPS	Isolated E-UTRAN Operation for Public Safety

ISL	Inter-Satellite Link

LEO	Low-Earth Orbit

MBS	Metropolitan Beacon System

MCS	Mission Critical Services

MCX	Mission Critical X, with X = PTT or X = Video or X = Data

MEO	Medium-Earth Orbit

MIoT	Massive Internet of Things

MMTEL	Multimedia Telephony

MOCN	Multi-Operator Core Network

MPS	Multimedia Priority Service

MSGin5G	Message Service Within the 5G System

MSTP	Multiple Spanning Tree Protocol

NGSO	Non-Geostationary Satellite Orbit

NIDD	Non IP Data Delivery

NPN	Non-Public Network

PIN	Personal IoT Network

PMS	Priority Messaging Service

PRAS	Premises Radio Access Station

RSTP	Rapid Spanning Tree Protocol

RVAS	Roaming Value-Added Service

SEES	Service Exposure and Enablement Support

SNPN	Stand-alone Non-Public Network

S&F	Store and Forward

SST	Slice/Service Type

TBS	Terrestrial Beacon System

TTFF	Time To First Fix

UAV	Uncrewed Aerial Vehicle

UTC 	Coordinated Universal Time

XR	eXtended Reality

******************* 2nd Change*****************

#### 6.22.2.1	General

Based on operator policy, the 5G system shall be able to prevent UEs from accessing the network using relevant barring parameters that vary depending on Access Identity and Access Category. Access Identities are configured at the UE as listed in Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in one or more areas of the RAN.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the UE.

In the case of multiple core networks sharing the same RAN, the RAN shall be able to apply access control for the different core networks individually.

The unified access control framework shall be applicable both to UEs accessing the 5G CN using E-UTRA and to UEs accessing the 5G CN using NR.

The unified access control framework shall be applicable to UEs in RRC Idle, RRC Inactive, and RRC Connected at the time of initiating a new access attempt (e.g. new session request).

NOTE 1:	"new session request" in RRC Connected refers to events, e.g. new MMTEL voice or video session, sending of SMS (SMS over IP, or SMS over NAS), sending of NIDD-based message, sending of IMS registration related signalling, new PDU session establishment, existing PDU session modification, and service request to re-establish the user plane for an existing PDU session.

The 5G system shall support means by which the operator can define operator-defined Access Categories to be mutually exclusive.

NOTE 2:	Examples of criterion of operator-defined Access Categories are network slicing, application, and application server.

The unified access control framework shall be applicable to inbound roamers to a PLMN.

The serving PLMN should be able to provide the definition of operator-defined Access Categories to the UE.

******************* 3rd Change*****************

#### 6.22.2.2	Access identities

Table 6.22.2.2-1: Access Identities

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for Multimedia Priority Service (MPS) |
| 2 (NOTE 2) | UE is configured for Mission Critical Service (MCS). |
| 3 | UE for which Disaster Condition applies (note 4) |
| 4 (NOTE 5) | UE is configured for Priority Messaging services (PMS) |
| 5-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. |
| NOTE 1: Access Identity 1 is used by an authorized Service User’s Voice, Video, Data and Messaging communication using UEs configured for MPS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN, PLMNs equivalent to HPLMN, and visited PLMNs of the home country. Access Identity 1 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 2: Access Identity 2 is used by UEs configured for MCS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN or PLMNs equivalent to HPLMN and visited PLMNs of the home country. Access Identity 2 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose, the home country is defined as the country of the MCC part of the IMSI. NOTE 4: The configuration is valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN. See clause 6.31. NOTE 5: Access Identity 4 is used by an authorized third paty user’s Messaging communication using UEs configured for PMS for Messaging, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN, PLMNs equivalent to HPLMN, and visited PLMNs of the home country. Access Identity 4 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. |  |

Any number of these Access Identities may be barred at any one time.

******************* 4th Change*****************

#### 6.22.2.3	Access categories

Table 6.22.2.3-1: Access Categories

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice (NOTE 3) |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories (NOTE 4) |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling (NOTE 5) |
| 10 (NOTE 6) | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |
| NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. When a UE is configured for EAB, the UE is also configured for delay tolerant service. In case a UE is configured both for EAB and for EAB override, when upper layer indicates to override Access Category 1, then Access Category 1 is not applicable. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. NOTE 3: Includes Real-Time Text (RTT). NOTE 4: Includes IMS Messaging, NIDD-based Messaging. NOTE 5: Includes IMS registration related signalling, e.g. IMS initial registration, re-registration, and subscription refresh. NOTE 6: Applies to access of a NB-IoT-capable UEto a NB-IOT cell connected to 5GC when the UE is authorized to send exception data. |  |  |

Access Category 0 in Table 6.22.2.3-1shall not be barred, irrespective of Access Identities.

NOTE:	The network can control the amount of access attempts relating to Access Category 0 by controlling whether to send paging or not.

**************** 5th Change (All New Texts)**************

## 6.X	Priority Messaging Service for Third Party User

### 6.X.1	Description

Priority Messaging Service (PMS) is applicable in EPS and 5GS systems.

The PMS is for an authorized third-party user that relies on third-party’s application services for alerting urgent events, and uses UE or IoT device for SMS, or NIDD based messaging services. The PMS provides priority treatment to increase the probability of an authorized third-party user’s messaging communication being successful. Especially when network congestion is blocking access attempts for messaging or session establishment, PMS allows qualified and authorized third-party users for urgent events to obtain priority access to the next available radio channel on a priority basis before other UEs using other normal commercial messaging services.

The PMS includes network functions that fall into the same categories (service invocation and revocation, authentication and authorization, signaling invocation, messaging origination and termination, and security aspects) as MPS for messaging indicated in clause 9.4 of TS22.153 [x] which is based on MPS priority provided by operator’s network for Service Users. The main difference is that PMS is for providing priority treatment in operator’s network to an authorized third-party user using third party service provider’s service for alerting urgent events. Based on regional/national regulatory requirements, service level agreement with third party, and operator’s policy, the operator’s network provides differential priority treatment between MPS authorized Service User, and PMS authorized third party user.

NOTE 1:  PMS is applicable for 3GPP system with terrestrial access and satellite access.

### 6.X.2	Requirements

Subject to service level agreement with the third party and operator’s policy for PMS, the system shall support requirements of PMS including SMS, and NIDD-based messaging for authorized third-party user as specified in TS 22.153, related to MPS for messaging for authorized Service User using UE or IoT device, unless otherwise indicated.

Message origination and termination:

When PMS for Messaging is authorized and activated for a UE or IoT device using NIDD-based messaging service, the system shall provide priority treatment for all messages to/from the third-party application server.

NOTE 1:	The NIDD-based messaging platforms and inter system aspects are outside 3GPP scope.

Authentication and Authorization:

The 3GPP system shall support means to authenticate and authorise a UE or IoT device used by a third-party user when its third-party application requests PMS activation for the following cases:

-	a specific PMS subscribed UE or PMS subscribed IoT device,

-	a group of PMS subscribed UEs or PMS subscribed IoT devices,

-	a non-PMS subscribed UE or non-PMS subscribed IoT device,

-	a group of non-PMS subscribed UEs or non-PMS subscribed IoT devices.

NOTE 3: 	the third-party user’s PMS subscription information (e.g., credentials) is assumed not always associated with the UE’s/IoT device’s HPLMN subscription.

NOTE 4:	the PMS subscription related information is associated with the UE’s HPLMN subscription if available and is either stored in the UE and the HPLMN, or only in the HPLMN.  In the case that the PMS subscription is stored in the UE, the UE’s membership in a special access class [X] as per TS 22.011 [xx] is used for E-UTRAN access to the EPC and membership in Access Identity [X] as per TS 22.261 [yy] is used for 5GS (NR and E-UTRA access to the 5GC). In the case where the MPS subscription is stored only in the HPLMN (5GC), the HPLMN (5GC) can provide PMS indication during the UE registration.

Roaming:

The system shall support PMS when a UE or IoT device with an PMS subscription is roaming, within the home country or outside the home country) and the visited and home network support PMS, and roaming agreements are in place for PMS.

NOTE 6:	For a third-party user, the PMS subscription related information associated with the UE’s HPLMN subscription is used for roaming within the home country.

NOTE 7:	The HPLMN can configure the UE to discover and select a network supporting PMS when roaming outside of the home country.

Priority Treatment:

The system shall provide differential priority treatment to the invocation signalling in the network once the MPS for Messaging request for Service User and/or PMS for third party user is identified by the system.

The priority session treatment for a Service User shall be handled by the originating/terminating networks with higher priority than for a third-party user subject to regional/national regulatory requirements and operator’s policy.

The system shall support a means to request authorization of the PMS’s priority level for a third-party user by a third-party service operator.

NOTE 8: 	Assignment of priority levels for a third-party user by the operator is a matter of service level agreement with third party service provider and operator policy. Upon PMS invocation the calling third-party user's priority level is used to identify the priority to be used for the messaging.

Access Control:

The system shall support a means for PMS activation when other normal commertial messaging service is congested.

### 6.X.4	Charging Aspects

A network supporting PMS shall be capable of recording the following charging information, in addition to non-PMS information:

-	PMS invocation attempt and successful session set-up,

-	Originations and/or terminations Session on which PMS was used to gain access to resources,

-	Recording of PMS information, e.g., priority level,

-	PMS related events on which messaging service was used and by which authroized third-party user.

The system shall associate PMS related charging events with an authorized third-party service operator for its authorized third-party user using a UE or IoT device that may not have an PMS subscription.

NOTE 9:	In this case, the third-party service provider sponsors the PMS usage for their third-party users to the operator.

*******************End of Change*****************