3GPP TSG SA WG 1 Meeting #108 	S1-244331

Orlando, USA, 18-22 November 2024

Source:	Nokia

pCR Title:	Pseudo-CR on updates of terms to add definition of multi-orbit satellite access

Draft Spec:	3GPP TR 22.887-020

Agenda item:	7.3

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.20 in FS_5GSAT_Ph4 TR.

1. Introduction

This contribution proposes to introduce an existing definition of satellite access and add a new definition of multi-orbit satellite access.

2. Reason for Change

We need to be consistent with the existing definition of satellite access in TS 22.261. As multi-orbit satellite access is extensively used throughout the multi-orbit use cases, we need a clear definition for that, keeping in mind that SA2 has already specified in clause 5.4.11.2 of TS 23.501 and clause 4.13.2 of TS 23.401 to consider LEO, MEO, GEO and other satellite access as different RAT types for differentiation by core network.

In addition, some editorial changes are provided.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887-020.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

network entry completion: in the context of this study, it refers to any remaining steps after network entry initiation has been completed to allow full registration of the UE.

network entry initialization: in the context of this study, it refers to initial steps required in entering the network, sufficient to validate that the UE is a valid authorized device and exchanges enough information to allow the UE to connect to a network of the preferred satellite access type.

Editor’s Note:	the above two definitions are FFS.

Resilient Notification: in the context of this study, it refers to a highly reliable mechanism that delivers notifications directly to user devices, such as smartphones, when in poor conditions in satellite access.

satellite access: direct connectivity between the UE and the satellite.

NOTE: 	This definition was taken from TS 22.261 [2].

satellite access type: in the context of this study, it refers to a satellite access with a specific set of characteristics (including orbit type, orbital characteristics, satellite capabilities, IoT or NR).

multi-orbit satellite access: in the context of this study, it refers to different satellite access types associated with a specific orbit being considered as different RAT types for differentiation by core network, for example,

- “NR(LEO)”, “NR(MEO)”, “NR(GEO)” and “NR(OTHERSAT)”.  (refer to TS 23.501)

- "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)".  (refer to TS 23.401)

satellite ground station location: in the context of this study, it refers to the geographical area where a 5G system with multi orbit satellite access is providing the network access to a UE.