3GPP TSG SA WG 1 Meeting #108 	S1-244339

Orlando, USA, 18-22 November 2024

Source:	Nokia, ETRI

pCR Title:	Pseudo-CR on updates of use case 5.19 to replace handover by mobility and GEO by GSO

Draft Spec:	3GPP TR 22.887-020

Agenda item:	7.3

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.19 in FS_5GSAT_Ph4 TR.

1. Introduction

This contribution proposes to update the description, pre-conditions, service flows, post-conditions and potential new requirements in use case “using GSO for sleep/power management”.

2. Reason for Change

The term “handover” assumes UE is in connected mode to GSO while searching & selecting NGSO, which is not the intention of this use case. Therefore the term “handover” should be replaced by 'mobility' being the more general term covering both idle/inactive mode and connected mode mobility.

In addition, to be consistent with GSO and NGSO in other use cases, GEO is replaced by GSO.

Also some format cleanup for pre-conditions.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887-020.

* * * First Change * * * *

## 5.19	Use case on using GSO for sleep/power management

### 5.19.1	Description

Many UEs are power limited and need to conserve power to extend battery life.  In some cases, UEs receive (or have to send) packets very infrequently.  In these cases, it is wasteful of the UE to continuously maintain and establish sync with a NGSO satellite that needs to be tracked through the sky.  A GSO satellite can be very useful for these UEs.  The UE would only try to connect to a NGSO satellite if the GSO satellite told it to do so.  The GSO satellite would only tell the UEs to connect to the NGSO if the network informed the GSO that packets were waiting for the UE.  In cases where the UE needs to send a packet, it would either send it directly to the GSO, or request (via the GSO) resources from the network on a NGSO satellite.

### 5.19.2	Pre-conditions

- The network supports both GSO and NGSO satellites

- The network supports mobility between GSO satellite(s) and NGSO satellite(s)

- A network plan that deconflicts frequency allocations between the GSO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the different satellite access network types

- The network should be aware of when to tell the UE to move to a NGSO satellite connection.

### 5.19.3	Service Flows

- UE establishes a connection with a GSO satellite

- UE enters sleep mode

- UE wakes ups periodically and re-establishes connection with a GSO

- GSO informs UE of whether it needs to wake up

- If there is no need to wake up, UE goes back to sleep

- If there is a need to establish a connection, UE wakes up, and establishes a connection with NGSO instead of the GSO

### 5.19.4	Post-conditions

UE is in appropriate sleep mode on GSO or monitoring communications mode on NGSO

### 5.19.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.19.6	Potential New Requirements needed to support the use case

[PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbit satellite access shall be able to support a mechanism for the network to inform a UE when coverage is available and in which satellite system.

Editor’s note: this requirement is FFS