3GPP TSG-WG SA1 Meeting #108	S2-244364

Orlando, FL, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.268 | CR | 0083 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 22.268 CR PWS support for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 28-10-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA1 recevied a LS R2-2409243 from RAN2 to inform that it has started work on further enhancements to support NB-IoT and eMTC for NTN, with the following agreements. - RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS - Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases) - The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. There is a note in clasue 4.2 that says that UEs “supporting eDRX may not support all requirements for PWS”. This note could be read to mean that UEs that indicate a capability for eDRX may not support all requirements for PWS. However, since eDRX is a mandatory feature when connected to an NB-IoT RAN, the UE sends no capability indication for eDRX. Thus, the note should be udpated to clarify that the PWS may not be supported when the UE is using NB-IoT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the note in clause 4.2 to clarify that a UE that is using NB-IoT may not support all requirements for PWS. Fixed NOTE numbering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.2	High level general requirements for Warning Notification delivery

The following list gives the high level general requirements for Warning Notification delivery:

-	PWS shall be able to broadcast Warning Notifications to multiple users simultaneously with no acknowledgement required.

-	PWS shall be able to support concurrent broadcast of multiple Warning Notifications.

-	Warning Notifications shall be broadcast to a Notification Area which is based on the geographical information as specified by the Warning Notification Provider.

-	PWS capable UEs (PWS-UE) in idle mode shall be capable of receiving broadcasted Warning Notifications.

NOTE 1:	A bandwidth reduced low complexity UE, a UE supporting eDRX, or a UE that is using NB-IoT, may not support all requirements for PWS, including ETWS, CMAS, EU-Alert and KPAS.

-	PWS shall only be required to broadcast Warning Notifications in languages as prescribed by regulatory requirements.

-	Warning Notifications are processed by PWS on a first in, first out basis, subject to regulatory requirements.

-	Reception and presentation of Warning Notifications to the user shall not pre-empt an active voice or data session.

-	Warning Notifications shall be limited to those emergencies where life or property is at imminent risk, and some responsive action should be taken.

NOTE 2: 	This requirement does not prohibit the use of the operator’s network (i.e. broadcast technology) implemented for Warning Notifications to be used for commercial services.

# * * * * End of changes * * *