| 3GPP TR 22.887 V0.3.0 (2024-11) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on satellite access - Phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Abbreviations	9

4	Overview	10

5	Use cases	11

5.1	Use case of IMS Voice Call Using GEO Access	11

5.1.1	Description	11

5.1.2	Pre-conditions	12

5.1.3	Service Flows	12

5.1.4	Post-conditions	13

5.1.5	Existing feature partly or fully covering use case functionality	13

5.1.6	Potential New Requirements needed to support the use case	13

5.1.6.1	Potential Service Requirements	13

5.1.6.2 	Potential KPI Requirements	14

5.2	Use case on Resilient Notification	14

5.2.1	Description	14

5.2.2	Pre-conditions	14

5.2.3	Service Flows	14

5.2.4	Post-conditions	15

5.2.5	Existing features partly or fully covering the use case functionality.	15

5.2.6	Potential New Requirements needed to support the use case	15

5.3	Use case on Resilient Satellite Communication for Public Safety	15

5.3.1	Description	15

5.3.2	Pre-conditions	16

5.3.3	Service Flows	16

5.3.4	Post-conditions	16

5.3.5	Existing features partly or fully covering the use case functionality	16

5.3.6	Potential New Requirements needed to support the use case	16

5.4	Use case on service continuity through multi-orbit satellite access	17

5.4.1	Description	17

5.4.2	Pre-conditions	17

5.4.3	Service Flows	17

5.4.4	Post-conditions	18

5.4.5	Existing features partly or fully covering the use case functionality	18

5.4.6	Potential New Requirements needed to support the use case	18

5.5	Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks	18

5.5.1	Description	18

5.5.2	Pre-conditions	19

5.5.3	Service Flows	19

5.5.4	Post-conditions	19

5.5.5	Existing features partly or fully covering the use case functionality	19

5.5.6	Potential New Requirements needed to support the use case	20

5.6	Use case on switching between multi-orbits satellite networks in remote area applications	20

5.6.1	Description	20

5.6.2	Pre-conditions	20

5.6.3	Service Flows	20

5.6.4	Post-conditions	21

5.6.5	Existing features partly or fully covering the use case functionality	21

5.6.6	Potential New Requirements needed to support the use case	21

5.7 	Use case on assisting vehicular communications via multi-orbit satellite access	21

5.7.1	Description	21

5.7.2	Pre-conditions	22

5.7.3	Service Flows	22

5.7.4	Post-conditions	23

5.7.5	Existing features partly or fully covering the use case functionality	23

5.7.6	Potential New Requirements needed to support the use case	24

5.8	Use case on traffic over different orbit satellites	24

5.8.1	Description	24

5.8.2	Pre-conditions	24

5.8.3	Service Flows	24

5.8.4	Post-conditions	25

5.8.5	Existing features partly or fully covering the use case functionality	25

5.8.6	Potential New Requirements needed to support the use case	26

5.9	Use case on Public Warning Service	26

5.9.1	Description	26

5.9.2	Pre-conditions	26

5.9.3	Service Flows	27

5.9.4	Post-conditions	27

5.9.5	Existing features partly or fully covering the use case functionality	27

5.9.6	Potential New Requirements needed to support the use case	27

5.10	Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay	27

5.10.1	Description	27

5.10.2	Pre-conditions	28

5.10.3	Service Flows	29

5.10.4	Post-conditions	29

5.10.5	Existing features partly or fully covering the use case functionality	29

5.10.6	Potential New Requirements needed to support the use case	29

5.10.6.1	Potential Service Requirements	29

5.10.6.2 	Potential KPI Requirements	30

5.11	Use case on emergency communication using satellite access	30

5.11.1	Description	30

5.11.2	Pre-conditions	30

5.11.3	Service Flows	31

5.11.4	Post-conditions	31

5.11.5	Existing features partly or fully covering the use case functionality	31

5.11.6	Potential New Requirements needed to support the use case	32

5.12	Use case on supporting different services with multi-orbit satellites	32

5.12.1	Description	32

5.12.2	Pre-conditions	32

5.12.3	Service Flows	32

5.12.4	Post-conditions	33

5.12.5	Existing features partly or fully covering the use case functionality	33

5.12.6	Potential New Requirements needed to support the use case	34

5.13	Use case on wide-area disaster control support using multi-orbit satellite networks	34

5.13.1	Description	34

5.13.2	Pre-conditions	34

5.13.3	Service Flows	34

5.13.4	Post-conditions	34

5.13.5	Existing feature partly or fully covering use case functionality	35

5.13.6	Potential New Requirements needed to support the use case	35

5.14	Use case on broadband services through multi-orbit satellite access	35

5.14.1	Description	35

5.14.2	Pre-conditions	36

5.14.3	Service Flows	36

5.14.4	Post-conditions	36

5.14.5	Existing features partly or fully covering the use case functionality	36

5.14.6	Potential New Requirements needed to support the use case	36

5.15	Use case on multi-orbit access for cross-hybrid service	37

5.15.1	Description	37

5.15.2	Pre-conditions	37

5.15.3	Service Flows	37

5.15.4	Post-conditions	37

5.15.5	Existing features partly or fully covering the use case functionality	38

5.15.6	Potential New Requirements needed to support the use case	38

5.16	Use case on using GEO for initial network entry and service initialization	38

5.16.1	Description	38

5.16.2	Pre-conditions	38

5.16.3	Service Flows	38

5.16.4	Post-conditions	39

5.16.5	Existing features partly or fully covering the use case functionality	39

5.16.6	Potential New Requirements needed to support the use case	39

5.17	Use case on  resource and energy efficiency in a multi-orbit Satellite Access System	39

5.17.1	Description	39

5.17.2	Pre-conditions	40

5.17.3	Service Flows	40

5.17.4	Post-conditions	40

5.17.5	Existing features partly or fully covering the use case functionality	40

5.17.6	Potential New Requirements needed to support the use case	41

5.18	Use case on using GEO to land data in a desirable geographical location	41

5.18.1	Description	41

5.18.2	Pre-conditions	41

5.18.3	Service Flows	41

5.18.4	Post-conditions	42

5.18.5	Existing features partly or fully covering the use case functionality	42

5.18.6	Potential New Requirements needed to support the use case	42

5.19	Use case on using GEO for sleep/power management	42

5.19.1	Description	42

5.19.2	Pre-conditions	42

5.19.3	Service Flows	42

5.19.4	Post-conditions	42

5.19.5	Existing features partly or fully covering the use case functionality	43

5.19.6	Potential New Requirements needed to support the use case	43

5.20	Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR)	43

5.20.1	Description	43

5.20.2	Pre-conditions	43

5.20.3	Service Flows	44

5.20.4	Post-conditions	44

5.20.5	Existing features partly or fully covering the use case functionality	44

5.20.6	Potential New Requirements needed to support the use case	44

5.21	Use case on combined terrestrial and multi-orbit satellite access for connecting MBSR and 5GC	45

5.21.1	Description	45

5.21.3	Service Flows	46

5.21.4	Post-conditions	46

5.21.5	Existing features partly or fully covering the use case functionality	46

5.21.6	Potential New Requirements needed to support the use case	46

5.22	Use case on aircraft communication services in Joint TN/NTN and multi-orbit deployments	47

5.22.1	Description	47

5.22.2	Pre-conditions	48

5.22.3	Service Flows	48

5.22.4	Post-conditions	48

5.22.5	Existing features partly or fully covering the use case functionality	48

5.22.6	Potential New Requirements needed to support the use case	50

5.22.6.1	Potential Service Requirements	50

5.22.6.2 	Potential KPI Requirements	50

5.23	Use case on connectivity served by multi satellite operators	51

5.23.1	Description	51

5.23.2	Pre-conditions	51

5.23.3	Service Flows	51

5.23.4	Post-conditions	52

5.23.5	Existing features partly or fully covering the use case functionality	52

5.23.6	Potential New Requirements needed to support the use case	52

5.24	Use case on Broadcast Services with satellite access for unregistered UEs	53

5.24.1	Description	53

5.24.2	Pre-conditions	54

5.24.3	Service Flows	55

5.24.4	Post-conditions	55

5.24.5	Existing features partly or fully covering the use case functionality	56

5.24.6	Potential New Requirements needed to support the use case	56

6	Consolidated requirements	57

6.1	Introduction	57

6.2	IMS voice calls using GEO satellite access	57

6.2.1	Consolidated functional requirements	57

6.2.2	Consolidated potential KPIs	57

6.3	Resilient Notification	57

6.4	Emergency communications and mission critical services using satellite access	58

6.4.1	PWS with satellite access	58

6.4.2	Other emergency communications and mission critical services using satellite access	58

6.5	Multi-orbit	58

6.5.1	General aspects	58

6.5.2	Service continuity aspects	58

6.6	Charging aspects	59

6.7	Other aspects for satellite access	59

7	Conclusions and recommendations	59

Annex A (informative): Mapping of use cases and topics covered	60

Annex B (informative): Change history	61

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes use cases and aspects related to enhancements of the 5G system over satellite, including:

- multi-orbit satellite access for multiple services

- resilient notification

- support for emergency communications and mission critical services using satellite access

- support IMS voice calls using GEO satellite access

- Broadcast services with satellite access for unregistered UEs

Editor's note:	Clarification on the use of the term GEO versus GSO is FFS

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: “Service requirements for the 5G system”

[3]	3GPP TS 22.228: “Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem“.

[4]	ITU-T E.800: "Definitions of terms related to quality of service ".

[5]	ITU-T G.114: "One-way transmission time".

[6]	ITU-T G.107: "The E-model: a computational model for use in transmission planning "

[7]	X. Huang, W. Qi, X. Xia, Y. Sun, Z. Sun and M. Peng, "IoT NTN for Voice Services: Architectures, Protocols, and Challenges," in IEEE Network, 2024.

[8]	“Satellite firms forge unlikely alliances to create seamless multi-orbit networks”, https://spacenews.com/satellite-firms-forge-unlikely-alliances-to-create-seamless-multi-orbit-networks/, March 2024

[9]	“Autonomous shipping”, https://www.imo.org/en/MediaCentre/HotTopics/Pages/Autonomous-shipping.aspx

[10]	Dhinesh Kumar R, Rammohan A, Revolutionizing Intelligent Transportation Systems with Cellular Vehicle-to-Everything (C-V2X) technology: Current trends, use cases, emerging technologies, standardization bodies, industry analytics and future directions. Vehicular Communications, Vol.43, 2023, 100638

[11]		5G Automotive Association (5GAA), C-v2x use cases and service level requirements -volume i，5GAA website, https://5gaa.org/c-v2x-use-cases-and-service-level-requirements-volume-i/, 2019

[12]	5GAA discusses how C-V2X turns Connected and Safe Mobility into Reality at Mobile World Congress 2022 in Barcelona, https://5gaa.org/5gaa-discusses-how-c-v2x-turns-connected-and-safe-mobility-into-reality-at-mobile-world-congress-2022-in-barcelona/

[13]	https://www.ncbi.nlm.nih.gov/pmc/articles/PMC8177735/

[14]	Directive (EU) 2018/1972 of the European Parliament and of the Council of 11 December 2018 establishing the European Electronic Communications Code, https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX:32018L1972

[15]	3GPP TS 22.268: "Public Warning System (PWS) requirements".

[16]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[17]	3GPP TS 22.137: Service requirements for Integrated Sensing and Communication

[18]	SAA (Seamless Air Alliance)  - White Paper “3GPP Non Terrestrial Network” for aviation connectivity - contributors: Airbus, Vodafone, Thales, Telesat, GlobalReach, Kontron, Panasonic, Astronics, Safran, Lufthansa Systems, and ST Engineering iDirect, https://www.seamlessalliance.com/wp-content/uploads/seamless-ntn.pdf, 2024

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Resilient Notification: in the context of this study, it refers to a highly reliable mechanism that delivers notifications directly to user devices, such as smartphones, when in poor conditions in satellite access.

satellite access: direct connectivity between the UE and the satellite.

NOTE: 	This definition is taken from TS 22.261 [2].

Very Small Aperture Terminal (VSAT): is a small-sized earth station used to transmit and receive data, voice and video over a satellite communication network, excluding broadcast television

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GEO	Geostationary satellite Earth Orbit

GSO	Geosynchronous Orbit

LEO	Low-Earth Orbit

MEO	Medium-Earth Orbit

NGSO	Non-Geostationary Satellite Orbit

VSAT	Very Small Aperture Terminal

# 4	Overview

The present document captures a set of use cases and potential service requirements related to the 5G system with satellite access taking into account new capabilities such as:

- multi-orbit satellite access for multiple services

- resilient notification service

In addition, the TR includes several use cases to enhance the following aspects:

- support for emergency communications and mission critical services using satellite access

- support IMS voice calls using GEO satellite access

- support Broadcast services with satellite access for unregistered UEs

# 5	Use cases

## 5.1	Use case of IMS Voice Call Using GEO Access

### 5.1.1	Description

Using regular mobile phones to make voice calls via satellite access is becoming increasingly popular, especially due to the advantage satellites have in providing coverage in rural and remote areas [7]. Satellite voice call services in 3GPP provide consistent network connectivity, accessible at any time and place, facilitating smooth communication across both terrestrial and satellite networks.

Since Release 17, GEO satellite access has been included in the 3GPP standards as an access technology for 5G, supporting all types of media like voice, data, and video by default [2]. However, due to the unique challenges of GEO satellites—such as 35,786 km distance from the earth, around 285ms signal delays, and atmospheric attenuation—the data rates cannot be increased simply by expanding the bandwidth, which resulting in the services supported by Non-Geostationary Satellite Orbit (NGSO) may not be supported by GEO.

Voice calls using the IMS (IP Multimedia Subsystem) platform have been a standard feature since the 3GPP's Release 5 [3]. Due to IMS's capability to support diverse multimedia services and ensure interoperability, several services have subsequently been added, including IMS emergency calls, messaging, group management, push-to-talk, and real-time communications. These enhancements have positioned IMS as a critical tool for connecting different operators and service providers for voice calls across various types of access networks. However, when integrating IMS with GEO satellite access within the 3GPP framework, 3 main aspects impact the quality of experience:

- One-way transmission delay aspect: also known as mouth-to-ear delay, is the time from when a call is initiated to when it is heard. This delay significantly impacts the R score in the E-Model [6], which assesses voice call quality. Managing this delay is key in network design to achieve higher R scores. The ITU-T recommends a maximum delay of 400 milliseconds for network planning [5]. However, with GEO satellite access, the propagation delay is much longer than other technologies (285ms), necessitating careful calculation of the delay budget. This careful management is crucial when adapting IMS voice calls to GEO satellite access to ensure greater user satisfaction.

- Codec bitrate aspect: codec bitrate refers to a bit rate used to encode/decode human voice speech for digital transmission. The relationship between a codec and data rate is critical, as it determines how efficiency a voice speech can be transmitted in the digital communication systems. For many years, the focus for the development of 3GPP voice codecs was to improve voice quality for clean and impaired channels at keeping the data rate approximately constant. For example, significant improvements regarding the audio quality have been made comparing the AMR codec, which supports bitrates ranging from 4.75 to 12.2 kbps, and the development of the 5G voice codec EVS, which delivers superior sound quality at bitrates between 5.9 and 128 kbps. In contrast, GEO satellite systems typically only support significantly lower data rates within the 3GPP framework.

- Call setup time aspect: call setup time is critical in both traditional telephony and modern IMS systems, as it heavily impacts user satisfaction and the perceived quality of the service [4]. Shorter call setup times are particularly important as they can significantly improve the user experience. Advancements in 3GPP standards have led to network optimizations that reduce these delays and enhance processing speeds. However, incorporating GEO systems into these networks poses challenges, typically resulting in longer setup times due to the latency, limited achieved data rate inherent in GEO system.

Thus, this use case is designed to leverage the integration of IMS systems and GEO in 3GPP to enhance the quality of voice call services.

### 5.1.2	Pre-conditions

Figure 5.1.2-1: Mobile phone supports both terrestrial and GEO satellite access to support IMS voice

MNO-A employs its own GEO satellites or those of a satellite operator under service/roaming agreements to offer an IMS voice call service using GEO satellite access.

The GEO satellite access might involve a new satellite deployment or an existing GEO satellite system, as depicted in Figure 5.1.2-1.

Tom uses a mobile phone that can connect to both terrestrial networks and the GEO satellite system. His phone is equipped with encoding and decoding technologies that compress and decompress human voices for transmission via GEO satellite access.

### 5.1.3	Service Flows

Step 1 (Tom’s mobile phone subscribes to the IMS voice call service using GEO satellite access): Tom resides in Mardiv and earns his living by leading a fishing team. Whenever he heads out to fish, he ensures to report his safety once he arrives at the fishing spot. To maintain communication, Tom currently uses two different devices: a satellite terminal for calling his family when terrestrial coverage is unavailable, and a regular mobile phone when within coverage. However, Tom is looking to streamline his communication setup by using a single mobile phone that works anywhere, anytime. MNO-A offers such a service, and Tom has opted into their IMS voice call service using GEO satellite access to stay connected seamlessly.

Step 2 (Tom’s mobile phone registers to the IMS platform provided by MNO-A):

MNO-A has upgraded its IMS platform to support the IMS voice call service using GEO satellite access. As with the existing IMS setup, Tom’s mobile phone must be registered with the IMS platform before he can access the service. During this registration process, the platform records details such as the capabilities of the mobile phone and access network information. To ensure he can use the IMS voice call service, Tom completes the registration, enabling his mobile phone to connect seamlessly through this enhanced IMS platform.

Step 3

Sub-scenario 1 (Satellite MO call to a terrestrial network): Tom calls his wife, who is in the city center of Male:

- Aware that he's using a satellite phone, Tom waits patiently for a while before the ringing tone starts.

- As the call is transmitted via satellite, the IMS platform transcodes the voice data stream between a codec that is suitable for low-bit rate transmission at the satellite MO side and a codec that is suitable for the terrestrial side.

Sub-scenario 2 (Satellite MT call from a terrestrial network): Tom’s mother, who is subscribed to MNO-B and located in Bangkok, calls Tom:

- MNO-B’s IMS platform communicates with MNO-A’s IMS platform to transfer the call request to Tom.

- MNO-A’s network locates Tom, paging him to reconnect to MNO-A’s IMS platform.

- Knowing she’s making a satellite phone call, Tom’s mother waits patiently a while before she hears the ringing tone.

- The IMS platforms transcodes the voice codec as in sub-scenario-1.

- During the call, Tom’s mother informs him about an approaching thunderstorm and advises him to head to the nearest safe harbor.

Sub-scenario 3 (Satellite MO call to a satellite MT call): Tom calls his friend Pasi, who also uses a mobile phone equipped with IMS voice service using GEO, to locate the nearest safety port as a thunderstorm approaches:

- The IMS platform does not need to transcode the codec since both mobile phones use the same low bit rate codec suitable for satellite communication.

### 5.1.4	Post-conditions

Thanks to this IMS voice call service using GEO satellite access, Tom can make an IMS voice call anywhere, anytime by using only one mobile phone.

### 5.1.5	Existing feature partly or fully covering use case functionality

From TS 22.261:

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system with satellite access shall be able to support low power MIoT type of communications.

The propagation delay via satellite associated with these orbit ranges can be summarized in Table 7.4.1-1:

Table 7.4.1-1: Propagation delay via satellite

|  | UE to serving satellite propagation delay [ms] [NOTE 1] |  | UE to ground max propagation delay [ms] [NOTE 2] |
| --- | --- | --- | --- |
|  | Min | Max |  |
| LEO | 3 | 15 | 30 |
| MEO | 27 | 43 | 90 |
| GEO | 120 | 140 | 280 |
| NOTE1: The serving satellite provides the satellite radio link to the UE NOTE2: delay between UE and ground station via satellite link; Inter satellite links are not considered |  |  |  |

From TS 22.228:

IMS shall be capable to provide transcoding (at least for voice sessions) where needed when two UEs do not support a common codec.

IP multimedia sessions shall be able to support a variety of different media types. A set of media types shall be identified to ensure interoperability (e.g., default codec selection and header compression).

The IMS network and intermediate networks shall support codec negotiation across one or multiple interconnects to minimise transcoding (and preferably eliminate it) to provide the highest quality service to the user.

If two UEs, belonging to two IMS networks, do not support a common codec for voice service session, the network and/or intermediate networks shall be capable to provide transcoding functionality at the interconnection point.

### 5.1.6	Potential New Requirements needed to support the use case

#### 5.1.6.1	Potential Service Requirements

[PR 5.1.6-001] The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [3].

[PR 5.1.6-002] The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) considering the transmission data rate, latency and packet size.

[PR 5.1.6-003] The 5G system with GEO satellite access shall be able to support a codec suitable for the transfer of voice considering the transmission data rate and latency provided by the 5G system with GEO satellite access.

#### 5.1.6.2 	Potential KPI Requirements

Table 5.1.6.2-1	Potential KPI for IMS voice call using GEO satellite

| Scenario | UE type | Transmission data rate |  | Call setup time NOTE 1 |
| --- | --- | --- | --- | --- |
|  |  | UL | DL |  |
| IMS voice call using GEO | Handheld | [1-3] kbit/s | [1-3] kbit/s | [4-30] s NOTE 2 |
| NOTE 1: call set up time refers to [4]; NOTE 2: the lower bound of 4s originated from the experience in terrestrial VoNR/VoLTE, while the upper bound of the 30s is derived based on the user’s patience suggestions (30s) in [13]; |  |  |  |  |

## 5.2	Use case on Resilient Notification

### 5.2.1	Description

Communication between Satellite and smartphone typically has very limited power margin and usually requires line of sight for best performance. For Mobile Originated calls, users are aware of the channel quality and can achieve LOS communication by consciously adjusting the spot of signal transmission/reception. However, for Mobile terminated communication, users are unaware of its channel condition and might miss important calls/messages due to poor reception channel condition. Satellite access is the last option when terrestrial network is not available, it will be beneficial to improve the user reachability in satellite access system to achieve better user experience.

This requires the satellite access system to be able to notify the user about missed incoming Mobile Terminated MMTel calls/messages (as specified in [3]) when the user is not reachable in satellite access system. “Resilient Notification Service" in the context of satellite direct-to-device communication refers to a highly reliable and efficient notification mechanism that delivers notifications directly to user devices, such as smartphones, via satellite access. This mechanism ensures that notifications of important calls reach users in areas where normal connectivity failed or is compromised.

### 5.2.2	Pre-conditions

Alice is enjoying her vacation in a remote rental vacation house with her friends, Lisa and Allen, in Rocky Mountain National Park after graduating from college Alice decides to take a hike in the mountain area one early morning. She noticed that there is no terrestrial network coverage at the parking lot and her smart phone has switched to satellite access network. The signal strength display of her smart phone at the parking lot of the trail has one or two bars.

She is a service subscriber of operator TTT for both terrestrial and satellite communication service. The operator has deployed Resilient Notification Service for satellite access users. Before her vacation, Alice has ensured that the Resilient Notification Service is available on her account.

### 5.2.3	Service Flows

Alice hears a beeping sound from her backpack half way to the top of the mountain, it is the Resilient Notification beep which last for a specified period of time. She takes out her smart phone and based on the incoming service information (e.g., caller information) provided by the Resilient Notification service, she finds that it is the important phone call she has been expecting from her recruiter. Alice then calls back the recruiter. It is the important phone call she has been expecting about her interview result of her dream job.

When Alice walks back to the parking lot of the trail, she needs to pass a dense forest area. Her smart phone beeps again but she has no open area to respond. The beep stops after a specified period of time. After few minutes of fast walking, Alice finally finds an open area and takes out her phone from her backpack. Based on the incoming service information (e.g., caller information) provided by the resilient notification service,  Alice knows that her Mom has called her several minutes ago and then she calls back her mom, it turns out that Alice’s mom just wants to ask her opinion about the birthday gift she is getting for her dad.

Alice has to detour and take an alternative route back to the rental vacation house. Her smart phone starts to beep again for Resilient Notification while she is driving. Based on the incoming service information (e.g., service type information) provided by the resilient notification service, Alice finds that it is a short message from Lisa and it is not urgent to give a feedback. Alice finally stops at  a scenery viewing outlook after 10 minutes’ drive and pulls over her car. Alice calls back Lisa and joins Lisa for lunch at the restaurant near the vacation house.

### 5.2.4	Post-conditions

When Alice comes back from her vacation, she was happy that her parents and friends were still able to reach her even when she is in the very deep coverage area. She will start her job in the fall and her job requires her to travel around all the time and some maybe in remote areas. She wants to make sure she can be reliably reached by her family.

The company she works for also encourage employee to ensure they have Resilient Notification Service to increase the reachability of the employees travelling around in remote area.

### 5.2.5	Existing features partly or fully covering the use case functionality.

None

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-001] Subject to operator’s policies, a 5G system with satellite access shall be able to support Resilient Notification service to notify users, with valid subscription to the notification service, about a missed mobile terminated service when the user is unreachable via satellite access.

NOTE: 	The Resilient Notification service can provide the user with service information e.g., caller’s information and service type.

## 5.3	Use case on Resilient Satellite Communication for Public Safety

### 5.3.1	Description

This use case describes a scenario where the resilient satellite communication system for public safety can provide  resilient services to UEs with proper operation (a.k.a. resilient operation) when either the backhaul link between the LEO satellite and the ground station is temporarily unavailable due to a natural disaster or the core newtork of LEO satellite was temporally unavailable due to cyberwarfare attack.

In order to preserve service continuity, the resilient operation could eliminate service interruption period before re-establish the backhaul link or restore the core network service.

Figure 5.3.1-1: Scenario for Resilient Satellite Communication for Public Safety

### 5.3.2	Pre-conditions

The Coast Guard has deployed a smart boat named Virginia for river patrol. Officers Jason and Alice can navigate the boat while reporting their status periodically to the Coast Guard Administration Centre.

Multiple core networks can be selected for disaster scenarios.

The LEO satellite can connect to GEO/MEO via ISLs or directly to the ground 5G core network via feeder link.

### 5.3.3	Service Flows

1. Connectivity to land 5GC:

- Both officers’ smartphones and their smart boat are connected to the 5G network via a LEO satellite link.

- They periodically report their current status to the Coast Guard Administration Centre.

2. Interruption of backhaul link:

- An EF5 tornado develops and moves toward the satellite ground station.

- The backhaul link between the LEO satellite and the ground station becomes temporarily unavailable because the ground station's antennas are damaged by the EF5 tornado.

3. Resilient operation to maintain service:

- The satellite communication system maintains the service after the LEO satellite loses the backhaul link connection.

- Officer Jason and Alice can still report their status to the Coast Guard Administration Centre through core network, and communicate with each other e.g. via UE-Satellite-UE communication.

4. Re-establish the connectivity to original land 5GC when it is available again.

### 5.3.4	Post-conditions

During the river patrol, Officers Jason and Alice can report status to the Coast Guard Administration Centre as expected

### 5.3.5	Existing features partly or fully covering the use case functionality

Existing requirements from clause 6.46.9 in TS 22.261 [2]:

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of a UE-Satellite-UE communication.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast).

### 5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-001] Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication.

NOTE: 	The resilient communication can be achieved through a single satellite or multiple satellites over different types of orbits.

## 5.4	Use case on service continuity through multi-orbit satellite access

### 5.4.1	Description

The future of satellite communications is moving toward hybrid satellite networks that combine the services from low-Earth orbit satellites and high-altitude satellites in medium-Earth and geostationary orbits. It is being driven by the commercial sectors including maritime and aviation, which are increasingly pursuing multi-orbit satellite solutions [8].

In the maritime sector, enhanced maritime communication technologies have been developed to support autonomous ships (e.g., Maritime Autonomous Surface Ship (MASS)), which are commercial vessels that operate independent of human interaction, using sensors, software, and communication systems to navigate, avoid collisions and perform the tasks and functions of the vessel [9]. In order to support autonomous ships, the maritime communication systems should enable an autonomous ship to exchange the information with a remote monitoring and control centre on the ground.

Considering the provision of global communication coverage and seamless network connectivity in maritime environment, communication technology utilizing multi-orbit satellites can be employed in maritime communication systems. During the rollout of LEO satellite constellations, the limited number of satellites in orbit inevitably leads to discontinuities in coverage. Therefore, it should be considered how to ensure the continuity of communication services through multi-orbit satellite access.

### 5.4.2	Pre-conditions

The shipping company AUTOSHIP operates many autonomous ships around the world. AUTOSHIP also operates several centres to monitor and control the autonomous ships in remote area on the ground.

The satellite operator SAT-OP has deployed multi-orbit (LEO, MEO and GSO) satellites and has an agreement with Terrestrial Operator TER-OP to provide communication services for UEs under satellite coverage.

AUTOSHIP has signed a contract with SAT-OP for communication services using satellite access. Thus, their devices on autonomous ships can communicate with a remote monitoring and control centre via satellite access.

### 5.4.3	Service Flows

Figure 5.4.3-1: Example of service continuity through multi-orbit satellite access

- Device A on an autonomous ship is under the coverage of LEO Satellite Sat-A and MEO/GSO Satellite Sat-B.

- Taking into account of communication latency and power consumption, Device A registers with the terrestrial 5G network via LEO Satellite Sat-A. After the registration, it can communicate with a remote monitoring and control center operated by AUTOSHIP company.

- During the communication service, due to the short service duration and discontinuous coverage of LEO satellites, if Device A is under the coverage of only MEO/GSO Satellite Sat-B, the communication path can be switched to MEO/GSO Satellite Sat-B depending on the required quality of service. Then, Device A communicates with a remote monitoring and control center via MEO/GSO Satellite Sat-B.

- If Device A is under the coverage of the following LEO Satellite Sat-C, the communication path can be switched to LEO Satellite Sat-C with the consideration of communication latency and power consumption. As a result, Device A communicates with a remote monitoring and control center via LEO Satellite Sat-C.

- The charging information of the traffic data exchanged via multi-orbit satellites is collected and reported to the remote core network.

### 5.4.4	Post-conditions

Device A on an autonomous ship can communicate with a remote monitoring and control center continuously through multi-orbit satellites connectivity.

### 5.4.5	Existing features partly or fully covering the use case functionality

Regarding TS 22.261[2], satellite access and satellite connectivity are supported in Rel-19, as follows:

6.3	Multiple access technologies

The 5G system shall be able to provide services using satellite access.

6.46	Satellite access

For a 5G system with satellite access, the following requirements apply:

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites).

9	Charging aspects

The 5G core network shall support collection of charging information based on the access type (e.g., 3GPP, non-3GPP, satellite access).

However, it is not sufficient in regards of supporting service continuity through multi-orbit satellite access.

### 5.4.6	Potential New Requirements needed to support the use case

[PR 5.4.6-001] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellites in different orbits (due to the movement of the UE and/or the satellites).

NOTE: 	Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits).

[PR 5.4.6-002] A 5G network with satellite access shall be able to support charging mechanisms for communication services using satellite access in different orbits based on the type of satellites (e.g., LEO, MEO, GEO).

## 5.5	Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks

### 5.5.1	Description

Mobile base station relay (MBSR) is a good option for providing continuous connectivity to moving UEs in a vehicle. MBSRs can be connected to the network through non-terrestrial networks (NTN). NTN deployments are essential for providing connectivity to relays in isolated and remote areas. The 5G system supports a multi-dimensional heterogeneous architecture, incorporating terrestrial access, low altitude aerial devices (e.g., UAVs), High Altitude Platform Station (HAPS) and satellites (Low Earth orbit (LEO), Medium Earth Orbit (MEO) and GEO)) to meet varying connectivity requirements. Different types of satellites in various orbits can serve different services based on their latency requirements. For example, GEO satellites can support services like video streaming (buffered), email, chat, file sharing while LEO satellites are better suited for services requiring low latency like V2X messaging and real-time gaming. With the movement of relays and depending on the requirements of connected UEs, switching between multi-orbit satellites can ensure all use case scenarios covered using NTN deployments only (as shown in Figure 5.5.1-1).

Figure 5.5.1-1: Multi-orbit satellite networks support for Mobile base station relays (The moving vehicle in the figure represents various scenarios for a mobile relay, whether it is on a ship, train, or aeroplane)

### 5.5.2	Pre-conditions

MBSR is connected through GEO satellite.  The service provider’s 5G system supports switching from GEO to available LEO connectivity based on service requirements.

### 5.5.3	Service Flows

Initially, the MBSR is connected to the network through GEO satellite links. When the vehicle moves where LEO  coverage is  available, the MBSR remains connected to GEO. However, since GEO cannot provide low-latency services, the MBSR’s connectivity is switched to LEO, ensuring that NTN can deliver all types of services to the connected UEs.

### 5.5.4	Post-conditions

MBSR is able to provide continuous connectivity to the connected UEs in the remote areas with the help of multi-orbit satellite network (when terrestrial network is not available for MBSR).

### 5.5.5	Existing features partly or fully covering the use case functionality

TS 22.261 [2] specifies the following requirements associated with the use case:

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE: 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE: 5 ms network latency is assumed and added to satellite one-way delay. The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g., a cell sector, a cell or a group of cells).

- A 5G satellite access network connected to 5G core networks in multiple countries shall be able to meet the corresponding regulatory requirements from these countries (e.g., Lawful Interception).

- The 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

- The 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

### 5.5.6	Potential New Requirements needed to support the use case

[PR 5.5.6-001] Subject to operator’s policy, the 5G network with multi-orbit satellite access shall be able to support mobile base station relays to access different orbit satellites considering e.g., availability of the coverage, required QoS.

## 5.6	Use case on switching between multi-orbits satellite networks in remote area applications

### 5.6.1	Description

NTN deployments are being used to provide connectivity in isolated and remote places. 5G system supports multi-dimensional heterogeneous architecture in which terrestrial access, low altitude aerial devices (e.g., UAVs), High Altitude Platform Station (HAPS), and satellites (Low Earth orbit (LEO), Medium Earth Orbit (MEO) and GEO) can be used together to facilitate ubiquitous coverage and varying connectivity requirements in various deployments. There can be various use case scenarios with different latency requirements such as basic communications services, video surveillance, search and rescue operation, urgent medical care, time sensitive emergency communication. Based on latency requirements of services, switching between multi-orbits satellites can serve use case scenarios with latency ranging from 35 ms to 285 ms (latency value range based on latency values for GEO, MEO and LEO as defined in 22.261) using NTN deployments only (in the case of not feasibility of terrestrial deployment in these remote sites).  To achieve continuous global coverage using LEO satellites, it often requires several satellites in a LEO constellation and also LEO satellites do not remain stationary over a single location for a longer time unlike the GEO satellites. However, in some scenarios (e.g., critical communications), switching from GEO to LEO can be beneficial as LEO satellites can provide more immediate, high-resolution imagery and faster communication, which are critical in urgent situations. For example, during an emergency, situations may arise that require search and rescue operations for injured people to ensure they receive urgent medical care to save their lives. (as shown in Figure 5.6.1-1).

Figure 5.6.1-1: Switching between multi-orbits satellite networks in remote areas when needed (based on emergency scenario)

### 5.6.2	Pre-conditions

There is satellite coverage for remote UEs via relay UEs in remote sites through MEO/GEO for basic connectivity services. The service provider’s 5G system supports switching of relay UE from MEO/GEO to available LEO connectivity if lower latency services are required to be served using NTN deployments only.

### 5.6.3	Service Flows

Connectivity in remote areas rely on Non-Terrestrial Network (NTN) deployments, such as MEO/GEO satellites, to provide essential connectivity services. The latency required for these services can be supported using MEO/GEO. However, certain emergency situations, such as search and rescue operations and providing urgent medical aid, demand low latency communication services. MEO/GEO satellites cannot meet the low latency requirements for these services. In such scenarios, LEO satellites are best suited, as they can support low latency applications. Therefore, when these emergencies are detected, connectivity for relay UEs in the affected sites is switched from MEO/GEO to LEO.

### 5.6.4	Post-conditions

Connectivity in isolated remote areas can be supported using NTN deployments. A varied range of QoS can be provided by switching connectivity between multi-orbit satellites based on requirements.

### 5.6.5	Existing features partly or fully covering the use case functionality

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency. [2]

NOTE 1: 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency. [2]

NOTE 2: 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency. [2]

NOTE 3: 5 ms network latency is assumed and added to satellite one-way delay. The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g., a cell sector, a cell or a group of cells).

- A 5G satellite access network connected to 5G core networks in multiple countries shall be able to meet the corresponding regulatory requirements from these countries (e.g., Lawful Interception). [2]

- A 5G system with satellite access shall be able to support relay UEs with satellite access. [2]

NOTE:	The connection between a relay UE and a remote UE is the same regardless of whether the relay UE is using satellite access or not. [2]

- A 5G system with satellite access shall support mobility management of relay UEs and the remote UEs connected to the relay UE between a 5G satellite access network and a 5G terrestrial network, and between 5G satellite access networks. [2]

- A 5G system with satellite access shall support joint roaming between different 5G networks of a relay UE and the remote UEs connected to that relay UE. [2]

### 5.6.6	Potential New Requirements needed to support the use case

[PR 5.6.6-001] A 5G system with satellite access shall support minimum service interruption when a relay UE is switching between multiple orbit satellite connections (e.g., GEO, MEO or LEO) of the same network.

NOTE: 	Relay UE as specified in TS 22.261 [2]

[PR 5.6.6-002] Subject to operator’s policy, the 5G system with satellite access shall be able to switch a  relay UE between multi orbit satellite accesses considering e.g. QoS, availability of the coverage.

## 5.7 	Use case on assisting vehicular communications via multi-orbit satellite access

### 5.7.1	Description

Cellular Vehicle-to-Everything technology (C-V2X) is expected to play a critical role in the development of Intelligent Transportation Systems (ITS) and connected vehicles in the near future [10]. Its evolution to 5G is fostering the synergies between the automotive industry and other verticals to support more scenarios and enhanced driving experience. The 5G Automotive Association (5GAA) is shaping the path to 5G and has identified 7 major groups of use cases as Table.5.7.1-1 depicts.

Table.5.7.1-1 Typical C-V2X Use Cases on 5GAA Technical Reports [11]

| Use Case Category | Use Case Examples |  |
| --- | --- | --- |
| Safety | - Emergency braking - Intersection Management | - Assisted collision warning - Lane change |
| Vehicle Operations Management | - ECU software update - Vehicle Health Monitoring | - Remote support |
| Convenience | - Infotainment - Assisted and cooperative navigation | - Autonomous smart parking |
| Autonomous Driving (L4 and L5) | - Control if autonomous/self-driving is allowed or not - Tele-operation | - Handling of dynamic maps(update/download) - Data/Sensor Sharing |
| Platooning | - Platooning management - Determine position in platoon - Dissolve a platoon | - Manage distance within platoon - Leave a platoon - Control of platoon in steady state |
| Traffic Efficiency and Environmental Friendliness | - Speed Harmonization - Traffic jam information, e.g. traffic Light coordination | - Routing advisor, e.g. Bus Lane sharing |
| Society and Community | - Emergency vehicle approaching - Traffic light priority | - Patient monitoring - Accident Report |

The communication needs of each specific use case can vary in e.g., latency, capacity, data rate, reliability, etc.  For example, software update will request a large amount of vehicles scattered all over the world to download Gigabyte-level data simultaneously without stringent requirements in the latency. Assisted collision warning requests timely information exchange between ITS and the vehicles but the message size is only hundreds of bytes.

However, it is almost consensus that ubiquitous connectivity shall be a fundamental pre-requisite to enabling full-scale digital experience for cars and a fully connected car. Relying only on the coverage of terrestrial access network, it is still a challenge to ensure the network availability and reliability of fully connected cars. Satellite access networks can complement terrestrial network based solutions, especially in white spots (e.g., rural or remote areas) for V2N use cases and in the event of outages and congestion. GEO satellite can provide the global coverage while LEO satellite can support low latency communication on top of this capability. Today, over 200 million connected vehicles worldwide are equipped with applications sharing hazard and traffic warnings on the road. [12] The differentiated characteristics of LEO and GEO satellites will lead to flexible and hybrid solutions based on multi-orbits satellite strategy and terrestrial access network.

This use case illustrates the usage of GEO satellite and LEO satellite in assisting the communication between ITS and the vehicles under the coordination of terrestrial network.

### 5.7.2	Pre-conditions

TerRAN deployed by the terrestrial network operator TerOP has full coverage in the urban areas (e.g., AreaA) and rural area, but limited or even no coverage in deserts and depopulated area (e.g., AreaB).

TerOP has service agreements with satellite operator SatOP1 for sharing SatRAN#1 (GEO satellite based NG-RAN) and SatOP2 for sharing SatRAN#2 (LEO satellite based NG-RAN) to realize the global coverage of all the country.

The telematics devices (e.g., UE#1, UE#2, UE#3) installed on the autonomous driving trucks are the subscribers of TerOP and are allowed for multi-orbits satellite services under the agreement between TerOP and the vehicle management company, who is the owner of the trucks.

All UEs are capable of GEO and LEO satellite access.

### 5.7.3	Service Flows

1. All the trucks are launched out to deliver the goods. UE#1, UE#2 and UE#3 are registered to 5GC via nearby TerRAN and communicate with ITS via TerRAN as Figure 5.7.3-1 (a) shows, where

- UE#1 and UE#3 are downloading new firmware of Infotainment system with the traffic routed following blue colour line.

- UE#2 and UE#3 are requested to share some data for autonomous driving when they are driven into AreaA, and the data traffic is routed following red colour line.

2. As UE#1 moves into AreaB, UE#1 is switched to SatRAN#1 according to service QoS, and/or satellite availability, to continue the firmware download (with minimum interruption) and the signalling exchange as Fig 5.7.3-1(b) shows.

3a. When Area A has no terrestrial coverage in disaster scenario, UE#2 is switched to SatRAN#2 according to the operator’s policy, to continue the transmission (with minimum interruption) as Fig 5.7.3-1(b) shows.

3b. When Area A has no terrestrial coverage in disaster scenario, according to the operator’s policy, UE#3 is switched to SatRAN#2 to continue the data transmission for autonomous driving (with minimum interruption). When SatRAN#2 flies away, UE#3 is switched to SatRAN#1 to continue the firmware download as Figure 5.7.3-1 (b) shows.

4. When UE#1, UE#2 and UE#3 detect the coverage of TerRAN, they are switched back to TerRAN regarding the operator’s policy. Also, UE#2 will be switched back to TerRAN after the congestion is solved.

(a) communication via terrestrial RAN	(b) communication via multiple satellite RANs

Figure 5.7.3-1 Assist V2N via Multi-orbit Satellite Access

Editor's Note:	The figure still needs to be updated according to the service flows.

### 5.7.4	Post-conditions

Thanks to multi-orbit satellite services, UE#1 and UE#3 complete the firmware download and upgrade successfully, UE#2 and UE#3 share some data for  autonomous driving with ITS.

### 5.7.5	Existing features partly or fully covering the use case functionality

SA1 has identified a series of requirements related to satellite access and captured them in TS 22.261.

For RAN sharing, the following requirement can be applicable for this use case.

- A 5G satellite access network shall support NG-RAN sharing.

For the service continuity and user plane aspects, the following requirements can be partly applicable for this use case but not consider multi-orbit situation.

- A 5G system with satellite access shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by different operators having an agreement.

- A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfils the agreed QoS.

- A 5G system with satellite access shall be capable of supporting simultaneous use of 5G satellite access network and 5G terrestrial access networks.

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] Subject to regulatory requirement and operator’s policy, a 5G network with multi-orbit satellite access shall support mechanisms to allow a UE connecting to one of satellite access networks in different orbits considering e.g., QoS, satellite availability.

[PR 5.7.6-002] Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbit satellite access shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN.

[PR 5.7.6-003] A 5G network with multi-orbit satellite access shall be able to collect charging information for a UE served by different orbit types of satellite access networks.

Editor's Note: the terminology of the above three requirements still needs to be aligned with other part of the TR

## 5.8	Use case on traffic over different orbit satellites

### 5.8.1	Description

This use case describes a scenario in a desert environment where an autonomous vehicle operates under remote control from a central station, facilitated by e.g., satellite-based access. The UE installed on the vehicle is served by two different orbit satellites (e.g., GEO and LEO) belonging to the same PLMN which is managed by a 5G satellite operator. The UE holds a subscription with a single PLMN.

Multi-orbits satellite access is applied to accommodate a high amount of data traffic for remote control operations. For better performance of the autonomous vehicle operations, the delay-sensitive applications (e.g., remote control operations, collision/accident prevention, emergency management, etc.) use LEO satellite link as it has smaller UE to ground propagation delay than GEO satellite connection. Other applications (i.e. delay tolerant, such as SMS, sensor data monitoring, video surveillance, etc.) may use GEO satellite link.

### 5.8.2	Pre-conditions

The autonomous vehicle supports multi-orbits satellite access (e.g., GEO and LEO) provided by same 5G satellite operator.

The UE installed on the vehicle has a subscription for using GEO and LEO satellite services of a 5G satellite operator.

Based on the service agreement between the vehicle managing company and the 5G satellite operator, the 5G satellite network has the following policies for the UE on the vehicle:

- The data traffic of delay-sensitive applications is routed via LEO satellite link whenever it is available.

- The data traffic of delay-tolerant applications is routed via GEO and/or LEO satellite links.

### 5.8.3	Service Flows

- The vehicle is in the desert and is being remotely controlled by the central station. The vehicle has two types of active data traffic – one for remote control operation and the other for SMS.

Figure 5.8.3-1: Multi-orbits 5G satellite access for autonomous vehicle operations

- LEO satellite 1 access becomes unavailable (e.g., due to loss of line of sight between the UE and a satellite 1). Therefore, the traffic that was routed via the LEO satellite 1 access is moved to LEO satellite 2 access or GEO satellite access, while the continuity of data sessions is maintained.

Figure 5.8.3-2: Traffic switching and mobility management due to loss of connection with LEO satellite access network

- After a while, the LEO satellite 1 access becomes available again. Therefore, the traffic distribution across three satellite accesses is returned to the state as in Step 1.

### 5.8.4	Post-conditions

The remote control centre controlled the vehicle throughout its route and the vehicle successfully reached its destination.

### 5.8.5	Existing features partly or fully covering the use case functionality

From TS 22.261:

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g., from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies (e.g., NR, E-UTRA, non-3GPP), to access one or more 3GPP services.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over select between access technologies in use, taking into account e.g., service, traffic characteristics, radio characteristics, and UE's moving speed.

The 5G system shall support UEs with multiple radio and single radio capabilities.

The 5G system shall be able to provide services using satellite access.

A 5G system with satellite access shall support different configurations where the radio access network is either a satellite NG-RAN or a non-3GPP satellite access network, or both.

Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites).

Subject to HPLMN policy and network control, the 5G system shall be able to support mechanisms to enable traffic steering and/or switching of a DualSteer device’s user data (for different services) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN), assuming data anchoring in the HPLMN and non-simultaneous transmission over the two networks.

Subject to HPLMN policy and network control, the 5G system may be able to support mechanisms to enable traffic steering and/or switching with simultaneous transmission of a DualSteer device’s user data (for different services) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN), assuming data anchoring in the HPLMN.

Subject to HPLMN policy and network control, the 5G system shall be able to support mechanisms to minimize service interruption when switching a DualSteer device’s user data, for one or multiple services, between two 3GPP access networks.

Subject to HPLMN policy and network control, the 5G system shall be able to collect charging information related to traffic steering and/or switching of a DualSteer device’s user data across two 3GPP access networks. (6.50.2.3)

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-001] The 5G system with satellite access shall be able to support a mechanism to satisfy the QoS when user data traffic moves across satellites at different orbits.

[PR 5.8.6-002] The 5G network shall be able to collect charging information related to traffic of user data when user data traffic moves across satellites at different orbits.

## 5.9	Use case on Public Warning Service

### 5.9.1	Description

A Public Warning System (PWS) is in place today in many countries and is for example mandatory in all EU Member States as per article 110 of the European Electronic Communication Code [14]. The EECC requires that "public warnings are transmitted by providers of mobile number-based interpersonal communications services to the end-users concerned" which makes it mandatory that PWS is also provided by NTNs.

### 5.9.2	Pre-conditions

Alice is enjoying her vacation in a remote rental vacation house with her friends, Lisa and Allen, in a National Park after graduating from college. Alice decides to take a hike in the mountain area one early morning. She noticed that there is no terrestrial network coverage at the parking lot and her smart phone has switched to satellite access network.

### 5.9.3	Service Flows

Alice has walked for some time when she hears the alerting ringtone of a public warning message. She takes her phone out of her backpack, and she reads the warning message which tells her that a severe storm is making its way towards her, and she should quickly get back.

Alice immediately decides to walk back to her car.

### 5.9.4	Post-conditions

Alice is happy that she received the warning message which allowed her to get back to the vacation house safe and sound.

### 5.9.5	Existing features partly or fully covering the use case functionality

TS 22.268 [15] specifies the requirements for PWS and the service is specified in TS 23.041 [16]. The procedure includes a Cell Broadcast Centre that selects RAN-nodes that cover the alert area for broadcasting the warning message.

It is however not specified how broadcasting is done in an alert area in case the RAN-node (a satellite) is not stationary.

### 5.9.6	Potential New Requirements needed to support the use case

The following requirement could be added to TS 22.268 [15]:

[PR 5.9.6-001] If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via Non-Terrestrial Networks.

## 5.10	Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay

### 5.10.1	Description

With the first commercial deployments of 5G satellite access, there is a strong demand for enhancing 5G emergency communication and mission critical services via satellite (e.g., emergency call for automotive, emergency messaging for handsets or IoT devices). Satellite communication, combined with 5G technology, enhances disaster response by providing real-time data and connectivity during emergencies, enabling swift and coordinated efforts for disaster management. The enhanced support for emergency communication and mission critical services using Satellite Access System marks a pivotal shift in how first responders can approach and handle emergencies. By providing them with reliable coverage, real-time data, enhanced communication capabilities, and advanced tools, 5G satellite access is a lifeline that empowers the public safety actors to save more lifes while reducing their own risk.

In the emergency situation, two important points are to be considered to save more lifes:

- Early alerts:

  - Emergency Notification Systems: Governments/Organizations can implement systems to quickly disseminate alerts to the population during emergencies.

  - Automated Systems: Set up automated alerts triggered by sensors (e.g., fire alarms, seismic detectors) to notify relevant parties instantly.

- Effective Reaction:

  - To be effective on the field and ensure the safety of everyone everywhere, the first responders — firefighters, law enforcement officers, and emergency medical service personnel — need to be complemented by a reliable network for accessing critical and real-time information in the field.

Figure 5.10.1-1 Use case for Mission Critical Services (Firefighter, Research & Rescue) using Satellite Access with Mobile Base Station Relay

### 5.10.2	Pre-conditions

Bob is a trail rider who has organized a hiking trail with a group of friends in the forests of Arizona. During their hike, in the middle of the trail, the forest catches fire due to extreme heat.

Very quickly, IoT sensors placed high up in the forest detect the fire (in direct visibility with satellites) and transmit the alert to the nearest rescue teams.

Bob tries to find a safe place to take shelter with his friends and adjusts the orientation of his phone, and when the signal strength indicator reaches one bar, he sends an emergency message for help, specifying that there is a fire in the woods, with his GPS location and the number of people in the area concerned.

A rescue team made up of ground and aerial firefighters, rescue teams, logistics teams (to set up a local command center) and drones are deployed in the fire zone.

The different intervention groups (Aerial and Ground firefighters) surround the area affected by  the fire and are in direct communication via a multicast channel with the local command center.

Unfortunately, the fire continues to spread, and in the meantime Bob and his friends have moved from the location communicated in the emergency message to find a new shelter.

The advanced drones are deployed equipped with high-resolution cameras and real-time data processing. Those drones can cover vast areas quickly and identify missing persons in emergencies with great efficiency. This technology is particularly useful in challenging terrains like the Arizona Forest, where traditional search methods are less effective.

The Command-and-Control Center is located in a truck with a mobile base station relay. The local Command-and-Control Center takes the mission plans and orders from a remote Command-and-Control Center. In case the link between the Local Command Center and the remote Command Center is not available, the local Command-and-Control Center can make the appropriate decisions and keeps dispatching the orders to the relevant teams.

The local Command-and-Control Center (C2) remotely monitors and controls drones to locate missing persons and assist firefighters and rescue teams in real time. The connectivity between the local C2 Center and the drones is established through 5G satellite access connectivity.

The rescue team commander has a backpack antenna to provide a Mobile Base Station Relay network for his local rescue team.

The firefighting teams have 5G relay antennas to provide a Mobile Base Station Relay network for the firefighters (to receive commands and send updates about the mission).

### 5.10.3	Service Flows

The command team controls the drone using 5G satellite access in real-time to locate the victims. This information is sent directly to the Command Center via satellite communication, where it is processed, filtered and the appropriate decision is made, which is then sent on a multicast channel using 5G satellite access in to the different teams (with several pieces of information, video from the drone, image from the drone, location of the drone, etc.).

These teams communicate with each other to synchronize in real time, and a cache is deployed (locally at the Command Center or at satellite level) to optimize multicast communications.

### 5.10.4	Post-conditions

A network (via satellite) enabling communication between different groups was rapidly established.

A large volume of data is exchanged and processed in real time to ensure the success of the operation.

Once the drone has located the victims, the different teams work in coordination (thanks to real-time group communication) to rescue them.

Using multicast services, the network is able to connect reliably different group of first responders by guaranteeing the expected QoS and optimizing resource allocation.

### 5.10.5	Existing features partly or fully covering the use case functionality

Existing requirement from 22.261 [2]:

The 5G system shall support delivery of cached content from a content caching application via the broadcast/multicast service.

The 5G system shall support configurations of content caching applications in the network (e.g. access network, core network), that provide content close to the UE.

Existing KPIs for services provided to the UAV applications and KPIs for UAV command and control TS 22.125

### 5.10.6	Potential New Requirements needed to support the use case

#### 5.10.6.1	Potential Service Requirements

[PR 5.10.6-001] A 5G system with satellite access, shall support delivery of a cached content from a content caching application via a broadcast/multicast service for mission critical services.

[PR 5.10.6-002] A 5G system with satellite access shall support configurations of content caching applications in the network (e.g., access network, core network), that provide content close to the UE for mission critical services.

[PR 5.10.6-003] A 5G network with satellite access shall be able to support multicast services via Mobile Base Station Relays using satellite access.

[PR 5.10.6-004] A 5G system with satellite access shall be able to support C2 communication of UAV operation with required QoS via a mobile base station relay using satellite access .

NOTE:	QoS is end-to-end QoS, with respect to KPIs for command and control as defined in table 7.2-1 TS 22.125.

#### 5.10.6.2 	Potential KPI Requirements

Table 5.10.6.2-1	Potential KPI for Video data over satellite access

| Scenario | Type of data | Function | Experienced data rate | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | End-to-end Latency | Max UE ground speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UAV | Video data over satellite access (used for Search and rescue and aid to UAV non-VLOS C2) | UAV terminated C2 | 300 kbit/s | TBD | TBD | TBD | 40ms | 60km/h | UAV mounted |
|  |  | UAV originated C2 | 6 Mbit/s | TBD | TBD | TBD | 140ms |  |  |
| Note 1: Area capacity is averaged over a satellite beam |  |  |  |  |  |  |  |  |  |

## 5.11	Use case on emergency communication using satellite access

### 5.11.1	Description

The necessity of satellite-enabled emergency communication cannot be overstated, especially in remote and inaccessible areas. In scenarios such as natural disasters, where traditional communication networks are disrupted, satellite access provides a crucial lifeline for victims in distress. It allows for rapid response and coordination of rescue efforts, ensuring that help reaches those in need as quickly as possible.

Moreover, satellite-supported emergency communication is invaluable for individuals venturing into remote locations, whether for work or exploration. In the event of an accident or unexpected emergency, a satellite-enabled emergency communication can swiftly trigger rescue operations, significantly improving the chances of survival.

The significance of this function enhancement lies in its ability to bridge the gap between vulnerable individuals and emergency services, ensuring that help is always available, regardless of location. As technology advances, the reliability and accessibility of satellite-enabled emergency communication will continue to grow, becoming an increasingly important safety network for our modern world.

### 5.11.2	Pre-conditions

Allen, Bob and Cable are all explorers and their phones have subscription with the terrestrial operator T_A.

S_A and S_B are different satellite companies and maintain serving satellites for the 5G users all over the world.  Some places of the desert are the S_A's serving area and some other places are the S_B's serving area.

T_A has service agreement with the satellite operator S_A and S_B for satellite access.

### 5.11.3	Service Flows

Figure 5.11.3-1: Emergency communication using satellite access

1. Allen and Bob are exploring along the planned route in the desert where is no cellular coverage. And they have connected to S_A network via satellite access.

2. Suddenly, the sandstorm is coming, Allen and Bob are lost with each other, and they left the planned route and lost in the desert.

3. After the sandstorm, Allen finds a safe place under the coverage of S_B and tires to makes emergency call for rescue. In another place within S_A's serving area, Bob tries to make the emergency call.

4. The network system of S_A and S_B provides the precise location information of Allen and Bob, recognizes the emergency calls, and forwards the call to public emergency rescue center .

5. The rescue center initiates search and rescue operation based on the received location information.

6. The public emergency rescue center triggers the emergency notification message broadcast in the area based on the received location information to inform the persons who also in the same area to notice if they can find Allen or Bod along their way and give them help.

7. Cable is the same area with Allen and receives the emergency notification message. Along his way, he finds Allen and gives him help.

### 5.11.4	Post-conditions

With the support of the network system of S_A, S_B and T_A, Allen and Bob are rescued safely.

### 5.11.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.46.2 on satellite access includes the following requirements:

A 5G system with satellite access shall be able to determine a UE's location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE.

NOTE:	This is also applicable for UE using only satellite access. The determination of a UE’s location can be based on 3GPP and/or non-3GPP positioning technologies subject to operator’s policies.

3GPP TS 22.261 [2], clause 6.13.2 on broadcast using satellite access includes the following requirements:

The 5G system shall support multicast/broadcast via a 5G satellite access network, or via a combination of a 5G satellite access network and other 5G access networks.

### 5.11.6	Potential New Requirements needed to support the use case

[PR 5.11.6-001] Subject to the regulatory requirements and operator’s policy, the 5G system should be able to support means to provide smaller granularity location information (compared with original cell information in serving satellite) of emergency communication user  to the trusted 3rd party when the user triggers the emergency communication via satellite access.

## 5.12	Use case on supporting different services with multi-orbit satellites

### 5.12.1	Description

The control center of the Meteorological Bureau monitors Earth's atmosphere, clouds, oceans and other aspects of weather through the data and images returned by satellites for weather forecasting and nature disaster prediction. Ministry of Natural Resources monitors earthquake, fire disaster, typhoon, hurricane, sandstorm, cold wave and other natural hazard through satellite for weather forecasting and nature disaster prediction.

Satellites for weather forecasting and nature disaster prediction take a set of high-resolution meteorological images at regular intervals, which clearly show the distribution, thickness and shape of cloud cover, topography. These images are rapidly transmitted back to the ground, where experts from the Meteorological Bureau analyze them to accurately predict the trend of weather systems. Experts of Ministry of Natural Resources could analyze these pictures to predict and monitor natural hazard accurately before it happens. There are a group of  satellite for weather forecasting and nature disaster prediction at LEO. Usually, they transmit these pictures back to ground. Application server on the ground will proceed data processing and analysis based on requirements of experts. Since these images are high-resolution and are generated at about 1.5G/s, which need very high bandwidth to transmit back to ground, it is more efficiency to process and analysis on the satellite and send the results to ground. However, as the load on satellite is limited as it equipped with a lot of high-resolution cameras, it’s not very practical for satellite to equip large amount of computing resource. Communication satellites are distributed in space on LEO, MEO, and GEO, they are more convenient to facilitate computing resource. And with the bandwidth at about 15-25Gb/s on ISL, satellites for weather forecasting and nature disaster prediction can send the data to communication satellites and 5G system is able to provide Service Hosting Environment on satellite for the data processing.

### 5.12.2	Pre-conditions

Operator A has multiple LEO satellites and also GEO satellites, servers of Ministry of Natural Resources are deployed in the edge application server on two LEO satellites of operator, and servers of Meteorological Bureau/Ministry are deployed in the edge application server on three GEO satellites. There are a group of LEO satellites for weather forecasting and nature disaster prediction to monitor same area which are also operated by Operator A.

Weather analysis service requires large number of data for analysis such as high-resolution pictures which take very large bandwidth of satellite, it’s difficult to transmit all the data at one time for LEO for weather forecasting and nature disaster prediction. Natural hazard service requires high reliability.

### 5.12.3	Service Flows

Figure 5.12.3-1  Satellite for weather forecasting and nature disaster prediction

1.Satellites for weather forecasting and nature disaster prediction take a set of pictures of earth’s atmosphere, clouds and topography with high definition.

2.As the pictures are high-resolution, and satellites are LEO, the data cannot be transmitted to ground at one time. So satellites for weather forecasting and nature disaster prediction send the data to the edge application server which is deployed on operator’s LEO to do on-board processing, so that only the processing result will be send to the ground which takes lower bandwidth.

3a.5G system select the most closely EAS on the operator LEO satellites. High-resolution pictures will be send to the EAS of Meteorological Bureau on the operator LEO via inter-satellite link.

3b.5G system select most closely EAS on the GEO. High-resolution pictures of all these satellites for weather forecasting and nature disaster prediction will be send to the EAS of Ministry of Natural Resources on the operator GEO via inter-satellite link.

4.EAS sends result to Meteorological Bureau/Ministry of Natural Resources on the ground.

5a.Alice, an expert of Meteorological Bureau, receives analysis results and part of pictures.

5b.Bob, an expert of Ministry of Natural Resources, receives analysis results and part of pictures.

### 5.12.4	Post-conditions

Alice receives analysis results and publish to citizens. It will be rainy tomorrow.

Bob receives analysis results and notify to related department of fire disaster.

### 5.12.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261, clause 6.5.2 on Service Hosting Environment:

The 5G network shall enable a Service Hosting Environment provided by operator.

3GPP TS 22.261, clause 6.3.2.3 on satellite access includes the following requirements:

The 5G system shall be able to provide services using satellite access.

3GPP TS 22.261, clause 6.5 on satellite access includes the following requirements：

A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfills the agreed QoS

### 5.12.6	Potential New Requirements needed to support the use case

[PR 5.12.6-001] Based on regulatory requirements and operators’ policy, the 5G system shall support Service Hosting Environment (e.g., edge application server) on multi-orbit satellites.

[PR 5.12.6-002] Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system shall support a mechanism to provide most suitable service hosting environment via multi-orbit satellites.

NOTE: 	One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits.

## 5.13	Use case on wide-area disaster control support using multi-orbit satellite networks

### 5.13.1	Description

5G systems utilize a blend of terrestrial and non-terrestrial networks (NTN) to deliver seamless connectivity. NTNs employ various satellites, including Geostationary Earth Orbit (GEO), Medium-Earth Orbit (MEO) and Low-Earth Orbit (LEO) satellites, to extend coverage and serve users effectively. NTN systems play a critical role in tracking and monitoring over wide coverage areas. These networks provide consistent, real-time surveillance and communication capabilities in such regions where traditional terrestrial infrastructure is either lacking or insufficient to cover wide area consisting of regions such as isolated, disaster-prone, or conflict-affected zones. By leveraging NTNs, authorities can monitor environmental changes, track the movement of objects, and detect early warning signs of natural disasters or security threats. The ability of NTNs to operate independently of ground-based systems ensures uninterrupted coverage, even in the most challenging conditions.

According to TS 22.261, LEO satellites offer an end-to-end latency of 35 ms, while MEO and GEO satellites have latencies of 95 ms and 285 ms, respectively. Based on these numbers, we can conclude that GEO satellites can provide continuous coverage over a large area with fewer satellite switches and LEO satellites can support services with low latency requirements. 5G systems with GEO satellite can support wireless sensing services that detect and collect data on environmental conditions, such as rainfall and flood levels. It can enable alerts to be sent to response systems immediately using connectivity through LEO satellite.

Availability of multiple-orbit satellite networks with a combination of wide area support and low latency support makes them invaluable to support disaster monitoring, border security, and environmental protection use cases, where timely actions can be taken to mitigate risk and provide rapid response.

### 5.13.2	Pre-conditions

For tracking and monitoring in wide coverage areas, connectivity is provided via MEO/GEO satellites for UEs in these areas. The 5G system enables switching from MEO/GEO to LEO for time-sensitive emergency communication.

### 5.13.3	Service Flows

- Sensing devices are deployed in a flood-prone area to detect flood conditions.
Note: For environment monitoring, the maximum sensing service latency is 60000ms, as specified in TS 22.137, which is fulfilled by GEO/MEO satellites.

- The devices are connected to MEO/GEO satellites for wider coverage to report its sensing information, which also conserves battery life due to fewer satellite switches.

- When a flood condition is detected by the sensing device, it switches its connectivity to a LEO satellite to start communicating with disaster relief support team.

- The LEO satellite connection enables low-latency communication and provides faster communication with the disaster relief support team.

### 5.13.4	Post-conditions

By switching between different multi-orbit satellites, low latency services can be provided based on specific needs.

### 5.13.5	Existing feature partly or fully covering use case functionality

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency. [2]

NOTE 1: 	5 ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency. [2]

NOTE 2: 	5 ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency. [2]

NOTE 3:	 5 ms network latency is assumed and added to satellite one-way delay. The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g., a cell sector, a cell or a group of cells).

A 5G satellite access network connected to 5G core networks in multiple countries shall be able to meet the corresponding regulatory requirements from these countries (e.g., Lawful Interception). [2]

A 5G system with satellite access shall be able to determine a UE's location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE. [2]

NOTE:	This is also applicable for UE using only satellite access. The determination of a UE’s location can be based on 3GPP and/or non-3GPP positioning technologies subject to operator’s policies. [2]

Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite. [2]

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing. [17]

### 5.13.6	Potential New Requirements needed to support the use case

[PR 5.13.6-001] Subject to regulatory requirements and operator policies, the 5G network with multi-orbit satellite network shall support switching between satellite access depending on the latency requirements.

NOTE: 	The switching between satellite access of a multi-orbit satellite network can be triggered by latency requirements.

## 5.14	Use case on broadband services through multi-orbit satellite access

### 5.14.1	Description

There are different kinds of satellites at different orbit altitudes, like GEO at about 36000 km from Earth, MEO between 5000 and 20000 km and LEO satellite between 500 and 1500 km distance from Earth. These satellite altitude differences lead to different performance. At the same time the satellite movement is also different for the different altitudes, where GEO satellites do not move relative to Earth while for instance LEO satellites at 600 km distance from Earth move with 7.5 km/s.

At a remote summer cottage, cameras are installed. Their purpose is twofold: first of all, they are used to keep an eye on the house and secondly, they monitor the wildlife. The summerhouse is outside terrestrial coverage, and the cameras are connected through satellites. The owners of the summerhouse can remotely watch the camera streams. The cameras have movement sensors and can be turned off if there is no movement to save energy, while they also can be woken up by the remote end user to share the current view.

In order to save further energy and being more resilient and available, the cameras/devices connect to GEO satellites such that reselections over LEO satellites are avoided, while when they are sharing a camera stream or picture, they will switch the connection to LEO satellites in order to benefit from larger capacity and lower latency.

### 5.14.2	Pre-conditions

Safety cameras connected via satellites in a remote area without terrestrial coverage, which need maximum power savings. GEO satellites and LEO satellites are in the same PLMN.

### 5.14.3	Service Flows

Broadband data service flow through multi-orbit satellite access as illustrated in Figure 5.14.3-1:

- Camera connects to a GEO satellite.

- Camera gets informed through a GEO satellite to wake up.

- Camera changes the connection to LEO_1 satellite.

- Camera shares stream/pictures via LEO_1 satellite.

- Upon completion of the sharing, camera switches off and connects back to the GEO satellite.

Figure 5.14.3-1: Broadband data service through multi-orbit satellite access

### 5.14.4	Post-conditions

Stream/pictures are shared and camera saves power.

### 5.14.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.3.2.3 on satellite access includes the following requirements:

The 5G system shall be able to provide services using satellite access.

3GPP TS 22.261 [2], clause 9.1 on charging includes the following requirements:

The 5G core network shall support collection of charging information based on the access type (e.g. 3GPP, non-3GPP, satellite access).

However, it is not sufficient in regards of multi-orbit satellite access especially for the request of services through a GEO satellite, but react of the request via a LEO satellite.

### 5.14.6	Potential New Requirements needed to support the use case

[PR 5.14.6-001]	Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support a UE to connect with NGSO satellite access for data transmission, but be served via a GSO satellite of the same network before and after the data transmission.

## 5.15	Use case on multi-orbit access for cross-hybrid service

### 5.15.1	Description

Each orbital altitude in satellite networks brings unique benefits to the overall ecosystem. While each satellite network can operate independently, integrating them into a unified network in space and on the ground offers significant advantages. The market is full of satellite network operators with diverse plans, some that seek to leverage the benefits of a single orbit and others that take a multi-orbit approach or seek partnerships with each other and with terrestrial mobile network providers as well.

With the surging demand for global connectivity, multi-orbit satellite solutions offer tremendous opportunities, providing global coverage that complements terrestrial networks and allowing mobile network operators to deploy hybrid solutions (using GEO and NGSO) for ubiquitous coverage and higher capacity. This cross-hybrid approach enhances 3GPP system utilization and extends the benefits of 3GPP standards. Broadening these cross-hybrid solutions in 3GPP standards can accelerate the growth of Non-Terrestrial Networks (NTN) and outperform other technologies without 3GPP system support. In addition, cross-hybrid solutions provide opportunities to enable switching communication paths among GEO, MEO, and LEO to match application needs, provide resilience, and increase coverage.

In the coming years, we will start to see cross-hybrid solutions where we have GEOs—like we are used to—complemented by Low Earth Orbit (LEO) and Medium Earth Orbit (MEO) satellites. With such multi-orbit approach, LEOs deliver high capacity and low latency, while GEOs bring ubiquitous coverage at the lowest possible cost wherever its needed—especially in exurban and rural areas with limited or no terrestrial access. We’ll then see the communication path determination, e.g., whether the bits transmitted will go over GEO, LEO, MEO—or even over terrestrial technologies, based on application need.

### 5.15.2	Pre-conditions

Alex, who lives in the city, is a subscriber of EchoTel, a network operator that provides satellite solutions utilizing both GEO and NGSO satellites. EchoTel’s network is seamlessly integrated with its terrestrial mobile system, offering a unified service. Additionally, EchoTel has roaming agreements with other mobile operators including Nex and Foos.

### 5.15.3	Service Flows

Alex decided to go away for the long holiday weekend with friends in a remote national park where coverage can be quite limited. One morning, Alex turns on her smartphone, which acquires a valid GNSS position. Wanting to reassure her mother that she and her friends are safe and enjoying their time, Alex decides to send a text. Given the low data rate required for the message, the network identifies the appropriate orbit and connects Alex to the GEO satellite in the region. Despite the marginal quality, the message is successfully sent, and her mother responds, encouraging Alex to enjoy the weekend

On her way back from the national park, Alex reached a rural area where the terrestrial network was still unavailable. She wanted to call her mom to let her know she on her way back to the city. When Alex placed the call, the network determined that her voice call could be handled by the LEO satellite. She spoke to her mom for five minutes, who was glad to hear that Alex was on her way home

As soon as Alex got back to the city, she wanted to send a video to her friends about their weekend adventures. The network quickly connected her to a nearby terrestrial base station, so she could easily share the video and some high-quality photos with her friends

In this use case, the network assesses the service level requirements of users (e.g., data-rate, latency, etc.) and determines the most suitable orbit to meet the expected grade of service. This approach enables effective management of capacity and quality, aligned with the anticipated service standards.

### 5.15.4	Post-conditions

The UE connects to satellite services based on the required grade of service. It utilizes GEO satellites for non-latency-sensitive applications, offering a more effective connection. In areas where terrestrial coverage is unavailable, the UE leverages LEO satellites for high-capacity, low-latency connectivity. When a higher grade of service is needed, the UE switches to terrestrial networks.

### 5.15.5	Existing features partly or fully covering the use case functionality

Requirements in 22.261 Rel19 [2] that partly cover the described use case are:

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

- A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

### 5.15.6	Potential New Requirements needed to support the use case

[PR 5.15.6-001] A 5G system with multi-orbit satellite access shall be capable of selecting communication paths across different satellite access (GEO, MEO, LEO) as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service.

## 5.16	Use case on using GEO for initial network entry and service initialization

### 5.16.1	Description

GEO satellites have a much wider field of view than NGSO satellites and typically have longer active duty cycles than NGSO systems.  They can be used to transmit cell broadcast signals and system information on a very large coverage area very efficiently, where a GEO satellite cell coverage area may be of a size that is equivalent to one or several NGSO satellite footprints areas (and thus to several NGSO cells). Cell search and network entry can be a simpler process using GEO satellites, while cell search and network entry can be an extended process for a UE served by NGSO satellites exclusively.

Furthermore, satellites often present significant payload power budget constraints, which can be more severe for NGSO satellites. In such scenarios, an NGSO satellite’s active coverage footprint might be selectively reduced in order to conserve power.  This has however the effect of creating coverage holes under the footprint of the satellite, in which cells may not be activated.

The presence of a GEO satellite coverage overlay on top of NGSO allows to bridge coverage gaps by providing broadcast signal coverage and initial access for the network.  In this mode, we assume that upon power-on, a UE will perform initial cell search, initial access and network registration (also known as “network entry”)  into a GEO satellite.  During the initial access and registration procedure the network may choose to serve the UE via an available NGSO satellite.  The NGSO satellite may be instructed by the network to activate a dormant beam or cell with a specific set of resources in the targeted coverage area, or the UE may be instructed to join an existing NGSO beam or cell.

This concept can be generalized from GEO and NGSO to multiple satellite access types but the concept is most easily understood and justified when considering GEO and NGSO.

### 5.16.2	Pre-conditions

- The network supports multiple satellite access types

- The network supports mobility between multiple satellite access types

- A network plan that deconflicts frequency allocations between the component satellite access types.

- The networks support network entry initiation on one satellite access of the network and network entry completion on a different satellite access network.

### 5.16.3	Service Flows

- UE is turned on and wants to establish a connection

- UE does initial network entry on a GEO satellite (or more generally a satellite of the appropriate satellite access type)

- The system provides the UE enough information to enable network entry completion on a satellite of the NGSO (or more generally a satellite access of the appropriate satellite access type)

- UE does network entry complete on an NGSO (or more generally a satellite access of the appropriate satellite network type)

- UE does network entry completion on an NGSO spot beam

### 5.16.4	Post-conditions

The UE is registered on the preferred satellite access from the network with satellite access supporting multiple satellite orbits types with different characteritics (e.g.; altitude, orbital characteristics, satellite capabilities) and carries out communication on that satellite network.

### 5.16.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-001] A 5G network with satellite access supporting multiple satellite orbits types with different characteritics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics.

Editor’s note: this requirement is FFS

[PR 5.16.6-002] A 5G network with satellite access supporting multiple satellite orbits types with different characteritics (e.g.; altitude, orbital characteristics, satellite capabilities) shall enable the UE to select the relevant satellite with a given characteristics for the initial connection to the network.

Editor’s note: this requirement is FFS

## 5.17	Use case on  resource and energy efficiency in a multi-orbit Satellite Access System

### 5.17.1	Description

Satellite access in a 5G system aims in bridging connectivity gaps, especially in remote areas thanks to the following characteristics:

- Global Coverage: Satellites can provide coverage across vast geographical regions, including oceans, deserts, and polar regions.

- Ubiquitous Connectivity: Satellite networks ensure continuous connectivity, even areas with sparse population. This makes them ideal for emergency communication, disaster response, and remote monitoring.

Satellite-based 5G networks can be served by satellites that are operating in either NGSO (LEO & MEO) and/or GEO. A multi-orbit satellite access system is a network that is served by satellites that operate in different orbits.

Satellite-based networks come with additional challenges compared to terrestrial networks, and each of the aforementioned orbits comes with its advantages and disadvantages.

The biggest challenge is that in space, satellites operate in a power limited environment, relying on solar panels and batteries that store the excess energy produced by the solar panels. In principle, satellites in NGSO (LEO & MEO) are more compact compared to GEO satellites, limiting the size and capacity of  solar panels and batteries of the former compared to the latter.  Furthermore, the exposure of the LEO and MEO satellites to sunlight varies significantly due to orbital dynamics. In addition, LEO and MEO satellites experience  atmospheric drag, which is higher for LEO compared to MEO. This necessitates frequent orbital adjustments, which in turn consume energy.

At the same time, NGSO satellites, due to their proximity to earth, can provide a superior capacity and lower latency, compared to GEO satellites,  playing a vital role in providing connectivity to  maritime vessels, aircrafts, offshore platforms, and remote islands.

On the other hand, the proximity to earth of NGSO comes with the price of very high orbital speeds, up to 7.5Km/s,  making cell search and camping a tedious process, especially for VSATs (Very Small Aperture Terminal),   which can take up to tens of minutes. In addition, the high orbital speed reduces the time that a UE is served by an NGSO satellite triggering frequent mobility events and associated control plane signalling.

A 5G system with multi-orbit satellite access shall exploit the advantages of each orbit to ensure optimal energy and resource efficiency of the satellite access network.

For example to achieve such energy efficiency,  NGSO satellites can be intelligently turned off or the power of specific beams be reduced, when a lower threshold of connected users and/or throughput is reached, while the same area can still be served through lower capacity GEO satellites.

Similarly, to achieve resource efficiency when a  UE (e.g., VSAT) is turned on it can connect to a GEO satellite and stay connected there until it needs to transmit data, in order to avoid a long NGSO satellite searching process and unneeded overhead control plane signalling required for frequent NGSO mobility events.

### 5.17.2	Pre-conditions

During the beginning of the tourist season in the Caribbean, a large cruise ship departs from its first port full with passengers and crew. The ship normally uses a connection with a GEO/NGSO satellite operator.

During the off-peak season the operator has switched off the NGSO beams over the path of the cruise ships to reduce energy consumption and prolong the  satellites’ life.

The VSAT UE onboard has a list of TLE coordinates of the operators’ GEO satellites

### 5.17.3	Service Flows

According to this use case:

- The VSAT UE onboard the ship is powered on and  based on its current position, determines which GEO satellite it should connect to and quickly establishes a connection.

- Once it connects to the GEO satellite, more passengers on the ship and connect to the WLAN onboard leading to higher data demand.

- The satellite operator, through automated O&M, needs to turn on NGSO satellites that serve the region to meet the demand.

### 5.17.4	Post-conditions

The satellite operator has successfully determined which beam/satellite from the NGSO constellation needs to be turned on. The VSAT UE has successfully established a link access to the network first through the GEO and then through the  NGSO satellites.

The VSAT returns to the GEO satellite during times of the day that passenger demand is dropping, and the satellite operator is able to switch off the NGSO satellites over the area to optimize the systems’ energy performance.

The quick initial connection to GEO and seamless mobility to NGSO as data demand increased ensured a positive connectivity for passengers and crew onboard the ship, while ensuring a resource and energy efficient operation of the satellite access system.

### 5.17.5	Existing features partly or fully covering the use case functionality

Requirements in 22.261 Rel19 [2] that partly cover the described use case are:

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to satellite one-way delay.

### 5.17.6	Potential New Requirements needed to support the use case

[PR 5.17.6-001] A 5G network supporting satellite access with multi-orbit operation shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand.

[PR 5.17.6-002] A 5G system supporting satellite access with  multi-orbit operation shall select satellite access type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand.

## 5.18	Use case on using GEO to land data in a desirable geographical location

### 5.18.1	Description

NGSO satellites typically have a much smaller field of view (FOV) than a GEO satellite.  Thus NGSO satellites have to either download traffic from a UE to a gateway within its FOV, or transfer via inter satellite links (ISL) the data to another NGSO satellite in order to land the data outside the FOV of the satellite connecting to the UE.  These ISLs sometimes often do not have as much bandwidth as the primary links.  ISLs may also be unavailable under various conditions due to satellite duty cycle constraints or routing conflicts.

It is also desirable for many customers to download data to network based in a specific geographic areas.  This may be for intellectual property, security, or national security requirements.  Many customers may want to avoid areas that belong to adversarial nations or have geopolitical instability. We will refer to this as data with a preferred satellite ground station location.

Using a multi-orbit satellite system allows differentiated data landing based on customer needs.  E.g., the UE can connect to a NGSO and then have its data sent to a GEO satellite from which the data can be landed anywhere in the field of view of the GEO.  It may also be the case that the UE with data with preferred satellite ground station location, directly communicates with a GEO satellite bypassing the NGSO entirely.

### 5.18.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The network must be able to determine which data has a preferred satellite ground station location.

- The network must be able to determine whether each satellite network has access to the preferred satellite ground station location.

### 5.18.3	Service Flows

- During communication, the network or UE determines that data will be exchanged that has a preferred satellite ground station location

- The network shall determine the optimal path to route traffic and send traffic through that path.

### 5.18.4	Post-conditions

Data with preferred satellite ground station location is received securely at preferred location(s).

### 5.18.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.18.6	Potential New Requirements needed to support the use case

[PR 5.18.6-001] A 5G system with multi-orbit satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location.

NOTE: 	The link between satellite and satellite ground station location is out of 3GPP scope

Editor's Note:	it is FFS how it is related to multi-orbit.

## 5.19	Use case on using GEO for sleep/power management

### 5.19.1	Description

Many UEs are power limited and need to conserve power to extend battery life.  In some cases, UEs receive (or have to send) packets very infrequently.  In these cases, it is wasteful of the UE to continuously maintain and establish sync with a NGSO satellite that needs to be tracked through the sky.  A GEO satellite can be very useful for these UEs.  The UE would only try to connect to a NGSO satellite if the GEO satellite told it to do so.  The GEO satellite would only tell the UEs to connect to the NGSO if the network informed the GEO that packets were waiting for the UE.  In cases where the UE needs to send a packet, it would either send it directly to the GEO, or request (via the GEO) resources from the network on a NGSO satellite.

### 5.19.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports mobility between GEO satellite(s) access and NGSO satellite(s) access

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the different satellite access types

- The network should be aware of when to tell the UE to move to a NGSO satellite connection.

### 5.19.3	Service Flows

- UE establishes a connection with a GEO satellite

- UE enters sleep mode

- UE wakes ups periodically and re-establishes connection with a GEO

- GEO informs UE of whether it needs to wake up

- If there is no need to wake up, UE goes back to sleep

- If there is a need to establish a connection, UE exists wakes up, and establishes a connection with NGSO instead of the GEO

### 5.19.4	Post-conditions

UE is in appropriate sleep mode on GEO or monitoring communications mode on NGSO

### 5.19.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.19.6	Potential New Requirements needed to support the use case

[PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the network to inform a UE when coverage is available and in which satellite system.

Editor’s note: this requirement is FFS

## 5.20	Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR)

### 5.20.1	Description

Satellite access is a communication technique for provisioning of connectivity in rural and remote areas not covered by terrestrial access networks. Satellite access could be provided through satellites located in various orbits e.g. GEO, MEO, LEO. Each of these orbits has its own specificities related to coverage, data rates, capacity and latency. Similarly different services require different access network capabilities. In line with this, services requiring low latency and high data rates would optimally be served via LEO satellite access while delay tolerant services (e.g. txt, messaging) could be routed through a GEO satellite access. Simultaneously combining multi-orbit satellite access links (LEO/MEO/GEO) could be of interest in cases when a single satellite access cannot meet the prescribed requirements (e.g., capacity, latency). Specific characteristic of each satellite access needs to be taken into account when combining it with another backhaul link type in order to achieve required QoS of each user within a group of users (aggregate QoS).

Figure 5.20.1-1 shows an example of multi-orbit satellite access where access  links can be established through GEO and NGSO satellite constellation. Simultaneous usage of multi-orbit satellite accesses serves a mobile base station relay (MBSR) located on the airplane (or possibly on the ground on a remote location).

Figure 5.20.1: Illustration of multi-orbit satellite use-case simultaneously exploiting GEO  and NGSO satellite access links serving mobile base station relay (MBSR) on board satellite. ISL indicate intersatellite links.

### 5.20.2	Pre-conditions

MLK is an airline company known for its high end on-board services. MLK takes good care of its passenger satisfaction and intends to upgrade its on-board entertainment services with on-line messaging, internet access, video streaming and gaming. MLK is looking for connectivity solutions that will enable the upgrade of its on-board entertainment. Cost efficiency of the connectivity solution is of great importance for MLK in order to remain competitive on the tight airline market.

### 5.20.3	Service Flows

A satellite operator SAOP having GEO and NGSO satellites in its satellite fleet offers satellite access connectivity solutions for the airplanes in MLK fleet anywhere around the globe. SAOP offers the most cost efficient solution for MLK as the different service types e.g., messaging, internet, video/gaming, will transfer their traffic over satellite access links optimal for the selected service e.g., GEO, MEO, LEO, respectively.

While on the ground the mobile base station relay (MBSR) mounted on an airplane is connected to the 5G core through a terrestrial mobile access network. The passengers on board the airplane acquire wide variety of services (latency sensitive, latency tolerant, messaging, video or gaming) through the terrestrial mobile access network and the MBSR.

After the take-off, the mobile base station relay mounted on the airplane loses the connectivity through terrestrial mobile access and switches over to satellite access through multiple satellite orbits i.e. LEO, MEO and/or GEO satellites. Satellite access through distinct orbits have different properties with respect to latency, data rate, spectrum, coverage and availability of the satellites in different orbits. 5G system is capable of differentiating traffic among various service types of the airplane passengers. For example, 5G system will route latency tolerant services’ traffic (e.g., messaging) through GEO satellite access, latency sensitive services’ traffic (e.g., mission critical communication, on-line gaming gaming) carried via a mobile base station relay through LEO satellite access and video traffic  carried via a mobile base station relay on demand through MEO satellite access.

### 5.20.4	Post-conditions

MLK improved offer of their services in a cost efficient manner. MLK is able to provide wide range of services (latency sensitive, latency tolerant, high data rate and high reliability) by simultaneously exploiting multiple satellite access paths.

### 5.20.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] specifies the requirements for satellite backhauling i.e. that the 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g., on the latency and bandwidth of the specific backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

The above listed requirement are focused on usage of a single satellite orbit. The cases of possible simultaneous usage of multi-orbit satellite connectivity are not explicitly considered.

### 5.20.6	Potential New Requirements needed to support the use case

[PR 5.20.6-001] The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbits shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the multi-orbit satellite accesses (e.g., over GEO, MEO, LEO) to provision required aggregated QoS for traffic relayed via a mobile base station relay.

[PR 5.20.6-002] The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbits shall be able to collect and distinguish charging information related to the multi-orbit satellite access  traffic carried via a mobile base station relay.

Editor’s note: whether the use of simultaneous connectivity to multi orbit satellite for MBSR is within the scope of the FS_5GSAT_Ph4 is FFS

## 5.21	Use case on combined terrestrial and multi-orbit satellite access for connecting MBSR and 5GC

### 5.21.1	Description

A mobile base station relay (MBSR) is connected with 5G Core network (5GC) via radio access network. Combining multiple radio access paths/types is needed in cases when a single access link cannot meet prescribed performance requirements (e.g. data rate) demanded by the MBSR. An example of such a use-case emerges when the link between 5GC and MBSR is established by simultaneously combining terrestrial and at least two multi-orbit satellite access links as indicated in the Figure 5.21.1-1. Specific characteristic (QoS) of each constitutive access link needs to be taken into account when combining it with another access link in order to achieve required aggregate QoS.

Figure 5.21.1-1: Illustration of how mobile base station relay (MBSR) combines terrestrial and multi-orbit satellite access simultaneously using terrestrial access link, NGSO satellite access  and a GEO satellite access to connect to the 5G core (data network).

- BlueWave is a mobile network operator on the tropical Blue Island. The island is a remote part of the Key Kingdom. Local government of Blue Island promoted improvement of telecom infrastructure on the island as well as the connectivity with the mainland. In line with this ambition the local government invested significant amount of money into submarine telecom cable connectivity with the mainland. BlueWave mobile operator made use of the availability of broadband submarine telecom cable connectivity and started offering high speed mobile data and video services for affordable prices. This resulted in large number of new mobile network subscriptions and rise of BlueWave profit.

Large cruise ships often visit Blue Island due to its natural beauties. Concordia Costa cruise ship recently visited the island staying in the harbour during the night. The captain and the officers enjoyed the night stay on the island and visited some clubs having great time. The next day the ships started departing the harbour and was already few hundred meters away from the shore. The captain had a bit of hangover and was feeling dizzy. The captain leaned on the command console and by chance pressed the button that released the anchor. The anchor fell to the sea bottom. As the ship was moving the anchor damaged some of the broadband submarine telecom cables. This immediately affected the quality of mobile services of the BlueWave.

BlueWave immediately started searching for a solution how to acquire and assure sufficient bandwidth in order to provide to the end customer the QoS specified in the Service Agreements.

- GreenWave is a mobile network operator offering services on the tropical Green Island. Green Island is making part of the Key Kingdom with the mainland 5000km away. The island is connected to the mainland through submarine cables placed 60 years ago. Capacity of the submarine cables placed long ago is lower than the current needs on the island. Government of the Green Island does not have sufficient means to lay a new set of submarine cables in order to increase data speeds on the island. Green Wave is investigating possibilities on how to increase the speed of the broadband backhaul in order to be able to provide mobile broadband data and video services that are in high demand by the existing customers.  ,

### 5.21.3	Service Flows

Alice is in a taxi bringing her to the local sea port. She is verifying the latest news on her mobile phone and subsequently watches the latest episode of her favourite soap opera. Alice is using her broadband connectivity she is acquiring through a subscription with Green Wave. Green Wave is a mobile network operator providing voice and broadband data/video services in the Key Kingdom and its overseas territories. Alice arrives in the harbour and boards the cruise ship with the destination Green Island. She is looking forward having some rest on-board the ship before enjoying the tropical Green Island. During the voyage Alice makes use of WiFi connectivity provided on the ship. After a few days Alice arrives to Green Island. She disembarks the cruise ship and goes for an excursion to the Paradise Waterfalls. While on the island Alice uses broadband connectivity provided by the GreenWave. She is sharing the  beauties of the island by sending to her friends live recordings during her excursions.

A satellite operator SAOP having GEO and NGSO satellites in its satellite fleet offers broadband satellite access connectivity solutions. SAOP offers cost efficient solutions on the pay-as-you-go basis. The government of the Green Island has insights in the planned visits of the cruise ships in the coming 12 months. Green Wave acquires this information from the local government and uses this information to assure sufficient network capacity to provide high speed connectivity to the inhabitants of the island as well as to the visiting tourists. As the submarine backbone has limited capacity Green Wave makes agreements with SAOP that will provide additional capacity through its GEO and NGSO satellite access.

The overall concept Green Wave is using represents a combination of terrestrial radio access (provided via submarine cables) and satellite (multi-orbit) satellite access. In order to minimize costs Green Wave needs to optimally match network traffic types (messages, data, photos, video, latency tolerant and delay sensitive) with the capabilities of each access link (terrestrial; GEO and NGSO satellite).

### 5.21.4	Post-conditions

Green Wave is capable of offering high quality services in a cost efficient manner. Green Wave is able to provide wide range of services (latency sensitive, latency tolerant, high data rate and high reliability) by simultaneously exploiting terrestrial and at least two multi-orbit satellite access paths.

### 5.21.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] specifies the requirements for satellite backhauling i.e. that the 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g. on the latency and bandwidth of the specific backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

The above listed requirement are focused on usage of connectivity through a single satellite orbit. The cases of possible simultaneous usage of at least two multi-orbit satellite access paths in combination with terrestrial access to connect MBSR and 5GC is not explicitly considered.

### 5.21.6	Potential New Requirements needed to support the use case

[PR 5.21.6-001] The 5G system shall support connectivity between a mobile base station relay (MBSR) and the 5G Core through simultaneously using terrestrial and at least two multi orbit satellite accesses taking into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the traffic with the  aggregated QoS required at the MBS.

[PR 5.21.6-002] The 5G system shall be able to collect and distinguish charging information per radio access path related to traffic between MBSR and 5GC that is simultaneously going over terrestrial radio access path and multiple satellite access paths.

Editor’s note: whether the use of simultaneous connectivity to multi orbit satellite for MBSR is within the scope of the FS_5GSAT_Ph4 is FFS

## 5.22	Use case on aircraft communication services in Joint TN/NTN and multi-orbit deployments

### 5.22.1	Description

Commercial aircraft are being equipped with more and more communication systems to provide a wide range of services, starting from Passenger Internet access to aircraft operations. Connectivity is a key driver for the digital transformation of the aviation industry, leading to fully connected aircraft.

It is expected that the Aviation market can take advantage of standardized solutions based on 5G systems that would operate with Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN) [x1]. Those solutions include 5G cellular networks for operations at the airport, as well as 5G air-to-ground and 5G-NTN through satellite communication systems for In-Flight Connectivity.

The aviation use case presented below is focused on delivering the capability to provide passenger access to internet services.

Internet access is provided on-board the aircraft for passenger entertainment. This service is available “gate-to-gate”, which means that the users can have access to the Internet from the start of their journey when boarding the airplane until their arrival at the gate. The communication system shall then provide consistent performances during all phases of the flight. The passenger will fly from Roma to New-York, meaning will use a transatlantic route in high latitude. During the flight, the service is delivered using a GEO Ku-band (this could be Ka-band as well) satellite then switching to a lower latency LEO constellation when it is required for service continuity, while improving passenger Quality of Experience (QoE).

It is important to note that 4G or 5G Air-to-Ground communication systems are or start to be also in operation today and could have been also reflected in this use case.

An illustration of the use case on aircraft communication services is provided in the following Figure 5.22.1-1.

Figure 5.22.1-1: Illustration of Gate-to-Gate aircraft communication service

### 5.22.2	Pre-conditions

An aircraft supports TN and NTN networks including Air-to-Ground (ATG) and multi-orbit satellite access (e.g., GEO, MEO and LEO constellations) provided by the same or different 5G terrestrial and satellite operators. It is equipped with one or multiple Mobile VSAT that can be associated with a TN and/or an ATG terminal.

The passenger device on-board the aircraft connects to the 5G system either directly through a relay installed in the aircraft or indirectly using a Wireless Local Area Network (or cellular network) associated with one or multiple UE(s) installed in the aircraft.

### 5.22.3	Service Flows

- Marcello, an Italian passenger, is boarding the aircraft in Roma on a flight to New York where he will spend his holidays.

- Marcello has booked an economy class ticket, and he knows that the company is providing a Free of Charge Bring Your Own Device experience to its passengers for this class.

- Marcello is now seating and starts using his personal tablet equipped with a 5G modem & SIM card and connected to the network of his MNO (Mobile Network Operator).

- He is surfing on the Internet to check which movies he will watch during the flight from his video on-demand streaming service provider.

- The doors are closing, and his tablet is automatically switching to the aircraft on-board network connected to the Internet. He can see that his connection has moved from his home MNO network to the airline network after a very short service interruption.

- While the aircraft system is connected to the TN, he is looking at his apps and then starts streaming the last movie promoted by his favorite service provider.

- He is watching the movie while the aircraft is taking off and climbing to cruise altitude with a 5G connection that is moved to a GEO satellite network providing high capacity over Europe.

- After 2 hours, while flying over the North Atlantic Ocean, the movie is over, and he is starting to surf on the internet to have a look at any events happening in New York. He can feel that his surfing experience is as good as when he was at home.

- Marcello does not know it, but during the flight the aircraft has performed a roaming between 2 Satellite Network Operators (SNO). It actually moved from the GEO satellite operator to a LEO constellation operator, which provides better coverage in high latitude and much lower latency. During the flight, the aircraft was seamlessly handing over from one LEO satellite to the next one (as well as switching between beams (i.e. cells)  of a given satellite) along the route to New York.

- The aircraft is now landing at Kennedy airport and the aircraft is now roaming from the NTN satellite network operator to the local TN MNO. Marcello can feel a short outage of the connection.

- When arriving at the gate and the door is opened, Marcello’s tablet is moving back to the local TN MNO, considering roaming agreements.

- When getting off the airplane, Marcello is extremely happy about his internet access experience during the flight. It is very clear to him that he will use the same company next time he flies away from Roma.

### 5.22.4	Post-conditions

While being in a high-speed airborne, the passenger has been able to access Internet services with good and consistent Quality of Experience in a joint TN / NTN deployment with a multi-orbit satellite network.

### 5.22.5	Existing features partly or fully covering the use case functionality

TS 22.261 [2] specifies the following requirements related to satellite access and roaming:

- Section 6.3.2.3

The 5G system shall be able to provide services using satellite access.

- Section 6.3.2.3

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285ms end-to-end latency.

NOTE 1:	 5ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95ms end-to-end latency.

NOTE 2:	 5ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35ms end-to-end latency.

NOTE 3:	 5ms network latency is assumed and added to satellite one-way delay.

A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

The 5G system with satellite access shall support communication service availabilities of at least 99,99%.

- Section 6.18.2

When a service is offered by multiple operators, the 5G system shall be able to maintain service continuity with minimum service interruption when the serving network is changed to a different serving network operated by a different operator.

NOTE 1: A business agreement is required between the network operators.

- Section 7.4/2

The KPIs for airplanes connectivity need to be updated

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/s | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Video surveillance (note 4a) | [0,5] Mbit/s | [3] Mbit/s | TBD | TBD | TBD | N/A | Up to 120km/h or stationary (note 4b) | Vehicle mounted or fixed installation |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 4a: Refer to video surveillance data transmitted (in UL) from a UE on the ground (e.g. picture or video from a camera) using satellite NG-RAN to connect to 5GC, and video surveillance-related configuration or control data sent (in DL) to the UE/device. 0.5 Mbit/s for DL experienced data rate is based on MAVLINK protocol that is widely used for UAV control. 3 Mbit/s for UL experienced data rate is based on the assumed sum from 2.5 Mbit/s for video streaming and 0.5 Mbit/s for data transmission. Note 4b: Up to 120km/h applies to vehicle mounted while stationary applies to fixed installation. Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analysed independently for each scenario. |  |  |  |  |  |  |  |  |

It shall be noted that in TS 22.261 [2], Quality of Experience (QoE) is mentioned but there is no specific KPIs associated. For airplane connectivity, the Seamless Air Alliance (3GPP MRP) has defined a set of KPIs specifically addressing the QoE of commercial aircraft passengers [18].

Additional specific KPIs for this use case that should be taken into account such as:

- Service interruption associated with beam and satellite handovers: < 1 second (implying the need for make-before-break” solutions

- Aircraft attitude with a maximum pitch angle of 20° and a maximum roll angle of 30°

- Service availability over a flight, taking into account all conditions (aircraft attitude, position, handovers and roaming): 98%

### 5.22.6	Potential New Requirements needed to support the use case

#### 5.22.6.1	Potential Service Requirements

[PR 5.22.6-001] Subject to regulatory requirements and operator’s policies, a 5G system with multi-orbit satellite access shall maintain user experience or QoE for a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform.

[PR 5.22.6-002] Subject to regulatory requirements and operator’s policies, a 5G system with multi-orbit satellite access shall support minimum service interruption when the communication path of a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform is moving between terrestrial access and satellite access networks or between satellites networks potentially operating in different orbits (e.g., GEO, MEO or LEO)

[PR 5.22.6-003] Subject to regulatory requirements and operator’s policies, a 5G system with multi-orbit satellite access shall support service continuity with no interruption when the communication path of a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform is moving between satellites of the same network operating in any orbit (e.g., GEO, MEO or LEO).

[PR 5.22.6-004] Subject to regulatory requirements and operator’s policies, a 5G network with multi-orbit satellite access shall be able to provide high throughput performances to the UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform ensuring consistent QoE to the end users.

#### 5.22.6.2 	Potential KPI Requirements

It is proposed to modify the following KPIs defined in the table 7.4.2-1 (performance requirements for satellite access) in TS 22.261 [2].

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airplanes connectivity (note 4) | [1] Gbit/s per plane | [300] Mbit/s per plane | N/A | N/A | N/A | 40% | Up to [1500] km/h | Airplane mounted |
| Note 4: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 3:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple links |  |  |  |  |  |  |  |  |

For the Airplanes connectivity use case, the area traffic capacity and overall user density cannot be expressed accurately because it will depend on a large number of parameters.

Editor’s Note: FFS to bring other KPIs

## 5.23	Use case on connectivity served by multi satellite operators

### 5.23.1	Description

This use case describes a scenario where satellite operators form a partnership to provide roaming services to users. The partnerships include GSO-GSO, NGSO-NGSO and GSO-NGSO interconnections and the PLMNs may differ among operators. For simplicity, the partnership is described as a partnership between two companies, but it is naturally extendable to three or more companies.

UE with a directional steerable antenna needs to acquire the position of a satellite when attempting to establish a connection. This procedure is quite difficult to perform without prior information on the target satellite position. Because position of satellites depends on the business strategy of the operator and changes dynamically, it is desirable to be able to notify the UE of the positions in a timely manner through the system. In addition to the positions, other information (e.g., cell coverage, polarization) may be required by UE in advance for suitable satellite acquisition, and collectively these are referred to as “ephemeris data for acquisition” in this use case.

Satellite operators can provide users with reduced disconnection times and power-saving services by providing the information to acquire and access the suitable satellite served by the partner operator in advance.

### 5.23.2	Pre-conditions

- Satellite operators A and B have established a roaming partnership to enhance coverage, capacity and quality of service.

- Satellite operators A and B share satellite ephemeris data for acquisition and coverage information with each other in a manner agreed between the operators in advance, e.g. through contracts.

- UE is registered to the network served by satellite operator A.

- UE supports portable use or on the move use.

### 5.23.3	Service Flows

- Alice is using satellite communication service served by operator A's satellite.

- Alice's UE uses an existing connection and/or preconfigured/stored data at the UE to obtain the information to acquire and access the suitable satellite among those available.

- The UE decides to reconnect to a new cell provided by operator B's satellite based on the following events:

  - The existing connection is lost and a new cell provided by operator B is available.

  - The UE decides that it is appropriate to move to a different cell provided by operator B i.e., when the reconnection is indicated by the current base station.

- Alice's UE reconnects to a new cell provided by operator B's satellite. She does not suffer long disconnection at this time because the UE knows in advance the ephemeris data for acquisition, allowing for a smooth reconnection.

- Alice is using satellite communication service served by operator B's satellite.

- In case the UE again decides that it is suitable to use operator A's connection, it reconnects to operator A's cell by following the same procedure.

Figure 5.23.3-1: Satellite connectivity served by multi operators

### 5.23.4	Post-conditions

Alice can use the communication services served by satellite operator A and B with minimal disconnection time under a single subscription.

### 5.23.5	Existing features partly or fully covering the use case functionality

Requirements in TS 22.261 cover:

- 3GPP access network selection.

- As a roaming aspect within the satellite access section, there is a UE requirement to support optimized network selection and reselection to PLMNs with satellite access, based on home operator policy.

However, to perform the suitable satellite acquisition, UE needs to know the position of satellites.

### 5.23.6	Potential New Requirements needed to support the use case

[PR 5.23..6-001] Subject to regulatory requirements and operator’s policy, a 5G system with multi orbit satellite access shall be able to provide the UE with the necessary information to acquire and access the suitable satellite among those available.

NOTE 1: The information provided to the UE can include satellites information operated by roaming partners.

NOTE 2: Acquisition of satellites may be required prior to the initial access procedure, including, e.g. the determination of the antenna pointing direction. In this use case, network selection procedures follow the existing procedures.

## 5.24	Use case on Broadcast Services with satellite access for unregistered UEs

### 5.24.1	Description

Live Television remains an essential application handled by broadcast networks today. The use case of live television, whereby hundreds of thousands of viewers watch the exact same content benefits from the scaling effect that is inherent in broadcasting.

Broadcasting however requires dedicated parallel networks to be built which are generally incompatible with the development of mobile networks. Broadcast receivers are also incompatible with other mobile reception devices.

Broadcast networks are expensive to build especially if large coverage areas are required.

For all these reasons a tighter integration with mobile network deployments both Terrestrial Networks and Satellite Networks would be desirable.

Furthermore, most of satellite networks for Public Media Broadcast services are designed today in a SIM-Card Less Mode of Operation (Unregistered UE), this is a key requirement for satellite operator to decide massive deployment of 5G/5G-Advanced for broadcast services over their existing and coming constellations.

This approach can be extended to a satellite overlay addressing not only video content but also any form of digital content that would need to be distributed towards several UEs considering the benefits of the large geographical coverage of satellite networks, with a SIM-Card Less (Unregistered UE) mode of operation.

In terms of the business model, live TV services can be offered as FTA (Free-To-Air) advertisement financed services, as subscription services or on a flexible pay per view basis, whereby viewers are charged for individual events. Other business models, for example a revenue sharing agreement between the mobile operator and broadcaster, can also be envisaged. Access to live TV could also be offered as a differentiating factor.

Figure 5.24.1-1: Illustration of a Satellite Network complementing a Terrestrial Network Broadcast environment in a rural/suburban area with fixed UEs/UEs moving through the coverage area

### 5.24.2	Pre-conditions

Consider a use case where a mobile network operator (MNO) provides broadcast services over a radio coverage area addressing urban and sub-urban areas. An MNO has included in its service package the distribution of television channels or video broadcast services. The demand for these distribution services is increasing steadily with the number of programs available, as well as with the quality of the content. This leads in some cases to the saturation of the transmission capabilities of MNO, yet UEs being subscribed to the service shall receive the corresponding content. At the same time, there are some areas which are with poor radio coverage even not under the coverage of the MNO, leading the UEs to experience bad QoS even preventing them to get access to the television channels or video broadcast services.

A satellite network operator SNO provides services over a radio coverage which is including the suburban area and the San Bernadino Mountain.

Let consider 3 users:

- User 1 is sitting at home in the MNO coverage area, watching TV and receiving the signal from TN coverage. User 1 has a device (fixed VSAT) with only a broadcast receiver component.

- User 2 is in a car exiting the Palm Springs area and going to the direction of San Bernadino Riverside. In this car, the user's kids are watching a cartoon movie broadcasted on a TV channel. User 2 has a device (antenna panel) with a broadcast receiver component and an independent component for unicast services.

- User 3 is sitting in a holiday home located in Queen Valley while watching an entertainment program on TV. User 3 has a device (fixed VSAT) with only a broadcast receiver component.

All the users are not registered in the 5G System for the reception of the broadcast TV service. User 2 requires registering to the 5G System through the unicast component when it wants to get access to unicast broadband services.

The broadcast receiver component is able to receive only broadcast TV either from the TN MNO or from the SNO.

The unicast component follows independently the regular registration procedure to the 5G System through MNO or SNO to access to unicast broadband services.

### 5.24.3	Service Flows

Using a joint Terrestrial Access Network – Satellite Access Network antenna with a chipset on the rooftop, User 1 will get the TV service from the Terrestrial Network MNO, and in case of traffic overload in the terrestrial network infrastructure, the MNO could use the resource of the SNO to redirect the TV services targeting the Palm Springs area.

Using a Joint Terrestrial Access Network - Satellite Access Network antenna with a chipset on the rooftop of his car, User 2 will be able to get continuously connectivity while exiting the suburban area and entering the mountain area. The TV channel traffic broadcast is in this case sent on all the infrastructure (satellite and terrestrial) in these areas. At a given point-in time the Joint Terrestrial Access Network – Satellite Access Network antenna will switch the reception of the data flows coming from the MNO to the one coming from the SNO after evaluation of better link quality on the SNO. User 2 is expected to experience some service interruption in the broadcast service due to the switching of the broadcast reception from MNO to SNO. At the same time, User 2 will benefit from a unicast broadband connection independently which will select the TN MNO which will select the appropriate access network (terrestrial or satellite) based on multiple criteria (network load balancing, latency requirements for QoS …).

Using a Satellite Antenna with gateway, User 3 will be able to watch the TV Channel broadcasted through the SNO in the Queen Valley area.

### 5.24.4	Post-conditions

MNO has maintained the QoS of the content delivery for UEs being subscribed to the content delivery service, while MNO has been able to cope with the increasing traffic on its infrastructure.

MNO has offered an improved QoS of the content delivery for some UEs which are in area not under the coverage of the terrestrial network infrastructure.

The Users don’t need to have been registered to the 5G System before receiving the broadcast service, simplifying the procedure especially when switching between Terrestrial Network and Satellite Network.

The SNO doesn’t need to take care of the provisioning of the Users when extending Broadcast Services, especially when it targets Public Media Broadcast Services (which is about ensuring equitable access to broadcast services, making them available to everyone without barriers).

### 5.24.5	Existing features partly or fully covering the use case functionality

Some capabilities related to 4G and TV services have been addressed with Free-to-air (FTA) in TS.22.201 (4G context) but not in the context of 5G system and not taking into account satellite access as such as part of the 5G system.

### 5.24.6	Potential New Requirements needed to support the use case

[PR 5.24..6-001] Subject to regulatory requirements and operator’s policy, a 5G System supporting satellite access and Broadcast Services shall be able to deliver Media Broadcast to a UE which is not registered.

NOTE: 	Subject to an agreement (SLA) between the MNO and the SNO the above requirement can be applied

Editor’s Note:	 the above requirement and the security impacts are FFS

# 6	Consolidated requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to a 5G system with satellite access.

The consolidation is organised by topics as following:

- IMS voice calls using GEO satellite access

- Resilient notification

- emergency communications and mission critical services using satellite access

- multi-orbit

- charging aspects

- others (if needed)

Editor's note:	to update topics is FFS

## 6.2	IMS voice calls using GEO satellite access

### 6.2.1	Consolidated functional requirements

The potential requirements corresponding to the support of IMS voice calls using GEO satellite access are listed in the table below.

Table 6.2.1-1 – Consolidated Requirements for IMS voice calls using GEO satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2.1-1 | The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [3]. | [PR 5.1.6-001] |  |
| CPR 6.2.1-2 | The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size. | [PR 5.1.6-002] [PR 5.1.6-003] |  |

### 6.2.2	Consolidated potential KPIs

Table 6.2.2-1	Consolidated Performance Requirements for IMS voice call using GEO satellite access

| Scenario | UE type | Transmission data rate |  | Call setup time NOTE 1 |
| --- | --- | --- | --- | --- |
|  |  | UL | DL |  |
| IMS voice call using GEO | Handheld | [1-3] kbit/s | [1-3] kbit/s | ≤30 s NOTE 2 |
| NOTE 1: call set up time refers to [4]; NOTE 2: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [12]; |  |  |  |  |

## 6.3	Resilient Notification

The potential requirements corresponding to the support of resilient notification are listed in the table below.

Table 6.3-1 – Consolidated Requirements for resilient notification

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3.1-1 | Subject to operator’s policies, a 5G System with satellite access shall be able to support Resilient Notification Service to notify users, with valid subscription to the notification service, about a missed mobile terminated service when the user is unreachable via satellite access. NOTE: The Resilient Notification service can provide the user with service information e.g. caller’s information and service type. | [PR 5.2.6-001] |  |

## 6.4	Emergency communications and mission critical services using satellite access

## 6.4.1	PWS with satellite access

The potential requirements corresponding to the support of PWS with satellite access are listed in the table below.

Table 6.4-1 – Consolidated Requirements for PWS with satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.4.2	Other emergency communications and mission critical services using satellite access

The potential requirements corresponding to the support of emergency communications and mission critical services using satellite access are listed in the table below.

Table 6.4-2 – Consolidated Requirements for emergency communications and mission critical services using satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.5	Multi-orbit

## 6.5.1	General aspects

The potential requirements corresponding to the support of multi-orbit general aspects are listed in the table below.

Table 6.5.1-1 – Consolidated Requirements for multi-orbit general aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.5.2	Service continuity aspects

The potential requirements corresponding to the support of service continuity aspects related to multi-orbit are listed in the table below.

Table 6.5.1-2 – Consolidated Requirements for service continuity related to multi-orbit

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.6	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.6-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.7	Other aspects for satellite access

The potential requirements corresponding to the support of enhancements of other aspects of satellite access are listed in the table below.

Table 6.7-1 – Consolidated Requirements for other aspects of satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

# 7	Conclusions and recommendations

This technical report identifies gaps, including potential requirements and KPIs, for enhancing existing use cases and defining new requirements for emerging use cases related to 5G systems with satellite access.

The resulting service requirements have been already consolidated in clause 6 for 6.2 and 6.3.

Editor's note:	Additional consolidation for the remaining topics is FFS

It is recommended to consider the consolidated requirements identified in this TR as the baseline for the subsequent normative work.

Annex A (informative):
Mapping of use cases and topics covered

The following table is proposing a mapping between the use cases described in the clause 5 and the main topic addressed as well as if it brings KPIs

Table A-1 – Mapping between use cases and topics covered

|  | Use Case | IMS voice call on GEO | Resilient Notification | Emergency communications /Mission Critical | Multi-Orbit | Broadcast services with satellite access for unregistered UEs |
| --- | --- | --- | --- | --- | --- | --- |
| 5.1 | Use case of IMS Voice Call Using GEO Access | X (note 1) |  |  |  |  |
| 5.2 | Use case on Resilient Notification |  | X |  |  |  |
| 5.3 | Use case on Resilient Satellite Communication for Public Safety |  |  | X | X |  |
| 5.4 | Use case on service continuity through multi-orbit satellite access |  |  |  | X |  |
| 5.5 | Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks |  |  |  | X |  |
| 5.6 | Use case on switching between multi-orbits satellite networks in remote area applications |  |  |  | X |  |
| 5.7 | Use case on assisting vehicular communications via multi-orbits satellite access |  |  |  | X |  |
| 5.8 | Use case on traffic over different orbit satellites |  |  |  | X |  |
| 5.9 | Use case on Public Warning Service |  |  | X |  |  |
| 5.10 | Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay |  |  | X (note 1) |  |  |
| 5.11 | Use case on emergency communication using satellite access |  |  | X |  |  |
| 5.12 | Use case on supporting different services with multi-orbit satellites |  |  |  | X |  |
| 5.13 | Use case on wide-area disaster control support using multi-orbit satellite networks |  |  |  | X |  |
| 5.14 | Use case on broadband services through multi-orbit satellite access |  |  |  | X |  |
| 5.15 | Use case on multi-orbit access for cross-hybrid service |  |  |  | X |  |
| 5.16 | Use case on using GEO for initial network entry and service initialization |  |  |  | X |  |
| 5.17 | Use case on resource and energy efficiency in a multi-orbit Satellite Access System |  |  |  | X |  |
| 5.18 | Use case on using GEO to land data in a desirable geographical location |  |  |  | X |  |
| 5.19 | Use case on using GEO for sleep/power management |  |  |  | X |  |
| 5.20 | Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR) |  |  |  | X |  |
| 5.21 | Use case on combined terrestrial and multi-orbit satellite access for connecting MBSR and 5GC |  |  |  | X |  |
| 5.22 | Use case on aircraft communication services in Joint TN/NTN and multi-orbit deployments |  |  |  | X (note 1) |  |
| 5.23 | Use case on Connectivity served by multi satellite operators |  |  |  | X |  |
| 5.24 | Use Case on Broadcast services with satellite access for unregistered UEs |  |  |  |  | X |
| NOTE 1: the use case provides KPIs |  |  |  |  |  |  |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-05 | SA1#106 | S1-241280 |  |  |  | TR skeleton for TR22887 - FS_5GSAT_Ph4 | 0.0.0 |
| 2024-05 | SA1#106 | S1-241422 |  |  |  | Inclusion of agreed pCRs: S1-241371, S1-241398, S1-241399, S1-241411, S1-241413, S1-241414, S1-241415, S1-241418 | 0.1.0 |
| 2024-08 | SA1#107 | S1-242461 |  |  |  | Inclusion of agreed pCRs: S1-242376, S1-242395, S1-242506, S1-242211, S1-242073, S1-242054, S1-242394, S1-242379, S1-242507, S1-242356, S1-242381, S1-242508, S1-242396, S1-242383, S1-242385, S1-242509, S1-242510, S1-242511, S1-242391 | 0.2.0 |
| 2024-11 | SA1#108 | S1-244695 |  |  |  | Inclusion of agreed pCRs: S1-244794, S1-244886, S1-244655, S1-244795, S1-244653, S1-244127, S1-244796, S1-244150, S1-244783, S1-244335, S1-244336, S1-244654, S1-244803, S1-244888, S1-244798, S1-244573, S1-244799, S1-244800, S1-244801, S1-244802, S1-244656, S1-244889, S1-244658 | 0.3.0 |