3GPP TSG-SA1 Meeting #109	S1-250063

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.840 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a new use case for physiological monitoring and indoor positioning in smart firefighting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hytera Communications Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AmbientIoT |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Propose introducing a new use case of Ambient IoT in smart firefighting in TR 22.840, enhancing real-time physiological monitoring and indoor positioning through end-to-end 3GPP technology. This would improve situational awareness for incident commanders, enabling informed decisions that enhance first responders' health, safety, and operational efficiency during emergencies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces a new use case in the newly added clause 5.31, to address the above aspect. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this use case, the current TR 22.840 will not be considered for addressing the needs of first responders, which is crucial for ensuring their health and safety in firefighting. The current requirements in TS 22.369 may not fully support related new capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.31 Use case on Ambient IoT device for physiological monitoring and indoor positioning in smart firefighting

5.31.1	Description

Firefighting is exceptionally strenuous and dangerous work that can have significant health consequences for first responders, including death. In public safety, IoT—known as the Internet of Life-Saving Things—plays a vital role in smart firefighting by integrating cyber-physical systems (CPS) and leveraging data. Firefighters currently rely on traditional battery-powered wearable IoT sensors and devices embedded in protective clothing (PPC) to monitor vital signs, environmental conditions, and movement, ensuring health and safety during emergencies.

With improvements in smart sensors and materials, intelligent textile sensing systems are capable of gathering and storing energy from various sources (e.g., vibration, temperature variations, thermal, and sunlight) without the need for recharging. Ambient IoT has emerged as a promising technology within this system, primarily for short-range communication.

These built-in wearable sensors collect small-sized physiological, motion, and environmental data. This data is transmitted to firefighters' mobile phones, then stored and/or relayed to a remote application server via the 5G network. Incident commanders and medics can access summary information through an app displaying physiological strain, alarm notifications, and other situational awareness indicators. This enables informed decisions on whether to remove a firefighter from an operation for their safety.

Besides 5G macro network coverage, deployable base stations and device-to-device (D2D) communication can complement coverage and enhance data link reliability, particularly in basements where RF signals are blocked.

Ambient IoT enables battery-less operation, eliminating risks like battery depletion and thermal runaway in extreme conditions. By leveraging MCX UE to create a personal access network (PAN), it removes the need for separate hubs or gateways, improving communication between Ambient IoT and MCX applications while reducing firefighter burden. Unlike Bluetooth, Ambient IoT enhances availability, interoperability, and E2E security through 3GPP technology, while NB-IoT/eMTC is more suited for long-range communication. Additionally, Ambient IoT provides indoor positioning and navigation, supporting search and rescue efforts. These features make the system compact, reducing burden while ensuring reliability. Beyond firefighting, the solution is also beneficial for EMS and patient monitoring.

Figure 5.30.1-1: physiological monitoring and indoor positioning for fire fighters

5.30.2 Pre-condition

- The rugged device carried by first responders are 3GPP UE, Ambient IoT-capable UE, and MCX UE.

- An array of physiological and environmental sensors are integrated inside personal protective clothing (PPC) worn by firefighters.

- The fireground is covered by 5G networks, including macro networks, indoor building coverage, mobile eBN, and ProSe U2U and/or U2N relay.

- Operators have deployed an indoor positioning system for call flow-b scenarios.

Call flow a- monitoring firefighters’ health and safety

- A structural fire occurs, and Engine 1, Engine 2, and Chief 1 are dispatched and arrive at the scene. The Incident Commander (IC) establishes a command post, assesses the situation, and deploys a mobile gNB outside the building to complement the PLMN’s coverage.

- Firefighters A, B, and C enter the building for search and rescue, equipped with rugged mobile phones and wearable AIoT sensors. Pre-installed vital sign monitoring and MCX applications are registered on servers and authorized to communicate with IoT devices. AIoT devices harvest energy from body temperature variations and radio waves, while mobile phones act as readers and gateways, transmitting data via 5G networks.

- Biometric and environmental data are collected and analyzed to monitor firefighter health and safety. A biometric summary is displayed on the HUD in the face mask, while data is sent to the Mobile Data Terminal (MDT) used by on-scene commanders for real-time monitoring. Alarms trigger for critical conditions, such as low blood glucose.

- Sensors detect falls, collapses, and motionlessness. As the fire intensifies, the IC and nearby firefighters receive an emergency alert and/or automatically triggered Emergency Group Communication from Firefighter A via MCX servicers over 5G on/off-network communications. Firefighters B and C locate and rescue A. (Indoor positioning and navigation may be used or may not be used).

Call flow-b Indoor positioning and navigating

- In complex structural fires, such as those in shopping centers, in addition to health and safety monitoring, the indoor positioning and navigating are needed. Pre-installed Ambient IoT devices in the shopping centers, powered by ambient energy sources (e.g., light, heat, radio waves), provide real-time indoor positioning to help firefighters navigate and find routes and exits.

- En route, firefighters receive instruction from the control room about the available indoor positioning system, download a floorplan-based navigation app and complete the AIoT service registration and authorization through 5G network.

- Upon entry to a building, their mobile phones trigger RF signals to nearby ambient IoT services. Nearby devices activate, responding to mobile phone with device IT. The mobile device calculates the relative distance and angle to Ambient IoT devices, determining the firefighter’s “best-possible” accurate position on the indoor navigation app, also displayed on a head-up display (HUD) without the need to look down at the hand-held mobile phone.

- With the help of the navigation app, firefighters use Z-axis positioning from the emergency caller and indoor positioning to locate, search for, and rescue an occupant on the third floor, then evacuate through the nearest exit.

Note: The Z-axis solution is out of the scope of the document.

5.31.4 Post-conditions

Thanks to 5G and Ambient IoT systems, along with monitoring, navigation and MCX services, firefighters’ health and safety are monitored in real-time, and the efficiency and effectiveness of rescue operations are significantly improved.

5.31.5   Existing features partly or fully covering the use case functionality

None

5.31.6   Potential New Requirements needed to support the use case

[PR 5.31.6-1] The 5G system shall be able to support 5G network or an Ambient IoT capable UE to communicate with a specific Ambient IoT device or a group of Ambient IoT devices simultaneously.

[PR 5.31.6-2] The 5G network shall support a mechanism to authorize an Ambient IoT capable UE to communicate with an Ambient IoT device or a group of Ambient IoT devices.

[PR 5.31.6-3] The 5G system shall be able to support mechanisms to communicate:

-between an Ambient IoT device and the 5G network using Ambient IoT indirect network communication via on-network and off-network connectivity, or

-between an Ambient IoT device and Ambient IoT capable UE using Ambient IoT device to UE communication.

[PR 5.31.6-4] The 5G system shall support positioning services for Ambient IoT devices (e.g., to locate Ambient IoT devices using absolute or relative positioning methods)

[PR 5.31.6-5] Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to obtain data from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party via the 5G network.

[PR 5.31.6-6] Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.

[PR 5.31.6-7] The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE, without compromising communication availability.

[PR 5.30.6-6] The 5G system shall support a mechanism that enables Ambient IoT UEs and MCX UEs to coexist within the same 3GPP UE, allowing IoT critical data to trigger MCX emergency alerts and MCX emergency group communication.

[PR 5.30.6-7] The 5G system shall provide the network connection to address the following KPIs for the use of Ambient IoT devices on smart firefighting.

Table 5.31.6-1: Potential key performance requirements for the use of Ambient IoT devices for smart fire fighting

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| physiological monitoring | 0.5S | 99.9% | 99.9% | <1 kbit/s | <1 kbits (Note 1 ) | <5 per 100 m2 | 10 m | Up to the local and/or PLMN network | Static or walking speed <6km/h | Ideal in real -time | 0.5S | 90% | 3 m (Note2, Note3) |
| Indoor location | 0.5 S | 99.9% | 99.5% | <1 kbit/s | 96 bits （Note 1） | 2500/1000 m2 | 10 m | Up to the whole PLMN | NA | NA | 0.5 S | 90% | 3 m (Note2, Note3) |
| NOTE 1: The payload includes Ambient IoT device information, e.g., Ambient IoT device ID [5]. NOTE 2: The positioning accuracy can be applied to horizontal NOTE 3: This value depends partly on the actual communication range. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * *