3GPP TSG-SA WG1 Meeting #109	S1-250093

Athens, Greece, 17-21 February 2025	                                                                 Revision of S1-244863

Title:	Computing service in Operator managed data network

Agenda Item:	8.1.8

Source:	NEC

Contact:                       Hiroshi Dempo, hiroshidempo@nec.com

Deepak Rohilla, deepak.rohilla@india.nec.com

Abstract: <provide a short description of the content>

It is expected that ubiquitous use of computing resources would expand throughout the IMT-2030. Roadmap to 6G [y] also has introduced a goal where communications/connectivity services in 6G work will work together with unified computing that scales across devices, network computing resources, and data centers. For simplicity, this use case discusses a ubiquitous computing service in the context of an autonomous delivery drone fleet in order to induce general requirements for ubiquitous computing.

* * * Start of 1st change * * * *

# W	Other Use Cases

Editor's Note: If there are any use cases that does not fall into any sections in this TR, these use cases could be placed in this clause, subject to further discussion. How this clause will be used is FFS.

## 10.x	Use case on Computing service in Operator managed data network

### 10.x.1	Description

According to ITU-R M.2160 [27], it is expected that ubiquitous use of computing resources would expand throughout the IMT-2030. In this context, the expansion of ubiquitous computing in the network infrastructure is closer to the origin of the data and support for proliferation of ubiquitous intelligence throughout the IMT-2030. The reference also described that the trend contributes to the improvements for applications requiring real-time responses and data transport. Ubiquitous computing disseminated across IMT-2030 is expected to enable efficient utilization of resources and optimal placement of workloads, as well as scales and manages the infrastructure to run the applications. Roadmap to 6G ［x］ also has introduced a goal where communications/connectivity services in 6G work will work together with unified computing that scales across devices, network computing resources, and data centers.

Ubiquitous computing can be utilized by variety of application workloads such as self-driving in automobile, augmented reality (AR)/virtual reality (VR)/immersive application, industrial and medical robotics, drones and vision/control systems. For simplicity, this use case discusses a ubiquitous computing service in the context of an autonomous delivery drone fleet in order to induce general requirements for the ubiquitous computing. An autonomous delivery drone fleet consists of unmanned aerial vehicles designed to transport goods quickly and efficiently without human intervention. Leveraging ubiquitous computing in 6G, these drones navigate safely while optimizing routes through a fleet management system.

Drones are responsible for delivering packages in an urban area. Each drone is equipped with a variety of sensors that monitor its systems, including battery health, motor performance, GPS functionality, and environmental conditions. These drones can dynamically select optimized routes by utilizing ubiquitous computing in the 6G system.

Drones can use the compute capability available at the edge of the network to determine flight path, etc. Drones can also use assistance with real time information and use the compute capability available on the flight path in network elements such as remote gNB, repeaters, etc. This remote network elements, termed as “Far-edge”, provides compute capability for collision detection and avoidance, real time path change, etc.  The far-edge capability enables computation even closer to the drones, provides extremely low-latency computations such as data processing and real-time analytics. This ubiquitous computing service for verticals enables workloads to navigate the drones in complex urban landscapes more effectively, reliable and responsive.

Operator managed data network, controlled and operated by a telecommunications service provider, includes components like edge compute domains. These can provide localized computational resources that enhance devices with limited processing power. Offloading tasks by UE to Operator managed data network can achieve better performance and efficiency, benefiting reduced latency and faster data processing.

### 10.x.2	Precondition

John, Manager in a company got a delivery order in his area and he assigned job to an autonomous delivery drone.

### 10.x.3	Service Flows

- Before the beginning of the business hour, an edge computation supports a computational workload for the fleet management system. The computational workload on the edge computation generates a set of initial delivery plan. Far edge computations support computational workloads for localized fleet management. The computational workloads on the edge and far edge computation may be deployed or instantiated by the telecom operator or verticals in air control markets.

- The computational workload on the edge computation delivers the initial plan to drones and far edge computations for the localized fleet management.

- The computational workloads on far edge computations further generates initial detail route plans for drones in the assigned zone that would come to the zone. The assigned drone collects to parcel and left the warehouse to delivery location.

- The drone continuously collects data from its sensors during flight. For instance, it monitors battery voltage, temperature, and motor RPM and sending a report to far edge computation in the 6G network.

- The computational workload on the far edge computation collects reports from drones flying in the assigned zone. The reports may also be converted into the acceptable format if different verticals use individual report format. Then the computational workload on far edge computation analyses the converted reports to infer instructions (e.g. optimized route, altitude from an entry to exit point) in the assigned zone. After the computation, the computational workload on far edge computation responds each drone with an optimized route for the drone in the step 3.

- To enhance their operational efficiency, the step 3 and step 4 may be repeated at every specified time. Therefore, the optimized route is frequently updated to adapt conditional and environmental changes.

- The drone changes from the current far edge computation to another far edge computation which manages another zone when it comes to a border of the zone.

- The drone repeats the step3, step 4 and step 5.

- At each step, information for the far edge computation can be exposed to verticals so that the vertical can utilise the information for their business improvements.

### 10.x.4	Post-condition

John is notified the successful delivery of the order.

### 10.x.5	Existing features partly or fully covering the use case functionality

- Not applicable

### 10.x.6	Potential New Requirements needed to support the use case

- [PR W.x.6-1] Subject to operator’s policy and user concent, 6G system shall support computation offload from a UE to operator managed data network (e.g. core network or edge compute domain).

- [PR W.x.6-2] Subject to operator’s policy and user concent, 6G system shall support discovery of computation resource availability in operator managed data network (e.g. core network or edge compute domain), which can support required QoS.

- [PR W.x.6-3] Subject to operator policy and user consent, 6G system shall support mechanism to manage offloaded computation task in operator managed data network (e.g. core network or edge compute domain) by UE.

Editor’s Note: Operator managed data network is FFS

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.70066: " 	Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[x]	 Next G Alliance Report: Roadmap to 6G, Feb. 2022.

* * * End of 2nd change * * * *