3GPP TSG SA WG 1 Meeting #109 	S1-250105

Athens, Greece, 17-21 February 2025

Source:	Orange

pCR Title:	Pseudo-CR on Completing the potential requirements for Use case on Green social media & email content download

Draft Spec:	3GPP TR 22.883

Agenda item:	7.2

Document for:	Agreement

Contact:	Philippe Lottin

Abstract: Adding 2 PRs to clause 5.8.6 and removing “regularly” from clause 5.8.3.

1. Introduction

For the “Use case on Green social media & email content download”, only one PR is defined, which covers one possible way to realize the service flows, whereby the application layer takes care of scheduling non time-critical data transfer.

2. Reason for Change

The service flows in clause 5.8.3 can be realized by having the network taking care of this scheduling non time-critical data transfer. This avoids operators to expose to third parties information that may be considered sensitive.

3. Proposal

It is proposed to add the following 2 PRs to cover the above realization.

[PR.5.8.6-x] The 5G System shall be able to receive information from applications indicating that a part of the traffic requires (delayed) green delivery instead of immediate delivery.

[PR.5.8.6-y] The 5G System shall be able to delay the establishment of traffic flows to a time when renewable energy ratio at the serving site is sufficiently high and/or the energy consumption parameters relating to radio signal conditions are favourable, as required by the application.

In addition, it is proposed to remove the work “regularly” in the 3rd bullet of the service flows, because the network can use predictions to perform the evaluation only once.

* * * First Change * * * *

### 5.8.3	Service flows

- A message with attachment(s) is sent to the End-User through social media or email.

- The message body is instantly delivered by the Mobile Network to the End-User’s UE, but its attachments are scheduled for (delayed) green delivery instead.

- The Mobile Network evaluates the renewable energy ratio at the serving cell site, as well as the energy consumption parameters relating to radio signal conditions experienced at the UE.

- When the renewable energy ratio at the serving site is sufficiently high and/or the energy consumption parameters relating to radio signal conditions are favourable, the attachments are also delivered to the UE.

- The MNO provides incentives to the End-User that has (delayed) green download enabled. These incentives may be based on the EC and/or CO2e gains obtained through (delayed) green delivery.

* * * Next Change * * * *

### 5.8.6	Potential new requirements needed to support the use case

[PR.5.8.6-1] The 5G system shall be able to expose information to an Application Function in the network on the (expected) energy related characteristics applicable to the serving site/network.

NOTE:	This information can be used by applications in the Application Function to schedule non time-critical data transfer to or from the UE.

[PR.5.8.6-x] The 5G System shall be able to receive information from applications indicating that a part of the traffic requires (delayed) green delivery instead of immediate delivery.

[PR.5.8.6-y] The 5G System shall be able to delay the establishment of traffic flows to a time when renewable energy ratio at the serving site is sufficiently high and/or the energy consumption parameters relating to radio signal conditions are favourable, as required by the application.