TSG-SA WG 1 (Services) meeting	S1-021461

Rome, Italy, 8-12th July 2002

Title:			On MBMS Service Discovery

Source:		Nokia

Agenda Item:	MBMS

Document for: 	Discussion & Decision

# Introduction

Currently 22.146 specifies that a user/subscriber can become aware about services, ongoing or future, that are available at his/hers current location.

It is proposed that we expand the current MBMS service discovery so that services available at neighbouring multicast/broadcast areas can also be advertised in the subscriber's current location.

# PROPOSED CHANGE

Current text:

## 4.3 	MBMS service discovery

The user should be informed that there are MBMS services available in the network. The network shall support service announcements both for the broadcast and multicast mode of MBMS in order to enable the user to discover that there are MBMS services available currently, or some time later, in the user's current location.

Proposed text:

## 4.3 	MBMS service discovery

The user should be informed that there are MBMS services available in the network. The network shall support service announcements both for the broadcast and multicast mode of MBMS in order to enable the user to discover that there are MBMS services available currently, or some time later, in the multicast/broadcast area the user is currently located. In addition, the user may discover, or become aware that there are multicast/broadcast services currently active, or multicast services that will become active at some time later in the multicast/broadcast area(s) that are neighboring to the multicast/broadcast area the user is currently located.

# rECOMMENDATION

It is recommended that the proposed text should be discussed, modified if necessary, and if agreed then Nokia will provide a corresponding CR in SA1#17.