3GPP TSG-SA WG1 Meeting #47	 S1-093283

# Rome, Italy, 3-7 August 2009

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | 0318 | rev | 1 | Current version: | 9.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access networks consideration for emergency call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RIM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10, IMS_EMER_GPRS_EPS |  |  |  |  |  |  | Date: |  | 06/08/2009 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | Certain access network doesn’t support emergency call so the emergenyc call attempted by a UE will be fail over the access network. In order to facilitate emergency call, it is better to try another emergency call over access network which is capable of emergency call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The Emergency service is required only if the UE supports voice and only if access network supports emergency call. It shall be possible to minimize delay to Emergency call when initial emergency call attempt and, if it fails, another attempt subject to operator policy and reguratory requirement. - Access network consideration for emergency call . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether to try another emergency call over access networks though the access networks maynot support emergency call Emergency call delay and high battery consumption |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1, 10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 10	Emergency Calls

## 10.1 	General requirements

It shall be possible to establish an emergency speech call. Emergency calls will be routed to the emergency services in accordance with national regulations for where the subscriber is located. This may be based upon one or more default emergency call numbers stored in the ME. It shall be allowed to establish an emergency call without the need to dial a dedicated number to avoid the mis-connection in roaming case, such as menu, by use of a 'red button', or a linkage to a car air bag control. Emergency calls shall be supported by the UE without a SIM/USIM/ISIM being present. No other type than emergency calls shall be accepted without a SIM/USIM/ISIM.

The Emergency service is required only if the UE supports voice and only if access network supports emergency call. It shall be possible to minimize delay to Emergency call when initial emergency call attempt and, if it fails, another attempt subject to operator policy and reguratory requirement.

NOTE 1: 	It will be left to the national authorities to decide whether the network accepts emergency calls without the SIM/USIM/ISIM.

It shall be possible to initiate emergency calls to different emergency call centres, depending on the type of emergency. The following types of emergency calls shall be possible:

- Police

- Ambulance

- Fire Brigade

- Marine Guard

- Mountain Rescue

- Manually Initiated eCall (MIeC)

- Automatically Initiated eCall (AIeC)

- Spare

When a SIM/USIM is present, subscriber specific emergency call set-up MMI shall be provided.  The Home Environment operator shall specify preferred emergency call numbers (e.g. 999 for UK citizens or 110, 118 and 119 for Japanese citizens).  These emergency call numbers shall be stored in the SIM/USIM and the ME shall read this and use any entry of these digits to set up an emergency call. It shall be possible to store more than one instance of this field.

NOTE 2: 	Release '98 and earlier SIM cards have the capability to store additional emergency call numbers. However in many cases this has not been used.

It shall be possible to tie any emergency call number to any single emergency call type or to any combination of emergency call types.  The association between emergency call numbers and emergency call type shall be able to be programmed by the Home Environment operator into the SIM/USIM.

Example:

19		Police (Albania)

100		Police and Fire Brigade (Greek cities)

100		Ambulance and Fire Brigade (Belgium)

112		Police and Ambulance (Italy)

112		General emergency call, all categories (Sweden)

115		Fire Brigade (Italy)

144		Ambulance (Austria)

NOTE 3:	If the UE does not recognise the emergency call numbers but the serving network recognises the dialled number as an emergency call number used in the country, a normal call set up takes place over the radio interface and after the serving network has recognised the emergency number the call is routed as an emergency call.

The user friendly MMI that specifies the type of emergency call directly (e.g. menu) should be supported for use in any (i.e. home or visited) PLMN to avoid the mis-connection in roaming case.  This shall be allowed both with and without SIM/USIM being present.

When emergency call establishment is initiated, the emergency call type shall be sent by the UE if it is available.

The serving network may download additional emergency call numbers to the UE in order to ensure that local emergency call numbers are known to the UE.  The UE shall regard these emergency numbers as valid in that country only (as identified by the MCC) and shall discard them when a new country is entered.

If permitted by local regulation, it shall be possible for the user to prevent the sending of his public user identifiers and the location information to the PSAP (i.e  emergency response centre).

NOTE:	Operator policies (e.g. requirements for support of emergency communications) may over-ride the user request for suppression.

## 10.x Access Network Consideration for emergency call

If a UE knows that some access networks are capable of emergency call, the UE should attempt an emergency call on the access networks that are emergency call capable. A UE attempting an emergency call should give priority to the voice capable access networks it is connected to. In case the call attempt fails, the UE should automatically make a second attempt on other voice capable access networks.