3GPP TSG-SA WG1 Meeting #59 	 S1-122220

# Chicago, USA, 30 July - 3 August 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.228 | CR | 0172 | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirement for IMS registration control based on location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RegCon |  |  |  |  |  |  | Date: |  | 2012-07-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | In legacy PSTN fixed phone services, it is implied that the fixed phone should only be used in specific location, restricted by physical line connection. When commercially launching IMS services, there are similar requirements to control user registrations based on UE location, e.g. restricting user registration within certain locations. The requirement comes from the customers’ concerns to support roaming or nomadic capability without any limitations. Some examples of their concerns: - Some enterprise customers hope that they can restrict their employees to use fixed SIP phones only in office, like using legacy POTS termials. Allowing the SIP phones to register from outside of the office may cause unexpected cost. - Operators may provide broadband service combined to IMS service with low price to residential users, but does not allow the users to use IMS service from any other places. Therefore it is proposed to introduce the capability of controling user registration based on user’s location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The capability of controling user registration based on user’s location information is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User’s requirement to control IMS registration based on user’s location can not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.2	Access to the IM CN subsystem

7.2.0 	General

IMS, complying with the principle of access independence, supports IP multimedia applications via IP multimedia sessions over a multitude of IP Connectivity Access Networks. These include e.g. E-UTRAN, UTRAN, GERAN, fixed line, I-WLAN, DOCSIS®, WiMAX™ and cdma2000® access.

7.2.1	Access control

The IM CN subsystem shall be able to verify at any time that the user is entitled to use the resources of the IM CN subsystem.

7.2.2	IMS Registration and De-registration

In order to be able to access services from the IM CN Subsystem a UE shall register on the IM CN Subsystem.

-	A UE that supports IMS shall be able to register on the IM CN Subsystem.

-	A UE may support automated IMS registration, e.g. when gaining access to the PS domain.

-	For fixed line, it shall be possible for IMS CN subsystem to control UE registration based on UE location, i.e. to restrict UE registration in specific locations.

Note:	The registration control is based on preconfigured location information per subscription.

-	It shall be possible for the network operator to request a UE to register on the IM CN Subsystem, provided that access to the PS domain is already available.

Note: 	the mechanisms by which the network operator requests the UE to perform an IMS registration may be outside the scope of the IMS.

-	A UE that supports IMS shall be able to de-register from the IM CN Subsystem.

-	The network operator shall be able to de-register a UE from the IM CN Subsystem.