3GPP TSG-SA WG1 Meeting #64	S1-135222

San Francisco, USA, 11-15 November 2013

Title:		eICBD use case: UE requests its MNO to help create IP connection to another UE

Agenda Item:	FS_eICBD

Source:		Intel

Document for:	Approval

Contact:	Kerstin Johnsson [kerstin.johnsson@intel.com]

# Introduction

In this use case, a UE with only a private IP address requests help from its MNO in establishing an IP connection to another UE with only a private IP address in a different MNO.

# Proposal

It is proposed to add the following to TR 22.8xy:

x.1 	UE requests its MNO to help create IP connection to another UE

x.1.1	Description

Bill wants to compete in a P2P video game on his UE against his friend, Dave.  However, since neither Bill nor Dave has a public IP address, Bill’s UE requests his MNO to help it create an IP connection with Dave’s UE.

x.1.2	Pre-conditions

- Bill and Dave are subscribers of LTE services deployed by different MNOs, and their UEs are associated with LTE eNBs.

- Neither Bill’s nor Dave’s UEs have public IP addresses, but their UEs are equipped with a service that allows them to request/receive assistance from their MNOs to help create IP connections to other UEs.

- Bill’s and Dave’s MNOs are equipped to assist UEs with establishing IP connections between UEs belonging to the same or different MNOs.

- Bill and Dave have the same P2P gaming application on their UEs.

x.1.3	Service Flows

- Bill starts the P2P gaming application on his UE and enters the game “lobby” to look for other users currently interested in playing the game.

- Bill sees that his friend Dave is in the game “lobby” and invites him to compete.

- Dave receives an invitation via the game “lobby” on his UE and accepts it.

- Bill and Dave negotiate game rules and P2P session parameters via the game “lobby”.

- Bill’s UE is chosen to be the game’s P2P session server, prompting Dave’s UE to forward its UE and MNO identifiers to Bill’s UE.

- Bill’s UE requests his MNO to help establish an IP connection to Dave’s UE using the identifiers provided by Dave’s UE.

- Bill’s MNO contacts Dave’s MNO and together they help establish an IP connection between the UEs.

x.1.4	Post-conditions

- Bill and Dave are competing with each other in a P2P session over an IP connection created with help from their MNOs.

x.1.5	Potential Requirements

TBD

x.1.6	Potential Impacts or Interactions with Existing Services/Features

TBD