3GPP TSG-SA WG1 Meeting #65	S1-140029

San Francisco, USA, 20th Jan – 24th Jan, 2014

Title:	Revision of Potential Requirements in CSIPTO for On-Demand Dual PDN Connection Use Case

Agenda Item:	X.X FS_CSIPTO:  Co-ordinated P-GW change for CSIPTO (SP 130417)

Source:	Samsung Electronics, Intel

Contact:  	Alper Yegin (alper.yegin@partner.samsung.com)

# Introduction

This contribution proposes introduction of a new requirement in section 4.4.6 (Potential Requirements for On-Demand Dual PDN Connection) of the TR for the missing case of “establishing a PDN connection with an LGW when a new flow does not require IP address preservation and LGW is available”, and clarification/editorial improvements.

_________________________________________________________________________________________________

### Proposal

It is proposed to revise the TR 22.828 Section 4.4 according to the changes marked in the following text.

_________________________________________________________________________________________________

## 4.4	Use Case 4 - CSIPTO On Demand Dual PDN Connection

## 4.4.1	Description

There is a trade-off between data-path optimization and IP address preservation (a.k.a. IP session continuity) when it comes to using SIPTO at the Local Network (LN). Using SIPTO at LN provides a more direct data-path between the UE and its communicating Remote Ends compared to using a PGW in the core network. On the other hand, UE handovers are more likely to break the IP session as the UE may end up changing IP subnets when changing LGW with SIPTO at LN.

The choice between data-path optimization and IP address preservation depends on the applications. Certain applications prefer data-path optimization because their flows are short-lived and they do not need any IP session continuity (e.g., DNS lookups), or they handle session continuity at a higher layer (e.g., Instant Messengers). Certain other applications prefer IP address preservation because an IP session break yields unacceptable user experience (e.g., live video streaming).

When the UE establishes a single PDN connection for a given PDN, it runs the risk of causing suboptimal results and/or disruption for its running applications. If the PDN connection is established with a PGW in the core network, it causes unnecessary transmission delay and core network load for the applications that are not in need of IP address preservation. On the other hand, if the PDN connection is established with an LGW, this causes disruption for the applications that require IP address preservation when the UE changes LGW.

This contribution suggests a use case for establishing a PDN connection with one type of GW, establishing an additional PDN connection to the same PDN using the other type of GW when required by the applications, and performing PDN connection selection on a per-application/flow basis.

4.4.2		Pre-conditions

Operator X has deployed SIPTO at the LN in its network.

LGW may be available to the UE depending on the location of the UE.

The subscription profile associated with the UE determines the default GW to be the LGW.

## 4.4.3	Service Flows

UE establishes the default PDN connection with an LGW, when an LGW is available. If an LGW is not available, then the default PDN connection is established with a PGW.

Figure 4.4.3-1: Default PDN Connection with an LGW.

When the UE attempts to initiate a flow that does not require IP address preservation, that flow can be carried over the PDN connection with the LGW (see the red dotted-line in Figure 4.4.3-1).

When the UE attempts to initiate a flow that requires IP address preservation, that flow shall not be carried over the SIPTO at LN PDN connection as that connection cannot ensure preservation of the IP address. So, the UE establishes an additional PDN connection with the same PDN but using a PGW this time. The new flow is carried over this new PDN connection (see the blue dotted-line Figure 4.4.3-2).

Figure 4.4.3-2: Dual PDN Connection.

The second PDN connection stays on as long as there is an active flow that requires IP address preservation. It can be terminated after the last flow that required IP address preservation has terminated (plus some grace period).

When the UE is attached to an eNB where there is no LGW available, then the default PDN connection is established with the PGW. The single PDN connection is used irrespective of whether the flows require IP address preservation or not (see Figure 4.4.3-3). But, UE can resume using dual PDN connections as soon as an LGW becomes available.

Figure 4.4.3-3: Falling Back to Single PDN Connection in the Absence of an LGW.

The aforementioned use case prefers LGW as the default GW when available. This preference is based on the assumption that most of the flows would not need IP address preservation. This preference can be reversed based on the operator policy and UE type.

## 4.4.4	Post-conditions

The network was able to improve network efficiency by dynamically establishing an additional PDN connection and selectively utilizing SIPTO at the LN without penalizing flows that required IP address preservation.

## 4.4.5	Potential Impacts or Interactions with Existing Services/Features

- This scenario requires ability to establish and simultaneously use two PDN connections with the same PDN (i.e., using the same APN). This is the basic impact of CSIPTO feature on the 3GPP system.

- The IP stack on the UE shall be able to bind each UE-initiated flow to one of the two PDN connections. The determination of the target PDN connection is outside the scope of this contribution.

Editor’s note: Impact of CSIPTO on APN-AMBR is FFS.

## 4.4.6	Potential Requirements

The requirements derived from this use case are:

- The system shall be able to establish a default PDN connection that either supports or does not support IP address preservation. Default preference is given in the subscription profile.

- The system shall be able to establish an additional PDN connection that supports IP address preservation for the same service type when a new flow requires IP address preservation, if the existing PDN connection does not support IP address preservation.

- The system shall be able to establish an additional PDN connection that does not support IP address preservation for the same service type when a new flow does not require IP address preservation, if the existing PDN connection supports IP address preservation and it is possible to establish a PDN connection that does not support IP address preservation.

- The system shall be able to bind a flow to the PDN connection that does not support IP address preservation, if such a PDN connection is available and the flow does not require IP address preservation.

- The system shall be able to bind a flow to the PDN connection that supports IP address preservation, if the flow requires IP address preservation or that PDN connection is the only available one at that time.

- The system shall be able to terminate the additional PDN connection if there are no flows using that connection anymore.