3GPP TSG-SA WG1 Meeting #65	     S1-140275

Taipei, Taiwan, 20th Jan – 24th Jan, 2014                            (Revised from S1-140016)

Title:	Definition of Optimal and Suboptimal PDN Connection in CSIPTO for IMS Use Case

Agenda Item:	8.4 FS_CSIPTO:  Co-ordinated P-GW change for CSIPTO (SP 130417)

Source:	Intel, Orange, Samsung, Sony Mobile Communications

Contact:  	Eric Siow (eric.siow@intel.com)

# Introduction

This contribution proposes revisions to the potential requirements in 4.2.6 of the TR in order to address a FFS item as outlined below:

- The system shall be able to detect when a PDN connection becomes suboptimal and establish a more optimized PDN connection.

Editor’s note:  Examples of “suboptimal” and “more optimized” FFS

_________________________________________________________________________________________________

### Proposal

It is proposed to revise the following in Clause 4.2.6 TR 22.828 (FS_CSIPTO)

_________________________________________________________________________________________________

## 4.2	Use Case 2 - CSIPTO for IMS Use Case

## 4.2.1	Description

While initially intended to address specific residential or enterprise use cases, the SIPTO at the Local Network (SIPTO@LN) architecture can also be used by the operator for offloading of large amounts of traffic as close to the radio network edge as possible. Extensive deployment of SIPTO@LN may lead to an extremely flat architecture like the one depicted in Figure 4.2.1-1, in which the MME controls a number of cluster areas, each one of them served by a co-located SGW/PGW node.

When a UE requests a PDN connection to a specific Access Point Name (APN), the MME selects PGW1 that is geographically close to the current UE location, in order to optimize the backhaul transport (S1 and S5 tunnels) via the Evolved Packet Core (EPC) network (see red dashed line in Figure 4.2.1-1).

As the UE moves from one cluster to another (e.g. from cluster A to cluster B in Figure 4.2.1-1), the SGW is initially relocated whereas the original PGW can be temporarily kept (by using the inter cluster S5 interface), to preserve IP address continuity (see green dashed line in Figure 4.2.1-1).

At some point the EPC may decide that the PDN connection needs to be streamlined and may invoke the SIPTO feature in order to reallocate the PGW. The execution of the SIPTO feature as currently defined involves a release of the original PDN connection, followed by a subsequent re-establishment of a new PDN connection to the same packet data network. Upon the PDN connection re-establishment, a new PGW is assigned that is geographically closer to the user’s current location. The relocation of PGW results in assignment of a new IP address, which means that IP address continuity, is broken and any ongoing IP services may be disrupted.

Figure 4.2.1-1: EPS architecture with SIPTO@LN

The break in IP address continuity may be particularly problematic for long-lived IP flows with IMS services such as an IMS video conference call. This use case describes how CSIPTO enables smooth and uninterrupted IMS services as the UE is transitioned from one PGW to another.

4.2.2		Pre-conditions

Operator X has deployed SIPTO@LN extensively in its network.

The policy of Operator X is to optimize its network efficiency and streamline the connections of UEs to the closest SGW/PGW whenever possible.

Bob initiates a video conference call (an IMS application) when he is in cluster area A.  He is connected to the network via SGW1/PGW1. (See red dashed line in Figure 4.2.3-1).

## 4.2.3	Service Flows

Figure 4.2.3-1: EPS architecture with CSIPTO

When Bob moves into Cluster Area B, his UE is switched from SGW1 to SGW2 but the traffic is still tunnelled to PGW1 (see dashed green line in Figure 4.2.3-1).

Since Bob is engaged in an IMS video conference call (a long-lived flow IMS service for which service continuity is essential), his UE maintains its connection to PGW1 (green dashed line in Figure 4.2.3-1) while requesting a new PDN connection for the same service (IMS). The new PDN connection is established to PGW2 (see dashed black line in Figure 4.2.3-1) because it is closer to Bob’s location.

Once the connection to PGW2 is made, the UE relies on existing IMS service continuity mechanisms for moving long-lived video flows from the PDN connection to PGW1 to the PDN connection with PGW2. When all long-lived flows have been moved to the new PDN connection, the PDN connection to PGW1 is released. The service continuity of the IMS video conference call is thus ensured.

## 4.2.4	Post-conditions

The network was able to improve the network efficiency by streamlining from PGW1 connections to the closer PGW2.

Bob was able to continue with his IMS video conference call without any interruptions even though the underlying user traffic has been switched from PGW1 to PGW2.

## 4.2.5	Potential Impacts or Interactions with Existing Services/Features

- There may be impact on the “Multiple PDN connections to the same APN” feature because in existing Stage 2 specifications, it is assumed that all PDN connections to the same APN are terminated on the same PDN GW.

Editor’s note:  Impact on APN Aggregate Maximum Bit Rate (AMBR) is FFS.

## 4.2.6	Potential Requirements

The requirements derived from this use case are:

- The system shall be able to determine and monitor the changes in the state (optimal or suboptimal) of a PDN connection.

Note: The definition of optimal and suboptimal can be based on a number of implementation criteria like geography, topology and load balancing etc.

- The system shall be able to detect when a PDN connection established becomes suboptimal and decide when to establish an optimal PDN connection for the same service type.

- The system shall be able to identify service flows that require IP address preservation and determine if a suboptimal PDN connection needs to be maintained after  an optimal PDN connection for the same service type is made in order to maintain service continuity.

- The system shall be able to ensure that new service flows are using the optimal PDN connection.

- The system shall be able to release a suboptimal PDN connection either when it determines that there are no service flows using that suboptimal PDN connection and when a time limit has expired.

- When a UE is engaged in IMS services, the system shall be able to seamlessly transition active service flows from a suboptimal PDN connection to the optimal PDN connection by utilizing existing IMS service continuity mechanisms.