3GPP TSG-SA WG1 Meeting #66	S1-141056

Sapporo, Japan, 12 - 16 May 2014

Title:	[IOPS] Minimum set of services for an Isolated E-UTRAN

Agenda Item:	IOPS

Source:	French Ministry of Interior

Contact:	Emmanuelle Villebrun	emmanuelle.villebrun@gendarmerie.interieur.gouv.fr

Abstract: This contribution proposes a minimum set of services to be provided by an Isolated E-UTRAN. An Isolated E-UTRAN may be created by either network backhaul failure or with the addition of one or more eNB(s) to the network to enhance capacity. The proposed text is intended for inclusion in TS 22.278.

This contribution proposes to define a minimum set of services to be provided by an Isolated E-UTRAN. An Isolated E-UTRAN may be created by either network backhaul failure or with the addition of one or more eNB(s) to the network to enhance capacity.

An Isolated E-UTRAN shall be able to provide Public Safety users with a minimum set of services. An isolated E-UTRAN shall be able to provide Public Safety users with professional (e.g. group) communications. Since it would be too complex to offer the same level of service whether the E-UTRAN is isolated or not, we define a minimum set of services that an E-UTRAN may provide when in isolated mode.

A Public Safety user shall be able to start a distress call that reaches:

-	A dispatcher (the dispatcher may be connected directly to the eNB or via a UE) and/or;

-	The members of the Public Safety user’s organization under the coverage of the Isolated E-UTRAN and/or;

-	The members of all Public Safety organizations.

It shall be possible to configure the distress call service on the Public Safety UE to contact a dispatcher, the members of Public Safety user’s organization or the members of all Public Safety organizations.

A distress call shall include voice group communication and shall be able to manage group video communication.

During Isolated E-UTRAN operation each organization shall be able to initiate at least one voice group communication that all the organisation’s members are allowed to join.

During Isolated E-UTRAN operation each organization shall be able to initiate at least one video group communication that all of its members are allowed to join.

During Isolated E-UTRAN operation Public Safety users shall be able to connect to an inter-organization voice group communication. Any Public Safety officer shall be able to join or initiate such communications.

During Isolated E-UTRAN operation Public Safety users shall be able to connect to an inter-organization video group communication. Any Public Safety officer shall be able to join or initiate such communications.

Group communications established during Isolated E-UTRAN operation shall be disconnected when the group communication is no longer required.