3GPP TSG-SA WG1 Meeting #66	S1-141058

Sapporo, Japan, 12 - 16 May 2014

Title:	[IOPS] Operational requirements for Isolated E-UTRAN operation

Agenda Item:	IOPS

Source:	French Ministry of Interior

Contact:	Emmanuelle Villebrun	emmanuelle.villebrun@gendarmerie.interieur.gouv.fr

Abstract: This contribution proposes additional requirements in TS 22.278 to take into account that an Isolated E-UTRAN can be added in the network to provide extra capacity and shall be able, when possible, to synchronize with the rest of the network.

An Isolated E-UTRAN shall possess a HPLMN identification.

The services such as Push to talk shall be reachable through standard IP addressing schemes. We define the home application portal as the entity containing critical communication services.

An Isolated E-UTRAN shall possess a home application portal identification.

An Isolated E-UTRAN shall be able to synchronize with the HSS of its HPLMN when communications can be established with a limited backhaul.

An Isolated E-UTRAN should be able to work with local databases (HSS and application portal). Local databases may be derived from network synchronization or any other way of getting the data locally (e.g. local hard disk). An Isolated E-UTRAN shall be able to synchronize with its home application portal when a limited backhaul link is established.

An isolated E-UTRAN may not be able to contain data related to a whole country wide network.

An isolated E-UTRAN may only need to serve a pre defined number of users or a predefined kind of users.

An Isolated E-UTRAN shall be able to synchronize with the HSS and with the application portal only for a limited part of the home data

The users that the Isolated E-UTRAN may have to serve will be  determined according different criteria:

-	Location

-	Organization

An Isolated E-UTRAN shall be able to use local databases without the need for connection to any HSS or application portal.

When available, the connection between the Isolated E-UTRAN and the HSS shall be used.

When available, the connection  between the Isolated E-UTRAN and  the application portal shall be used.

An Isolated E-UTRAN shall be able to synchronize with the home application portal (for group management) of its HPLMN when a limited backhaul connection can be established.