3GPP TSG-SA WG1 Meeting #66	S1-1411997

Sapporo, Japan, 12 – 16 May 2014	(revision of S1-14xxxx)

Title:	EPS priority management

Agenda Item:	7.2

Source:	Alcatel-Lucent

Contact:	Betsy Covell (betsy.covell@alcatel-lucent.com)

Abstract:

In order to support different priorities of MCPTT calls, requirements on the 3GPP EPS are needed in addition to application level requirements.  Currently EPS systems provide 16 priority levels.  The network operator manages these levels according to the needs of the various customers and applications they support.

The following EPS requirements are proposed to be included in TS 22.179.

## 4.6	PTT Priority Requirements

The MCPTT application manages the MCPTT priorities and issues requests to the EPS for prioritized services.  The EPS maps the MCPTT priorities into the 16 available priority levels.  The following requirements support the EPS management of MCPTT priorities.

It shall be possible to have multiple MCPTT Application priorities which are mapped to available EPS priority levels, based on network operator policy.

It shall be possible for the MCPTT Client on the UE to signal a different statuses to the MCPTT Application relative to a specific group, including at least “Public Safety Emergency” and “Normal”.