3GPP TSG-SA WG1 Meeting #66	S1-1401457

Sapporo, Japan, 12th – 16th May 2014                                                                  (revision of S1-141033)

Title:		Traffic steering in MVNO scenario

Agenda Item:	8.6

Source:		Huawei, China Mobile, Orange, Alcatel-Lucent

Document for:	Discussion and Approval

Work Item:	Flexible Mobile Service Steering (FMSS)

# Introduction

Traffic steering should enable different steering for MNO and MVNO end-users:

- While a MNO has certain steering preferences for traffic of its end-users,

- a MVNO may have different ones for its own end-users even under the same network conditions.

# Discussion

A MNO will select the same traffic steering for 2 users (Simon and Alice) if the 2 users are both under the same network conditions.

But the MNO could select a different traffic steering for another user (Bob) of a MVNO under the same network conditions for example if MVNO want to provide specific VIP treatment for Bob.

| User of MNO or MVNO | Network information: Service type | Network information: Current RAT | Preferred steering |
| --- | --- | --- | --- |
| Simon | Video | 2G | No routing via a video optimizer |
| Alice | Video | 2G |  |
| Bob User of MVNO-1 | Video | 2G | Specific Routing via a video optimizer, advertisement blocking… |

# Proposal

It is proposed to add the following to TR 22.808 (FMSS).

***** 1st CHANGE *****

## 5.x	Traffic steering in MVNO scenario

#### 5.4.1	Description

This use case illustrates the traffic steering policy generation in MVNO scenario.

#### 5.4.2	Pre-conditions

MVNO1 is a mobile virtual network operator of an Operator MNO-A.

Simon is a user of MNO-A.

Bob is a user of MVNO-1.

Both Simon and Bob access video content.

MVNO-1 has an Service Level Agreement with MNO-A.

MVNO-1 decides to provide a gold service for video to guarantee the experience to its users in certain period. Within this plan, its users can access more advanced feature (e.g. advertisement blocking).

#### 5.4.3	Service flows

There is no specific traffic steering policy for MNO-A user Simon.

But MNO-A applies a specific traffic steering policy (e.g. video optimization with advertisement blocking …) for Bob based on MVNO-1 needs.

#### 5.4.4	Post-conditions

#### Bob experience video service as expected by MVNO-1. Several advertisements are delivered to  Simon during the video watching period, and Bob watches the video program without any advertisements.

#### 5.4.5	Potential impacts or interactions with existing Services/Features

None identified.

#### 5.4.6 	Potential requirements

An Operator shall be able to define specific traffic steering policies to apply to users of an MVNO, based on MVNO’s preferences.

The MNO may apply different traffic steering policies for MVNO users than for MNO users.

***** END OF CHANGES *****