3GPP TSG-SA WG1 Meeting #66	S1-141535

Sapporo, Japan, 12 – 16 May 2014	(revision of S1-14xxxx)

Title:	Editor´s notes on TR 22.802

Agenda Item:	8.3 – MAPN study item

Source:	Rapporteur

Contacts:	Ilario Macchi (ilario.macchi@gi-de.com)

Abstract:

Proposal to include some editor´s notes in TR 22.802.

4.1.5.1	UICC based solution

A list of APNs (all the mandatory and optional fields that allow an APN to be identified unambiguously)  is stored in the UICC. This list contains some additional information to link one specific APN to one specific application. The UICC may also contain some information to restrict the access to the list of APNs to specific applications. Each application retrieves the corresponding APN from the list in the UICC, then informs the UE which APN to use to route the application data connection traffic. The list of APNs in the UICC can be updated OTA.

[Editors notes:

- It is FSS what entity/function that identifies APNs unambiguously in the sentence “A list of APNs (all the mandatory and optional fields that allow an APN to be identified unambiguously) is stored in the UICC.”

- It is FFS how it can be possible to link different applications in the ME to the correct APN stored on the UICC.

- It is FFS if it is possible for an ME to prevent an un-autherized application on the ME to read info on another application´s APN info.

- It is FFS if application in the sentence “Each application retrieves the corresponding APN from the list in the UICC, then informs the UE which APN to use to route the application data connection traffic.” is refering to applications on the UICC or applications in the ME.

- It is unclear what the meaning is with “route the application data connection traffic”.

- It is FFS how a new downloaded applications in the ME can get the new associated APN configured timely into the UICC so the application can use the correct APN when requesting a PDN connection the first time. How will a SIM-OTA Server know that the user of the UE has installed a new application and APN needs to be configured for that application.

]

4.2.5.1		UICC based solution

An APN (all the mandatory and optional fields that allow an APN to be identified unambiguously)  is stored in the UICC. The UE retrieves the APN from the UICC and use this APN for the data connection.

[Editors notes:

It is FFS how the USIM subscription profile and other EF files are configured to the UICC when the MNO is seleced first in step 2 and activated in step 3.

It is FFS how the EF files can be kept up to date with correct info for Subscriptions that are pre-provisioned into the UICC and that is not activated for a long period of time.

]