3GPP TSG-SA WG1 Meeting #67	S1-143222

Sophia-Antipolis, France, 18-22 August  2014

Title:		TS22.179 v0.6.0 6.14.2 (.1) Changes to Late Call Entry Performance (General)

Agenda Item:	7.2 MCPTT: Mission Critical Push To Talk over LTE

Source:	Harris Corporation

Contact:	Tom Hengeveld (Thomas.Hengeveld@harris.com)

Abstract: Proposes removing the editor’s note and highlighting in section 6.14.2.1, and making a minor editorial change to the highlighted words.

# Introduction

This contribution is a change  addition to TS22.179 Technical Specification Group Services and System Aspects; Mission Critical Push to Talk MCPTT (Release 13) V0.6.0 section 6.14.2.1, which is a general description of late call entry requirements.  This section contains highlighted text for which an editorial improvement is proposed, and an editor’s note, which was addressed with an accepted contribution in Dublin and should be removed.

Finally, the contribution proposes accepting the performance parameters in 6.14.2.2 and thereby removing the brackets and FFS note in that section.

# Proposed Changes

>>>>>>>>>>>>>>>>>>>>>>>> Start of changes  <<<<<<<<<<<<<<<<<<<<<

### 6.14.2	Late Call Entry performance

#### 6.14.2.1	General

An MCPTT User is able to join or leave already ongoing MCPTT Sessions. Late Call Entry is the activity when an Affiliated MCPTT Group Member joins an MCPTT Session in which other Affiliated MCPTT Group Members are already active. The Late Call Entry time (KPI 4) is the time to enter an ongoing MCPTT Session measured from the time that the user decides to monitor such an MCPTT Session, to the time when the UE’s speaker starts to play the audio. The performance requirements for Late Call Entry time only applies to when there is ongoing voice transmitted at the time the MCPTT User joins the call.

In a Late Call Entry there might be an initial lost audio of the voice burst sent to the new Receiving MCPTT Group Member. Figure 6.14.2.1-1 illustrates the Late Call Entry time

Figure 6.14.2.1-1: Illustration of Late Call Entry time

#### 6.14.2.2	Requirements

The KPIs defined in this section shall apply in an LTE network under traffic load not exceeding [70] % of each network nodes capacity.

The maximum Late Call Entry time (KPI 4a) for calls without application layer encryption within one MCPTT system shall be less than 400 ms for 95 % of all Late Call Entry requests.

The maximum Late Call Entry time (KPI 4b) for application layer encrypted calls within one MCPTT system should meet the requirements for KPI 4 for unencrypted calls.

The maximum Late Call Entry time (KPI 4b) for application layer encrypted calls within one MCPTT system shall be less than 600 ms for 95% of all Late Call Entries into encrypted calls.

The Late Call Entry Time for encrypted calls interworking with other non-LTE PTT systems should meet the requirements for KPI 4b for application layer encrypted calls within one MCPTT system.

The additional Late Call Entry Time for a PTT UE late entering an application layer encrypted call interworking with other non-LTE PTT systems shall not exceed the difference in the encrypted and unencrypted Late Call Entry Times for the interworking system.

For Late Call Entry, a period of Initial lost audio may be acceptable.

>>>>>>>>>>>>>>>>>>>>>>>> End of changes<<<<<<<<<<<<<<<<<<<<<