3GPP TSG-SA WG1 Meeting #67	S1-143472

Sophia-Antipolis, France, 18-22 August 2014	(revision of S1-143150)

# Source:	Qualcomm Incorporated

# Title:	New WID for ACDC Applicability in Connected State

# Document for:	Approval

Agenda Item:	5.2

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	ACDC Applicability to UEs in RRC_Connected State

## Acronym: 	ACDC_CONN

## Unique identifier: 	tbd

## 1	3GPP Work Area

| X | Radio Access |
| --- | --- |
| X | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification

This work item is a …

|  | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
| X | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |
| 40052 | ACDC | 22.011 |

This work item is …

| X | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1	Stage 1

| Source of external requirements (if any) |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |
| Qualcomm | S1-143153 | Revision pending |

Go to §3.

#### 2.3.2	Stage 2

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify:

Go to §3.

#### 2.3.3	Stage 3

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) or external document | Clause | Remarks |

If no identified source of stage 2 information, justify:

Go to §3.

#### 2.3.4	Test spec

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5	Other

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4	Work task

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification

That need for controlling access from RRC_CONNECTED state has been growing, since UE connection state transition timers have been extended in recent Releases, to combat negative impact of frequent state transitions on the network, in face of highly dynamic communications brought about by advent of “smartphones”.  As a result, in Release 12, SA1 introduced in TS22.011 (see Section 4.6) the requirement for “Prevention of mobile-originating signalling and/or data traffic”.  Consequently, the RAN 2 TSG approved the CR #1351 to TS36.331 (see contribution R2-133013), which extends applicability to SSAC to UEs in connected state.  The justification for this CR addresses a weakness in SSAC, if the control is applicable only to UEs in idle state, stating:

Reason for change:

“In addition to that, in the present SSAC specification, even when SSAC is applied in IDLE, an IDLE UE can still  access MMTEL voice by starting  the call after a successful establishment of packet access. This problem also needs to be solved.”

Adverse consequence if not changed:

“MMTEL signalling burst during disaster case can not be controlled. This may cause IMS node to collapse and consequently it may cause high call drop of emergency and high priority voice call. In addition, SSAC can be bypassed by IDLE UE by establishing any packet service first before starting MMTEL call.”

In summary, the quoted CR is intended to rectify a weakness of applying SSAC on idle mode only, namely possibility of gaining access via packet data service, then bypassing SSAC controls to access MMTEL services, when intended to be blocked or reduced via those controls.

Similar kind of weakness would exist in ACDC, unless operators are given the tool to disable a possibility of making a low ACDC category access by first making a high category access, then as UE remains in the connected state for a period of time, making the access for that lower category app.

Assume that ACDC controls are imposed such that all but the highest ranked category are blocked.  However, a technically savvy user may be able to briefly launch an application in the highest ranked ACDC category, gain network access, and therefore get to RRC_CONNECTED state; then due to extended connected state timers, the user can quickly launch an ordinary application belonging to a lower ACDC category (e.g. video upload from the scene of a disaster).  This would not only contribute to the network congestion, but also may add to the RACH load.  Though incidences of such behaviour may be initially limited to technically savvy users, the knowledge of this “trick” could rapidly spread through various means available today, such as social media.

By indicating that at launch of an application even when in RRC_CONNECTED state of the UE, ACDC controls are applicable, this sort of vulnerability can be overcome.

## 4	Objective

The objective of this work item is to address UE behaviour for Release 13, by applying unified ACDC controls, while allowing operator to indicate how these controls apply in various UEs states.

## 5	Service Aspects

None are anticipated.

## 6	MMI-Aspects

None are anticipated.

## 7	Charging Aspects

SA5 will be consulted on charging aspects.

## 8	Security Aspects

SA3 will be consulted on security aspects.

## 9	Impacts

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  |  |
| Don't know |  |  |  | X | X |

## 10	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
| TR22.011 |  | SA1 | n/a | SA 65 (Sep 2014) | SA 65 (Sep 2014) |  |

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |
| 22.011 |  | ACDC | 65 |  |

## 11	Work item rapporteur(s)

Aleksandar Gogic (agogic at qti dot qualcomm dot com)

## 12	Work item leadership

SA1

## 13	Supporting Individual Members

Supporting IM name

Qualcomm