3GPP TSG-SA WG1 Meeting #71	S1-152048

Belgrade, Serbia, 17-21 August 2015	revision of S1-152XXX

Title:	Access to, charging of TV programs and anonymity of reception

Agenda Item:	 8.5

Source:	Nokia Networks, Qualcomm Incorporated

Contact:	juergen.merkel(at)nokia.com

Abstract: This paper proposes a use case on Access to and Charging of TV programs and Privacy of Reception

***** BEGIN CHANGE *****

## 5.x	Use Case N Access to and Charging of TV programs and Anonymity of Reception

### 5.x.1	Description

The MNO can make LTE based TV programs available under the following accessibility and charging regime:

- The MNO may offer programs free to air (FTA) which implies any receiver can receive the program at no associated cost. 
The programs are receivable by all receivers irrespective of whether they are subscribers of the MNO, roaming subscribers of other MNOs or are device or UEs that use receive mode not requiring any subscription. 
FTA supports anonymity of reception, i.e. it is not possible to find out which program was received by the UE and viewed by the subscriber. (This is to mimic the privacy implicit to receiving FTA DVB TV programs)

- The MNO may offer programs free to view (FTV) which implies only subscribers entitled to view the program can receive the program at no associated cost. The receivers of the programs are subscribers of the MNO or are roaming subscribers of other MNOs.

- The MNO may offer programs to subscribers entitled to receive the program and charges on a per program viewed basis e.g. all content on that program can viewed (Pay per program (PPP). The receivers of the programs are subscribers of the MNO or are roaming subscribers of other MNOs.

- The MNO may offer programs as pay TV to subscribers entitled to receive the program and charges on a per view basis e.g. per movie (Pay per view (PPV)). The receivers of the programs are subscribers of the MNO or are roaming subscribers of other MNOs.

### 5.x.2		Pre-conditions

A MNO runs a LTE Based TV service.

### 5.x.3	Service Flows

Depending on the accessibility and charging regime applied by the MNO a subscriber is allowed to receive a certain LTE based TV program and the 3GPP network collects the relevant charging information.

### 5.x.4	Post-conditions

Reception of LTE based TV programs is allowed for certain subscribers and charging data is collected. Receiver can receive the LTE based TV programs as requested by the operator either under  FTA, FTV, PPP or PPV accessibility and charging regime.

### 5.x.5	Potential Requirements

Subject to regulatory or MNO requirements the 3GPP network shall provide mechanisms to provide LTE based TV FTA programs receivable by all UE. The UE may be subscribed to an MNO, a roaming UE of another MNO or may be a receive only device without any subscription.

The 3GPP network shall not collect charging data for FTA. Subject to regional regulation the 3GPP network shall provide the ability to maintain the anonymity of the receiving user if required by the regulator or MNO.

The 3GPP network shall provide mechanisms to restrict the reception of some or all LTE based TV programs to groups of subscribers. The receivers of the programs are subscribers of the MNO or are roaming subscribers of other MNOs.

The 3GPP network shall provide mechanisms to collect charging data on a per subscriber basis indicating at least, but not restricted to, the program viewed, time and duration. The receivers of the programs are subscribers of the MNO or are roaming subscribers of other MNOs.

***** END of CHANGE *****