3GPP TSG-SA WG1 Meeting #71	S1-152183

Belgrade, Serbia, 17 – 21 August 2015

Title:	V2X consideration on inter PLMN/Country

Agenda Item:	8.4 FS_V2XLTE

Source:	LG Electronics Inc.

Contact:	SungDuck Chun <sungduck.chun (at) lge.com>, KD Lee

Abstract: This paper describes the scenario where V2X communication occurs betweens UEs from different PLMNs or countries.

- Introduction

From the car owner’s point of view, the first question that would arise is how much the V2X application would be effective and beneficial. As one may easily guess, the effectiveness of V2X application will be maximized when all vehicles are involved in V2X communication.

In this sense, it is important to make it sure that all vehicles (i.e., UEs supporting V2X) within certain range can communicate with others, regardless of

- to which MNO each vehicle is subscribed to,

- in which country a vehicles moves around,

- how many different frequency bands an MNO has allocated for V2X

However, this requirement should not be translated into excessive UE burden. For example, requiring a UE to be able to transmit and receive via V2V Service in all countries or via all deployed frequency bands is costly and unrealistic.

- Proposal

In this document, we propose to capture one V2X use case and the related potential requirements in Technical Report on LTE-based V2X as described below.

* * * * Start of First Change * * * *

## 5.z	V2X in inter-PLMN/country scenario

### 5.z.1		Description

This use case describes the V2X application information exchange between vehicles belonging to different PLMNs or different countries.

### 5.z.2		Pre-conditions

Figure 5.z.2-1: Overview of inter-PLMN/country scenario

Vehicle A is equipped with UE A supporting V2X and subscribed to MNO M. Vehicle A is registered in country X and currently located in country X. MNO M operates in country X and uses Band B1 for V2X. UE A supports only Band B1.

Vehicle C is equipped with UE C supporting V2X and subscribed to MNO N. Vehicle C is registered in country X and currently located in country X. MNO N operates in country X and uses Band B2 for V2X. UE C supports only Band B2.

Vehicle D is equipped with UE D supporting V2X and subscribed to MNO O. Vehicle D is registered in country Y and currently located in country Y. MNO O operates in country Y and uses Band B3 for V2X. UE D supports only Band B3.

Vehicle E is equipped with UE E supporting V2X and subscribed to MNO P. Vehicle E is registered in country Y and currently located in country Y. MNO P operates in country Y and uses Band B4 for V2X and uses Band Band B1 for non-V2X application. UE E supports only Band B4. MNO P has roaming agreement with MNO M.

Table 5.z.2-1: Summary of pre-conditions for UEs.

| Vehicle (UE) | Subscription | Home country | Supporting Band |
| --- | --- | --- | --- |
| Vehicle A (UE A) | MNO M | X | B1 |
| Vehicle C (UE C) | MNO N | X | B2 |
| Vehicle D (UE D) | MNO O | Y | B3 |
| Vehicle E (UE E) | MNO P | Y | B4 |

Table 5.z.2-2: Summary of pre-conditions for MNOs.

| MNO | Country | Operating Band | Band used for V2X | Partnership |
| --- | --- | --- | --- | --- |
| MNO M | Country X | B1 | B1 | MNO P |
| MNO N | Country X | B2 | B2 |  |
| MNO O | Country Y | B3 | B3 |  |
| MNO P | Country Y | B1, B4 | B4 | MNO M |

### 5.z.3		Service Flows

Application layer of Vehicle A generates V2X application messages. UE A transmits the V2X application message over Band B1 which is used for V2X by MNO M.

The message transmitted by UE A is delivered to nearby UEs which includes UE C which is subscribed to MNO N.

Vehicles A moves into country Y which has MNO O and MNO P.

UE A register itself to MNO P which has roaming agreement with MNO M.

Application layer of Vehicle A generates V2X application messages. UE A transmits the V2X application message over Band B1.

The message transmitted by UE A is delivered to nearby UEs which includes UE D which is subscribed to MNO O and UE E which is operating on other frequency band.

### 5.z.4		Post-conditions

Vehicle C, Vehicle D and Vehicle E are aware of direction, speed and location of Vehicle A.

### 5.z.5		Potential Requirements

The following potential requirements are derived from this use case:

[PR.5.z.5-001]	The system shall be able to support that V2X application message from one UE supporting V2X Service is delivered to other nearby UEs supporting V2X Service belonging to different PLMNs which uses different frequency bands for V2X Service.

[PR.5.z.5-002]	When available frequency bands for V2X Service in different countries are not supported by a UE supporting V2X, the system shall be able to support that V2X application message from the UE is delivered to other nearby UEs supporting V2X Service belonging to different countries.

* * * * End of Addition * * * *