3GPP TSG-SA WG1 Meeting #71	S1-152452

Belgrade, Serbia, 17-21 August 2015

Title:	Adding typical freeway velocity scenarios in Table A.1

Agenda Item:	8.4

Source:	CATT

Contact:	xuhui@catt.cn; lifeng@catt.cn

- Introduction

In this contribution, we propose to add typical freeway velocity scenarios into Table A.1 in 3GPP TR 22.885 v0.2.0.

- Discussion

In China, the velocity of freeway is designed by 120kmph, 100kmph and 80kmph. According to the traffic flow, different design speed has different traffic service levels. In [1], four service levels are given.

Table1 freeway service level class

| Service level | Density [pcu/ (km.ln)] | Design velocity (km/h) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120 |  |  | 100 |  |  | 80 |  |  |
|  |  | Velocity (km/h) | V/C | Maximum service traffic flow[pcu/( h.ln)] | Velocity (km/h) | V/C | Maximum service traffic flow[pcu/(h.ln)] | Velocity (km/h) | V/C | Maximum service traffic flow[pcu/(h.ln)] |
| 1 | ≤7 | ≥109 | 0.34 | 750 | ≥96 | 0.33 | 700 | ≥78 | 0.30 | 600 |
| 2 | ≤18 | ≥90 | 0.74 | 1600 | ≥79 | 0.67 | 1400 | ≥66 | 0.60 | 1200 |
| 3 | ≤25 | ≥78 | 0.88 | 1950 | ≥71 | 0.86 | 1800 | ≥62 | 0.78 | 1550 |
| 4 | ≤45 | ≥48 | Near to 1.0 | <2200 | ≥47 | Near to 1.0 | <2100 | ≥45 | Near to 1.0 | <2000 |
|  | >45 | <48 | >1.0 | 0~2200 | <47 | >1.0 | 0~2100 | <45 | >1.0 | 0~2000 |

Different vehicle density has different velocity in freeway. Velocity of 160kmph is captured and the corresponding requirement is given in Annex A, table A.1 [1]. However, for lower velocity, the requirement is not given yet. We think it’s necessary that service requirements can reflect the more typical velocity cases.

In China, the speed limit is 120kmph. According to four service levels, there are 4 typical absolute velocities, which are 109kmph, 90kmph, 78kmph and 48kmph. The last velocity is captured in #1[2].So we propose to add these velocity in Table A.1 to reflect their requirements. Note that the effect range is calculated as relative velocity (m/s) *4(TTC second).

- Proposal

Proposal: Adding typical freeway velocity scenarios into table A.1 on 3GPP TR 22.885 (see following change).

References

[1] JTG D20-2006 “Design Specification for Highway Alignment”, published by Ministry of Transport of China

[2] 3GPP TR 22.885 v0.2.0.

Start of Change

Table A.1: Example parameters for V2X Services

|  | Effective range | Absolute velocity of a UE supporting V2X Services | Relative velocity between 2 UEs supporting V2X Services | Maximum tolerable latency | Minimum application layer message reception reliability |
| --- | --- | --- | --- | --- | --- |
| #1 (suburban) | 200m | 50kmph | 100kmph | 100ms | 90% |
| #2 (freeway) | 320m | 160kmph | 280kmph | 100ms | 80% |
| #2a (freeway) | 240m | 110kmph | 220kmph | 100ms | 90% |
| #2b (freeway) | 200m | 90kmph | 180kmph | 100ms | 90% |
| #2c (freeway) | 180m | 80kmph | 160kmph | 100ms | 90% |
| #3 (autobahn) | 320m | 280kmph | 280kmph | 100ms | 80% |
| #4 (NLOS / urban) | 100m | 50kmph | 100kmph | 100ms | 90% |
| #5 (urban intersection) | 50m | 50kmph | 100kmph | 100ms | 95% |

End of Change