3GPP TSG-SA WG1 Meeting #71	S1-152725

Belgrade, Serbia, 17-21 August 2015	(revision of S1-152495)

(revision of S1-152128)

Title:	SMARTER Use Case on Broadcasting Support

Agenda Item:	8.1

Source:	Telefonica,

Contact:	    Chris Friel (chris.friel@telefonica.com),

## Broadcasting Support

### x.1.1	Description

The system shall be able to support an enhanced form of MBMS that includes transmission of scheduled linear time Audio and Audio &Video programmes.

### x.1.2	Pre-conditions

Both Mobile and Network support an enhanced and flexible form of MBMS.

The Network has previously allocated MBMS resources.

### x.1.3	Service Flows

- A 3GPP device accesses an advertised MBMS service for broadcast information via a broadcast management system.

- The mobile downloads the 3GPP resource group nomination (e.g. a channel code) for the cell that they are camped on for this broadcast channel from the broadcast management system.

- The User keys in the channel code and starts receiving the broadcast channel subject to any authorisation.

### x.1.4	Post-conditions

### x.1.5	Potential Impacts or Interactions with Existing Services/Features

### x.1.6	Potential Service Requirements

The 3GPP system shall support an interface from external Broadcasters’ management systems.

The 3GPP system shall be able to reserve groups of resources for Broadcast channels

The 3GPP system shall be able to support broadcast of lossless state of the art video streams such as 4k UHD.

The 3GPP device shall be able elect to receive a reduced quality version of the broadcast for display on their device screen (typically less than 12”) or a full quality version of the same channel for presentation  to a video presentation monitor (typically much larger than their device , ie:32” to 72”screen size)

The 3GPP System shall allow the UE to receive broadcasts selected by the user (from the Broadcaster’s management system) in accordance with any appropriate authorisations by the Broadcaster.

Note:

UHD: 3840 x 2160, 50FPS, AVC ~300Mbit/s
UHD: 3840 x 2160, 50FPS, HEVC ~ 150Mbit/s