3GPP TSG-SA WG1 Meeting #71	S1-152594

Belgrade, Serbia, 17 – 21 August 2015	revision of S1-152182

Title:	V2X by UE type RSU

Agenda Item:	8.4 FS_V2XLTE

Source:	LG Electronics Inc.

Contact:	SungDuck Chun <sungduck.chun (at) lge.com>

Abstract: This paper describes a use case involving UE type RSU.

- Introduction

According to the current definition for RSU in TR 22.885 v0.2.0, RSU can be implemented either in a stationary UE (a.k.a. UE-type RSU in the rest of this contribution) or in an eNB (a.k.a. eNB-type RSU in the rest of this contribution). As the next step of defining what an RSU is required to do when it is implemented in a stationary UE, this contribution proposes a use case and a set of potential requirements based on the scenario where UE supporting V2X interacts with UE-type RSU.

Unlike an eNB-type RSU capable of scheduling to allocate radio resources for V2X, a UE-type RSU has limited capability. It does not necessarily have authority to perform scheduling since it is kind of UE and therefore it should follow the instruction given by someone else before it can transmit. Also, it is not expected to provide PDN connectivity for other UEs supporting V2X unless it is acting as UE-to-Network Relay (e.g., ProSe UE-to-Network Relay).

Furthermore, once UE-type RSUs are deployed in a certain area, several MNO’s can do so in the same areas. If coordination among MNOs is limited in that situation, RSUs of same purpose are likely to be deployed. And, because UE-type RSUs are easy to deploy and can be customized to each different applications, the number of UE-type RSUs can be quite huge. Thus, the UE should not be required to monitor all UE-type RSUs in the area.

- Proposal

In this document, we propose to capture the following V2X use case and potential requirements in Technical Report on LTE-based V2X as described below.

* * * * Start of Addition * * * *

5.z	Use Case – V2X by UE type RSU

5.z.1	Description

This use case describes the scenario where UE supporting V2X discovers and communicates with UE type RSU.

5.z.2	Pre-conditions

Vehicle A is normal passenger car equipped with a UE A supporting V2X Service.

UE A, subscribed to MNO X, is located in Cell 1 managed by eNB X.1.

Vehicle B is also normal passenger car equipped with a UE B supporting V2X Service.

UE B, subscribed to MNO X, is located in Cell 1 managed by eNB X.1.

Cell 1 and Cell 2 are managed by eNB X.1 owned by MNO X.

UE C is configured to function as RSU C and located within Cell 1. RSU C broadcasts traffic signal information such as when green light is turned on and off.

UE D is configured to function as RSU D and belongs to MNO Y.

Both RSU C and RSU D are located near UE A.

Cell 2 is a neighbouring cell of Cell 1. In Cell 2, there are no UE-type RSUs.

Fig. 5.z.2-1: Example deployment scenario

5.z.3	Service Flows

UE A and UE B gets authorization to use V2X service and starts transmission of V2X application information such as its speed, heading and location.

RSU C, which is same as UE C, requests for eNB X.1 to allocate with radio resources. eNB X.1 allocates radio resources to RSU C, and RSU C uses the allocated radio resources to broadcast traffic light signal information.

UE A moves to near RSU C and receives information transmitted by RSU C.

Meanwhile, owner of UE A starts streaming service application. UE A establishes RRC connection in Cell 1 through eNB X.1 to serve the streaming service application.

5.z.4	Post-conditions

Owner of UE A adjusts speeds so that the vehicle passes the intersection just before the traffic light turns red.

Owner of UE A keeps track of movement of Vehicle B and avoids car collision.

5.z.5	Potential Requirements

[PR.5.z.5-001] The Network shall be able to provide means for MNO to authorise UE-type RSU.

[PR.5.z.5-003] The Network shall be able to control radio resources used by UE-type RSU.

[PR.5.z.5-004] When UE supporting V2X service is served by E-UTRA(N), the UE shall use the E-UTRA(N) for PDN connectivity.

* * * * End of Addition * * * *