3GPP TSG-SA WG1 Meeting #72	S1-154130

Anaheim, California, 16-20 November 2015	(revision of S1-15xxx)

Title:	SMARTER: Access from less trusted networks

Agenda Item:	8.5

Source:	Qualcomm Incorporated

Contact:	Jack Nasielski (jackn at qti dot qualcomm dot com)

Abstract: Text for Section 5.6  of SMARTER TR for Network Operation

# Proposal

This document proposes text to fill in the skeleton of the Network Operations TR.  Specifically it provides text for the currently empty sections marked “Access.”  This text will need to be merged with text from other use cases in that section.

--- START MODIFIED CLAUSE ---

## 5.6	Access

### 5.6.1	Description

In order to enable more flexible deployments, operators will support access and connectivity via less trusted networks. For example, current core network functions are expected to be deployed nearer the edge and multiple parties may own the network equipment traversed. In these networks, privacy across access networks elements cannot be assumed.

The current 3GPP system requires a UE to provide its IMSI unencrypted over the air during the initial attach. A passive attacker can identify a user from on the IMSI by observing the OTA traffic. This also enables the attacker to track the user while the user roams to another network.

In addition, when the UE is roaming, the UE has provide its IMSI to the serving network for authentication and the IMSI is again stored across the network elements of the roaming network, e.g., in the MME, S-GW, P-GW. This enables the serving network to trace the user.

Example: Angela visits another country and switches on her mobile.  The visited PLMN requests her IMSI in order to register.  The visited PLMN (and potentially other agencies) are then aware of her visit.  3GPP should consider a method to protect subscribers’ identities while roaming.

### 5.6.2	Potential Requirements

Subject to regulatory requirements, the UE may use a temporary identifier for initial attach that hides its long-term identity. The home network shall be able to associate this temporary identifier to the long-term identity.

--- END NEW CLAUSE ---