3GPP TSG-SA WG1 #73	S1-160099

Okinawa, Japan, 1 – 5 February 2016

Title:	mIoT Terminology for Connection mode

Agenda Item:	8.2	FS_SMARTER-mIoT

Source:	LG Electronics Inc., Sony, Intel

Contact:	SungDuck Chun <sungduck.chun (at) lge.com>

Abstract: This contributions proposed to align terminologies used to describe connection mode in mIoT connectivity families.

- Introduction

The contribution proposes to align terminology used to describe different connection models in mIoT.

- Discussion

In section 5.2.1 and section 5.2.2, following wording is used without clear definition or alignment:

- Relay UE

- Direct connection to the network

- Connection to the network through a relay

- Short range communication to nearby devices

- A RAT connection to a 3GPP network

- A direct connection

- A relayed connection

- A relay connection

- A standalone mode

- Network communications

- Network connection

- Communication link

- Relayed device

- Relaying device

- Direct link

- Direct access

- Indirect access

- indirect access from a relay

- devices which are connected with network only via a relay UE

Undefined and ambiguous text leads to concerns. Following is a For example:

- Short range communication

  - How short is ‘short range’? When a cell radio is 500ms, can a UE located within 10m from cell tower also be considered as ‘within short range’? If the intention is communication between devices, it is better to clearly state it.

  - In some text, ‘short range communication’ is used to describe to relayed network connection. In other text, it is used to describe communication between devices.

- A relay connection

  - Is it a connection between a relayed UE and relaying UE?

  - Or, is it an end to end connection between a relayed UE and network?

  - Or, is it a connection from a relaying UE to network?

- The description and the figure 5.2.1-1 is not aligned. I.e., the description talks about the different connection mode but the figure shows 3GPP RAT and non-3GPP RAT connection

- Due to the text ‘relayed or relaying device’, it is ambiguous what relay device refers.

- “direct link’, ‘direct access’, ‘a direct connection’ is ambiguous because it can mean either interface between devices or either interface between UE and Network. While wording such as “direct connection to the network’ exist, wording such as ‘direct connection to the nearby device’ is not used.

- Different wording ‘connection to the network through a relay’, ’a relayed connection’, ’a relay connection’ may refer same connection mode.

- The term ‘A standalone mode’ is ambiguous because it can mean either ‘direct connection to the network’ or ‘communication among devices without network connection’

- The term ‘devices which are connected with network only via a relay UE’ is ambiguous because it can mean either a device which does not support direct network connection, or a device which supports also direct network connection but currently connected to network via a relay UE.

- The expression “network connection” and similar expression needs to be modified because TR21.905 already includes definition for ‘network connection’ and its meaning is different from the mIoT BB perspective.

Considering the intention of the used wording, following figure describes different types of connection.

Following is description of the above figure:

- 3GPP Connection is the communication between a IoT device and 3GPP network:

  - Direct 3GPP Connection is one type of 3GPP Connection, where there is no other relay UE between the IoT device and the 3GPP network.

  - Indirect 3GPP Connection is another type of 3GPP Connection, where there is another relay UE between the IoT device and the 3GPP network.

    - Indirect 3GPP Connection is established using at least one Direct Device Connection and at least one direct 3GPP Connection.

- Direct device connection is the communication between two IoT devices without any Network in-between:

For Indirect 3GPP Connection, following figures shows what is a relay UE and a relayed device.

In above figure, a Relay UE is a device which relays a traffic for other devices. And, a Relayed Device is a device which is using a relay UE for a direct device connection or for a 3GPP Connection.

- Proposal

It is proposed to agree on text proposal as described below.

* * * * Start of 1st Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Direct device connection: The connection between two devices without any network entity in the middle

Direct 3GPP connection: One mode of 3GPP connection, where there is no relay UE between the device and the 3GPP network

Indirect 3GPP connection: One mode of 3GPP connection, where there is a relay UE between the device and the 3GPP network. Indirect 3GPP connection is established using at least one direct device connection and at least one direct 3GPP connection.

3GPP connection: The connection between a device and a 3GPP network

Device: An entity with a wireless connection, direct and/or indirect, to the 3GPP network

Relay UE: an entity that has its own subscription/credentials for 3GPP network access and is capable of relaying communications to the network for other device.

Editor's note:	In above, it is assumed that relay UE is one of device. As study on m progresses, further update of definition or change to other term may need to be done for a device. It is FFS whether a device is same as a UE or whether definition of a UE needs update.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

* * * * Start of 2nd Change * * * *

## 5.2	Connectivity aspects

### 5.2.1	Description

As described in the Smart Wearables and Bio-connectivity related use cases in [2], the Internet of Things will support various connectivity models. The  devices can connect with the network directly or connect with the network using another device as a relay UE, or they may be capable of using both types of connections. The  devices can range from simple wearables, such as a smart watch or a set of sensors embedded in clothing, to a more sophisticated wearable device monitoring biometrics. They can also be non-wearable devices that communicate in a Personal Area Network such as a set of home appliances (e.g., smart thermostat and entry key), or the electronic devices in an office setting (e.g., smart printers), or a smart flower pot that can be remotely activated to provide water to the plant. Essentially, one or more of the following connection models will apply for any  device. Figure 5.2.1-1 illustrates the connection models,

- Direct 3GPP connection (e.g., a sensor that communicates with an application server or with another  device through 3GPP network)

- Indirect 3GPP connection (e.g., a smart wearable that communicates through a smart phone to the 3GPP network)

- Direct device connection (e.g., a bio metric device that communicates directly with other biometric devices or with a smart phone associated with the same patient)

3GPP and non-3GPP RATs may be supported for  connectivity scenarios.  For example, a RAT might be trusted or untrusted WLAN or a 5G 3GPP RAT.  A RAT used for direct device connection might be Bluetooth, WLAN or a 3GPP RAT. A RAT used for direct 3GPP connection might be a 3GPP RAT or a trusted or untrusted WLAN

Figure 5.2.1-1: Connectivity mode for devices

Various combinations of these connectivity scenarios will also need to be supported, including the following.

- Devices that can switch between a direct 3GPP connection and an indirect 3GPP connection

- Devices that only support a direct device connection (e.g. does not support 3GPP connection)

- Groups of devices that communicate among themselves using an direct device connection, and also through an indirect 3GPP connection to the network through one of the devices (e.g., personal area network, home office network)

An example of a combination connectivity scenario is a Personal Area Network for biometric devices. Figure 5.2.1-2 illustrates a group of biometric devices that communicate among themselves using a direct device connection and also communicate with a 3GPP network through an indirect 3GPP connection. When used in a hospital setting, the biometric devices may communicate with each other and provide information to local display equipment for doctors and nurses to monitor. However, these biometric devices do not need to operate only in a direct device connection mode. When information needs to be uploaded to a server, for example to update the patient’s records, it can be done through a relay UE. In this scenario, the relay UE will need to provide 3GPP connection for multiple  devices supporting onlydirect device connection.

Figure 5.2.1-2: Connectivity model for devices using the direct device connection and the indirect 3GPP connection

Some additional scenarios might apply when a Personal Area Network of  devices (e.g., a person wearing several smart wearables) is in indirect 3GPP connection mode:

- A relayed  device (e.g., smart watch) and relay UE can belong to the same subscriber or different subscribers of the same PLMN.

- A relayed  device (e.g., smart watch) and relay UE can have subscriptions associated with different PLMNs. This is a roaming case.

In either case, the devices need to determine that the smart watch is authorized for an indirect 3GPP connection before a 3GPP connection is established.  Both devices also need to be authenticated with the network before the 3GPP connection is established.

In all of the scenarios described above, key aspects for 5G include the following:

- Providing secure communications between the  devices using direct device connection and between  devices and the network,

- Taking into consideration QoS when choosing connection mode and RAT,

- Supporting the desired end user services, including real-time voice and data,

- Minimizing power consumption on devices,

- Supporting roaming access to the network, whether the relayed  device or a relay UE is roaming,

- Supporting multiple relayed  devices behind a relay UE,

- Providing service continuity for  devices that switch between an indirect 3GPP connection and a direct 3GPP connection,

- Providing service continuity for  devices that switch from one indirect 3GPP connection to another indirect 3GPP connection,

- Ensuring  devices using a direct device connection, or using a relay UE for a 3GPP connection, are authorized to do so,

- Providing flexibility in the choice of RAT (within the scope of 5G) used by the devices.

In each of the connectivity models, use of different type of access to the network (3GPP and non-3GPP RAT), will imply different types of requirements:

- Some requirements will be specific to  devices connecting to the 3GPP system through a 3GPP-RAT

- Some requirements will be specific to  devices connecting to the 3GPP system through a non-3GPP RAT

- Some general requirements will apply regardless of the RAT used for the direct device connection or the 3GPP connection.

NOTE 2:	Groups of devices that only communicate among themselves using direct device connection over a non-3GPP RAT with no 3GPP connection do not need to be considered in 3GPP.

### 5.2.2	Traffic scenarios

In this section, we propose the traffic scenarios for the Connectivity Aspects family. According to the description, there are several types of connectivity aspects, including  devices in a direct 3GPP connection mode,  devices in an indirect 3GPP connection modeand  devices in a direct device connection mode. According to these several types of connectivity aspects, there are several traffic scenarios in the below. The traffic scenarios below use one  device to access the relay UE as an example. Actually, the relay UE can support more than one  device to access and relay their traffic to the network.  In each case excluding the direct 3GPP connection, the  device may communicate with the relay UE using 3GPP or non-3GPP RAT.

#### 5.2.2.1	Traffic scenario 1 of a  device in direct 3GPP connection mode

The traffic scenario below is a  device in a direct 3GPP connection mode, including  the case where the device and eNB belong to the same or different PLMNs.

- The  device is subscribed to PLMN1. The eNB belongs to PLMN1.

- The  device is subscribed to PLMN2. The eNB belongs to PLMN1. The  device is in the roaming case. The PLMN1 and PLMN2 have a roaming agreement.

When the  device is out of coverage of a relay UE (e.g., larger than [TBD]m), the  device is in direct 3GPP connection mode. The  device might support both real time services (e.g., up to [TBD]bps real time voice and/or up to [TBD]bps real time video) and data transmission services (e.g., up to [TBD]bps).

Figure 5.2.2.1: Traffic scenario1 of a  device in direct 3GPP connection mode

#### 5.2.2.2	Traffic scenario 2 of a  device in indirect 3GPP connection mode

In this scenario2, the  device and the relay UE belong to the same PLMN and have the same or different subscriptions. The detail scenario2 is:

- The  device has subscription A with PLMN1. The relay UE has the same subscription A with PLMN1. The eNB belongs to PLMN1.

- The  device has subscription A with PLMN1. The relay UE has the subscription B with PLMN1. The eNB belongs to PLMN1.

For the 3GPP RAT case:

When the  device is in the coverage of the relay UE (e.g., less than [TBD]m), the  device is in indirect 3GPP connection mode. The  device might support both real time services (e.g., up to [TBD]bps real time voice and/or up to [TBD]bps real time video) and data transmission services (e.g., up to [TBD]bps).

For the non-3GPP RAT case:

When the  device is in the coverage of the relay UE, the  device is in indirect 3GPP connection mode. The  device might support both real time services (e.g., real time voice and/or real time video) and data transmission services.

Figure 5.2.2.2: Traffic scenario2 of a  device is in indirect 3GPP connection mode

#### 5.2.2.3	Traffic scenario 3 of a  device in indirect 3GPP connection mode in the roaming case

In this scenario3, the  device and relay UE belong to the different PLMNs and have 2 roaming cases. One is the  device in the roaming case, and the other is the relay UE in the roaming case. The detail scenario3 is:

- The  device has subscription A with PLMN1. The relay UE has the subscription B with PLMN2. The eNB belongs to PLMN1. The relay UE is in the roaming case.

- The  device has subscription A with PLMN2. The relay UE has the subscription B with PLMN1. The eNB belongs to PLMN1. The  device is in the roaming case.

The PLMN1 and PLMN2 have a roaming agreement.

For the 3GPP RAT case:

When the  device is in the coverage of the relay UE (e.g., less than [TBD]m), the  device is in indirect 3GPP connection mode. The  device might support both real time services (e.g., up to [TBD]bps real time voice and/or up to [TBD]bps real time video) and data transmission services (e.g., up to [TBD]bps).

For the non-3GPP RAT case:

When the  device is in the coverage of the relay UE, the  device is in indirect 3GPP connection mode. The  device might support both real time services (e.g., real time voice and/or real time video) and data transmission services.

Figure 5.2.2.3: Traffic scenario3 of a  device in indirect 3GPP connection mode in the roaming case

#### 5.2.2.4	Traffic scenario 4 of  devices communication with each other in direct device connection mode

In this scenario4, each  device has the same or its own subscription. The detail scenario4 is:

- The  device1,  device2 and  device3 have the same subscription A.

- The  device1 has subscription A. The  device2 has subscription B. The  device3 has subscription C.

For the 3GPP RAT case:

When the  devices are in a coverage (e.g., less than [TBD]m), the  devices can communicate with each other using direct device connection mode using a 3GPP RAT. The  devices supports the data transmission service (e.g., up to [TBD]bps) for communication with each other.

For the non-3GPP RAT case:

When the  devices are in a coverage, the  devices can communicate with each other using direct device connection mode using a non-3GPP RAT case. The  devices support the data transmission service for communication with each other.

Figure 5.2.2.4: Traffic scenario4 of  devices communication in direct device connection mode

#### 5.2.2.5	Traffic scenario 5 of service continuity

For the 3GPP RAT case:

For the  device which supports direct 3GPP connection mode and/or indirect 3GPP connection mode, the 3GPP system needs to guarantee the service continuity when the  device changes from direct 3GPP connection to Indirect 3GPP connection mode in which the direct device connection uses 3GPP RAT, and vice versa or when the  device changes relay UEs in indirect 3GPP connection modein which direct device connection uses 3GPP RAT. So this type of  devices will support service continuity for all the cases from case 1 to case 10 in the below Figure 5.2.2.5.

For the  device which supports only indirect 3GPP connection modeand 3GPP RAT, the 3GPP system needs to guarantee the service continuity when the  device changes relay UEs in indirect 3GPP connection mode in which direct device connection uses 3GPP RAT. So this type of  device will support service continuity only for the case 7, case 8, case 9 and case 10 in the below Figure 5.2.2.5.

For the non-3GPP RAT case:

For the  device which supports direct 3GPP connection mode and/or indirect 3GPP connection mode, the 3GPP system needs to maintain the ability to route signalling and data and minimize the interruption time for the service provided for the  device:

- when the  device changes from direct 3GPP connection mode to indirect 3GPP connection mode in which direct device connection uses non-3GPP RAT, and vice versa, or

- when the  device changes relay UEs in indirect 3GPP connection mode in which the used RAT for direct device connection changes from 3GPP RAT to non-3GPP RAT, or

- when the  device changes relay UEs in indirect 3GPP connection mode in which the used RAT for direct device connection continues to be non-3GPP RAT.

Device which supports both a direct 3GPP connection and an indirect 3GPP connection will maintain the ability to route signalling and data and minimize the interruption time for the service provided for the  device for all cases from case 1 to case 10 in the below Figure 5.2.2.5.

Device which supports only an indirect 3GPP connection will maintain the ability to route signalling and data and minimize the interruption time for the service provided for the  device only for the case 7, case 8, case 9 and case 10 in the below Figure 5.2.2.5.

Figure 5.2.2.5: Traffic scenario5 of service continuity

### 5.2.3	Potential requirements

Requirements below are for all kinds of connectivity aspects, including  devices which supports both direct or indirect 3GPP connection mode and  devices which supports only indirect 3GPP connection mode, using 3GPP RAT or non-3GPP RAT, in licensed band, or unlicensed band:

- The 3GPP system shall support a  device to access to the 3GPP network in direct 3GPP connection mode.

- The 3GPP system shall support a device to access to the 3GPP network indirectly via a relay UE when the  device and the relay UE can communicate in direct device connection mode.

- The 3GPP system shall have the means to authorize a relay UE to provide relay functionality to other  devices.

- An  device shall establish a trusted relationship with the relay UE, before the  device is indirectly connected to the 3GPP network via the relay UE.

- Before using  direct device connection mode, devices shall first establish a trusted relationship with each other.

- Editor’s Note: For the non-3GPP RAT case, to establish a trusted relationship for  devices communication with each other needs to be further studied.

- The 3GPP system shall support an indirect 3GPP connection where  device and a relay UE subscribe to different PLMNs, when the PLMNs have a roaming agreement.

- Editor’s Note: Identities and subscriptions for  devices need to be further studied.

- The 3GPP system shall support real time services (e.g. real time voice and/or real time video) for a  device, whether the  device is in indirect 3GPP connection mode or in direct 3GPP connection mode.

- The 3GPP system shall support data transmission services for a  device, whether the  device is in indirect 3GPP connection mode or in direct 3GPP connection.

- The 3GPP system shall support service continuity for a  device, when the  device changes relay UEs in indirect 3GPP connectionusing 3GPP RAT.

- The 3GPP system shall maintain the ability to route signalling and data and minimize the interruption time for the service provided for a  device, when  device changes from indirect 3GPP connection via a relay UE using 3GPP RAT to indirect 3GPP connection via another relay UE using non-3GPP RAT, vice versa.

- The 3GPP system shall maintain the ability to route signalling and data and minimize the interruption time for the service provided for a  device, when  device changes from indirect 3GPP connection via a relay UE using non-3GPP RAT to indirect 3GPP connection via another relay UE using non-3GPP RAT.

- The 3GPP system shall optimize the battery consumption of a  device, whether the  device is in direct 3GPP connection mode or in indirect 3GPP connection mode.

- The 3GPP system shall optimize the battery consumption of a relay UE, via which a  device is in indirect 3GPP connection mode.

- The 3GPP system shall identify, address and reach a  device, when the  device is in indirect 3GPP connection mode.

- The 3GPP system shall support end to end QoS for a  device, when the  device is in indirect 3GPP connection mode.

Requirements below are only for  devices which is in direct 3GPP connection mode and in indirect 3GPP connection mode for 3GPP/non-3GPP RAT case, in licensed band, or unlicensed band:

- The 3GPP system shall support service continuity for a  device, when the  device changes from a direct 3GPP connection to an indirect 3GPP connection using 3GPP RAT, and vice versa.

- The 3GPP system shall maintain the ability to route signalling and data and minimize the interruption time for the service provided for a  device, when the  device changes from a direct 3GPP connection to an indirect 3GPP connection using non-3GPP RAT, and vice versa.

Requirements below are only for  devices which is in indirect 3GPP connection mode only for 3GPP/non-3GPP RAT case, in licensed band, or unlicensed band:

- The 3GPP System shall support a timely, efficient, reliable and secure mechanism to transmit the same information to multiple  devices which supports only an indirect 3GPP connection.

# 6	Considerations

## 6.1	Considerations on security

The 3GPP system shall support end-to-end integrity protection and confidentiality for data transmitted to/from an  device, when the  device is in indirect 3GPP connection mode.

The 3GPP system shall support a resource efficient mechanism to authenticate an  device, when the  device is in indirect 3GPP connection mode.

The 3GPP System shall support a resource efficient mechanism that provides security, authentication and authorization for a  device, which only support indirect 3GPP connection mode.

## 6.2	Considerations on charging

This section describes the requirements for collecting charging data for a  device. The requirements also apply in the roaming case.

The 3GPP system shall support online and offline charging for a device, whether the  device is in direct 3GPP connection mode or indirect 3GPP connection mode.

The 3GPP system shall be able to separate the charging data of a device from the charging data of a relay UE, when the  device is in indirect 3GPP connection via the relay UE.