3GPP TSG-SA WG1 Meeting #73	S1-160104

Okinawa, Japan, 1-5 February 2016

Title:	Proposed text for Overview section for V2X TS

Agenda Item:	7.4

Source:	CATT

Contact:	lifeng@catt.cn

Abstract: This contribution proposes the text for Overview section of TS 22.cde v0.1.0 on Normative Work for V2X (V2XLTE).

- Introduction

In[1],the V2V,V2I,V2N and V2P potentail requirements are discussed.This document proposes to introduce V2V,V2I,V2N and V2P into overview section in [2].

- References

[1] 3GPP TR 22.885 v14.0.0.

[2] S1-154608 3GPP TS 22.cde V0.1.0 (2015-11)

Start of Change 1

4	Overview on V2X (informative)

4.1	Types of V2X

The vehicular communication in the present specification, referred to as Vehicle-to-Everything (V2X), contains the following four different types:

-	Vehicle-to-Vehicle (V2V) Communications

-	Vehicle-to-Infrastructure (V2I) Communications

-	Vehicle-to-Network (V2N) Communications

-	Vehicle-to-Pedestrian (V2P) Communications

Figure 4.1-1: Types of V2X (V2V, V2P, V2N and V2I)

Note: 	These four types of V2X can use “co-operative awareness” to provide more intelligent services for end-users. This means that transport entities, such as vehicles, roadside infrastructure, application server and pedestrians, can collect knowledge of their local environment (e.g., information received from other vehicles or sensor equipment in proximity) to process and share that knowledge in order to provide more intelligent services, such as cooperative collision warning or autonomous driving.

Three basic classes of applications for providing ITS services: road safety, traffic efficiency, and other applications can be found in e.g., [3], [4].

4.1.2	Vehicle-to-Vehicle (V2V) communications

E-UTRAN allows such UEs that are in proximity of each other to exchange V2V-related information using E-UTRA(N) when permission, authorisation and proximity criteria are fulfilled. The proximity criteria can be configured by the MNO. However, UEs supporting V2V Service can exchange such information when served by or not served by E-UTRAN.

The UE supporting V2V applications transmits application layer information (e.g. about its location, dynamics, and attributes as part of the V2V Service). The V2V payload must be flexible in order to accommodate different information contents, and the information can be transmitted periodically according to a configuration provided by the MNO.

V2V is predominantly broadcast-based; V2V includes the exchange of V2V-related application information between distinct UEs directly and/or, due to the limited direct communication range of V2V, the exchange of V2V-related application information between distinct UEs via infrastructure supporting V2X Service, e.g., RSU, application server, etc.

4.1.3	Vehicle-to-Infrastructure (V2I) communications

The UE supporting V2I applications sends application layer information to RSU. RSU sends application layer information to a group of UEs or a UE supporting V2I applications.

4.1.4	Vehicle-to-Network (V2N) communications

The UE supporting V2N applications communicates with a serving entity supporting V2N applications. Both parties communicate with each other via LTE network. The serving entity which is out of 3GPP scope may be located in the radio access network, the core network or the internet.

4.1.5	Vehicle-to-Pedestrian (V2P) communications

E-UTRAN allows such UEs that are in proximity of each other to exchange V2P-related information using E-UTRA(N) when permission, authorisation and proximity criteria are fulfilled. The proximity criteria can be configured by the MNO. However, UEs supporting V2P Service can exchange such information when served by or not served by E-UTRAN.

The UE supporting V2P applications transmits application layer information. Such information can be transmitted either by a vehicle with UE supporting V2X Service (e.g., warning to pedestrian), or by a pedestrian with UE supporting V2X Service (e.g., warning to vehicle).

V2P includes the exchange of V2P-related application information between distinct UEs (one for vehicle and the other for pedestrian) directly and/or, due to the limited direct communication range of V2P, the exchange of V2P-related application information between distinct UEs via infrastructure supporting V2X Service, e.g., RSU, application server, etc.

The main difference to V2V communications lies in the properties of the UE. A UE supporting V2P applications (i.e. used by pedestrian) might have lesser battery capacity, the radio sensitivity might be more limited e.g. due to antenna design, etc.

End of Change 1