3GPP TSG-SA WG1 Meeting #73	S1-160352

Okinawa, Japan, 1-5 February 2016	(revision of S1-160049)

Title:	FS_SMARTER-NEO network capability exposure clean up

Agenda Item:	8.5

Source:				ZTE Corporation, Huawei, China Unicom,China Mobile, China Telecom

Contact:			xu.ling@zte.com.cn

Abstract: This document proposes to clean up the network capability exposure description and potential requirements in NEO TR.

Discussion:

- The follow description has been in section5.1.1.

Flexibility enabler 3:  Network capability exposure

Network capability exposure is to provide the network capabilities that are in need by the 3rd party ISP/ICP. Based on 3GPP SEES and FMSS, the operator can provide network capabilities e.g. QoS policy to 3rd party ISPs/ICPs. However, with the advent of 5G, some new network capabilities need to be considered to be exposed to the 3rd party. There are the following examples:

- to allow the 3rd party to dynamically custom the dedicated network slice cater for different diverse use cases, the network slicing capability may allow third parties to create, manage a network slice configuration via suitable APIs, within the limits set by the operator (cp. [2], clause 5.49).

- the 3GPP network enables hosting of services (including both MNO provided services and 3rd party provided services) closer to the end user to improve user experience and save backhaul resources(cp. [2], clause 5.37).

It need to clarified that the services hosted is the application not network service :

- to allow hosting of applications (including both MNO provided applications and 3rd party provided applications) closer to the end user to improve user experience and save backhaul resources(cp. [2], clause 5.37).

2. about the requirement for network capability exposure in section5.1.2.3

#The requirement follow in section5.1.2.1 network slice section has more relationship with network capability exposure, so it is suggested to move to section 5.1.2.3 as follow:

The 3GPP system shall support APIs that provide network status information to applications (e.g., to allow applications to use network resources efficiently).

Note: move from section5.1.2.1

After read the existed requirement in section5.1.2.3, follow requirement can be seen the higher level requirement cover the above requirements:

The 3GPP System shall be able to support to expose the 5G network information and capabilities to the 3rd party, such as the network slicing capability

So, it is suggested to merge these two parts as follow:

The 3GPP System shall be able to support to expose the 3GPP network information and capabilities to the 3rd party, such as the network slicing capability and the network status information).

Proposed Text Change:

5.1	System flexibility

5.1.1	Description

……

Flexibility enabler 3:  Network capability exposure

Network capability exposure is to provide the network capabilities that are in need by the 3rd party ISP/ICP. Based on 3GPP SEES and FMSS, the operator can provide network capabilities e.g. QoS policy to 3rd party ISPs/ICPs. However, with the advent of 5G, some new network capabilities need to be considered to be exposed to the 3rd party. There are the following examples:

- to allow the 3rd party to dynamically custom the dedicated network slice cater for different diverse use cases, the network slicing capability may allow third parties to create, manage a network slice configuration via suitable APIs, within the limits set by the operator (cp. [2], clause 5.49).

- to allow hosting of applications (including both MNO provided applications and 3rd party provided applications) closer to the end user to improve user experience and save backhaul resources(cp. [2], clause 5.37).

……

5.1.2.3	Network Capability Exposure

The 3GPP system shall be able to support the existing network capability exposure.

The 3GPP System shall be able to support APIs exposure of 3GPP network information and capabilities to 3rd parties, such as the network slicing capability, network status information).

NOTE:	The specific network slicing capability which is needed to be exposed should be defined after the slicing definition is completed.