3GPP TSG-SA WG1 Meeting #73	S1-160405

Okinawa, Japan, 1-5 February 2016	(revision of S1-160404, 104, 067, 090)

Title:	Proposed text for Overview section for V2X TS

Agenda Item:	7.4

Source:	Huawei, CATT, Nokia Networks, Alcatel Lucent, LGE

Contact:	Yi Shi		wn.shiyi@huawei.com

Feng Li		lifeng@catt.cn

Juergen Merkel 	juergen.merkel(at)nokia.com

Betsy Covell 	betsy.covell@alcatel-lucent.com

SungDuck Chun  sungduck (dot) chun (at) lge (dot) com

Abstract: This contribution proposes the text for Overview section of TS 22.cde v0.1.0 on Normative Work for V2X (V2XLTE).

- Introduction

In[1],the V2V,V2I,V2N and V2P potentail requirements are discussed.This document proposes to introduce V2V,V2I,V2N and V2P into overview section in [2].

- References

[1] 3GPP TR 22.885 v14.0.0.

[2] S1-154608 3GPP TS 22.cde V0.1.0 (2015-11)

Start of Change 1

4	Overview on V2X (informative)

4.1	Types of V2X application support in 3GPP

4.1.1	General

The V2X applications in the present specification, referred to as Vehicle-to-Everything (V2X), contain the following four different types:

-	Vehicle-to-Vehicle (V2V)

-	Vehicle-to-Infrastructure (V2I)

-	Vehicle-to-Network (V2N)

-	Vehicle-to-Pedestrian (V2P)

Figure 4.1-1: Types of V2X applications (V2V, V2P, V2N and V2I)

These four types of V2X applications can use “co-operative awareness” to provide more intelligent services for end-users. This means that entities, such as vehicles, roadside infrastructure, application server and pedestrians, can collect knowledge of their local environment (e.g., information received from other vehicles or sensor equipment in proximity) to process and share that knowledge in order to provide more intelligent services, such as cooperative collision warning or autonomous driving.

These intelligent transportation services and the associated message sets have been defined in automotive SDOs outside 3GPP. Three basic classes of applications for providing ITS services: road safety, traffic efficiency, and other applications can be found in e.g., [3].

3GPP only handles the transport of these messages to support different types of V2X applications. The message transport expectations are described in requirements defined in this specification.

4.1.2	Vehicle-to-Vehicle (V2V) application

V2V applications expect UEs that are in proximity of each other to exchange V2V application information. 3GPP transport of messages containing V2V application information requires the UE to have a valid subscription and authorization from a network operator. Transport for a valid subscriber is provided whether the UE is served or not served by E-UTRAN.

The UE supporting V2V applications transmits messages containing V2V application information (e.g. location, dynamics, and attributes). The message payloads may be flexible in order to accommodate varying amount of information.

3GPP transport of message containing V2V application information is predominantly broadcast-based as illustrated in Figure 4.1-2. Such 3GPP transport includes the transport between UEs directly and/or, due to the limited direct communication range, the transport between UEs via infrastructure supporting V2X communication, e.g., RSU, application server, etc.

# Figure 4.1-2: Broadcast-based V2V communications

4.1.3	Vehicle-to-Infrastructure (V2I) application

The UE supporting V2I applications transmits messages containing V2I application information to an RSU. An RSU transmits messages containing V2I application information to one or more UEs supporting V2I applications.

4.1.4	Vehicle-to-Network (V2N) application

The UE supporting V2N applications communicates with an application server supporting V2N applications. Both parties communicate with each other via EPS. The application server and its location are out of 3GPP scope.

4.1.5	Vehicle-to-Pedestrian (V2P) application

V2P applications expect UEs that are in proximity of each other to exchange V2P application information.  3GPP transport of messages containing V2P application information requires the UE to have a valid subscription and authorization from a network operator. Transport for a valid subscriber is provided whether the UE is served or not served by E-UTRAN.

The UE supporting V2P applications transmits messages containing V2P application information. It is expected that V2P application information can be transmitted either by a UE supporting V2X application in a vehicle (e.g., warning to pedestrian), or by a UE supporting V2X application associated with a vulnerable road user (e.g., warning to vehicle).

3GPP transport of messages containing V2P application information includes the transport between UEs directly and/or, due to the limited direct communication range, the transport between UEs via infrastructure supporting V2X communication, e.g., RSU, application server, etc.

The main difference between 3GPP transport of messages with V2P and V2V application information lies in the properties of the UE. A UE supporting V2P applications used by pedestrian might, for example, have lower battery capacity, the radio sensitivity might be limited, e.g. due to antenna design.

4.2	3GPP support for V2X with other features

Placeholder for potential informative text to be added to improve readability.

End of Change 1