3GPP TSG-SA WG1 Meeting #73	S1-160534

Okinawa, Japan, 1-5 February 2016	(revision of S1-160366)

Title:	CriC: Local UAV Collaboration and Connectivity

Agenda Item:	8.3

Source:	Qualcomm Incorporated

Contact:	Jack Nasielski (jackn at qti dot qualcomm dot com)

Abstract: Text for SMARTER TR for Critical Communications

# Proposal

This document proposes text to fill in the skeleton of the Critical Communications (CriC) TR.  Specifically it provides text for the “Higher reliability and lower latency” section.  It supports uses cases 5.12 and 5.54 in the initial SMARTER TR.

--- START MODIFIED CLAUSE ---

## 5.1	Higher reliability and lower latency

### 5.1.1	Description

### 5.1.2	Traffic scenarios

### 5.1.2.x	Traffic scenario - Local UAV Collaboration and Connectivity

Unmanned Aerial Vehicles (UAVs) can collaborate to act as a mobile sensor and actuator network to execute tasks in uncertain and dynamic environments while being controlled by a single user. Accuracy in sensing tasks is increased when deploying a team of UAVs versus just one as there are multiple vantage points using multiple sensors. Examples of uses for deploying a team of UAVs include:

- Searching for an intruder or suspect

- Continual monitoring of natural disasters

- Performing autonomous mapping

- Collaborative manipulation of an object (e.g. picking up corners of a net.)

Figure 5.x.x.x depicts how communication occurs in UAV local vehicle collaboration and connectivity. Both node to node and UAV to mobile network links are required.

Figure 5.x.x.x: Communication Path

--- END MODIFIED CLAUSE ---

--- START MODIFIED CLAUSE ---

### 5.1.3	Potential requirements

For UAV collaboration, the 3GPP system shall support:

- One-way latency of [5 ms] on the radio layer for direct inter-UAV links.

For UAV connectivity, the 3GPP system shall support:

- Continuous wireless coverage for UAV flying at low altitude of [10-1000] meters with maximum speed of [200km/h.]

- The 3GPP system shall be able to provide at least message integrity protection. Other security requirements are FFS.

- Priority, Precedence, Preemption (PPP) as failure to transmit communications in reliable and timely manner could result in injury or loss of property.

--- END MODIFIED CLAUSE ---