3GPP TSG-SA WG1 Meeting #74	S1-161359

Venice, Italia, 9-13 May 2016	(revision of S1-161183)

Title:	FS_MIOT Relay UE

Agenda Item:	9.2

Source:				Nokia

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes to remove the editor’s note on Relay UEs by clarifying the distinction between a device and a UE.

First Proposed Text Change:

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3GPP connection: The connection between a device and a 3GPP network

Active communication: a UE is in active communication when it has one or more connections established. A UE may have any combination of CS connections or PS connections (e.g., PDP contexts, active PDN connections).

Device: An entity with a wireless connection, direct and/or indirect, to the 3GPP network

Direct 3GPP connection: One mode of 3GPP connection, where there is no relay UE between the device and the 3GPP network

Direct device connection: The connection between two devices without any network entity in the middle

Indirect 3GPP connection: One mode of 3GPP connection, where there is a relay UE between the device and the 3GPP network. Indirect 3GPP connection is established using at least one direct device connection and at least one direct 3GPP connection.

Relay UE: a UE that is capable of relaying communications to and from the network for other devices.

Service continuity: The uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 1: 	In particular Service Continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the change.

NOTE 2: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct 3GPP connection and an indirect 3GPP connection.