3GPP TSG-SA WG1 Meeting #74	S1-161599

Venice, Italia, 9-13 May 2016	(revision of S1-161355)

(merger of S1-161110 and S1-161200)

Title:	FS_NEO Consolidation of Potential requirements

Agenda Item:	9.5

Source:				Nokia, Huawei

Contact:			Zhiming Li Lizhiming@huawei.com

Betsy Covell betsy.covell@nokia.com

Abstract: This document provides PR numbers to all requirements in TR 22.864 and proposes to consolidate them in clause 7.

First Proposed Text Change:

#### 5.1.2.1	Network Slicing

Editor’s Note: The following requirements are copied from [2], clause 5.2.2:

[PR 5.1.2.1-001] The 3GPP System shall allow the operator to compose network slices, i.e. sets of network functions (e.g. potentially from different vendors), resources to run these network functions and policies and configurations, e.g. for hosting multiple enterprises or MVNOs etc.

[PR 5.1.2.1-002] The 3GPP system shall allow the operator to dynamically create network slice to form a complete, autonomous and fully operational network customised to cater for different diverse market scenarios.

[PR 5.1.2.1-003] The 3GPP System shall be able to identify certain terminals and subscribers to be associated with a particular network slice.

Editor’s Note: The following requirements are agreed in meeting #72.

[PR 5.1.2.1-004] The 3GPP System shall enable a UE to simultaneously obtain services from one or more specific network slices of one operator e.g. based on subscription or terminal type.

[PR 5.1.2.1-005] The 3GPP system shall allow the operator to operate and manage network slices that fulfil required criteria for different market scenarios.

[PR 5.1.2.1-006] The 3GPP system shall allow the operator to operate different network slices in parallel with isolation that e.g. prevents data communication in one slice to negatively impact services offered by other slices.

[PR 5.1.2.1-007] The 3GPP System shall have the capability to conform to service-specific security assurance requirements in a single network slice, rather than the whole network.

[PR 5.1.2.1-008] The 3GPP System shall have the capability to provide a level of isolation between network slices which confines a potential cyber-attack to a single network slice.

[PR 5.1.2.1-009] The 3GPP system shall allow the operator to authorize third parties to create, manage a network slice configuration (e.g. scale slices) via suitable APIs, within the limits set by the network operator.

[PR 5.1.2.1-010] The 3GPP system shall support elasticity of network slice in term of capacity with no negative impact on the services of this slice or other slices.

[PR 5.1.2.1-011] The 3GPP system shall be able to change the slices with minimal impact on the ongoing subscriber’s services served by other slices, i.e. new network slice addition, removal of existing network slice, or update of network slice functions or configuration.

[PR 5.1.2.1-012] The 3GPP System shall be able to support E2E (e.g. RAN, CN) resource management for a network slice.

Editor’s Note: The following requirements are agreed in meeting #72.

[PR 5.1.2.1-013] The 3GPP system shall enable operators to use the network slicing concept to efficiently support multiple 3rd parties (e.g. enterprises) that require similar network characteristics.

Editor’s Note: The following requirements are copied from [2], clause 5.69.3:

[PR 5.1.2.1-014] The 3GPP system shall enable operators to define and identify network slices with common functionality to be available for home and roaming users.

[PR 5.1.2.1-015] The 3GPP system shall enable operators to specify the network functionalities that a network slice is required to provide.

[PR 5.1.2.1-016] The 3GPP system shall support the inclusion of 3GPP defined functions as well as proprietary 3rd party or operator provided functions in a network slice, including

- Hosting multiple 3rd parties (e.g.,enterprises) or MVNOs.

- Serving home and roaming user

- Supporting diverse market scenarios

NOTE:	The specific functional areas for which the system should support proprietary or operator provided functions should be identified.

[PR 5.1.2.1-017] The 3GPP system shall support to associate the user in a VPLMN to the network slice that provides the required functionality for this user (e.g. the same functionality as the associated network slice in the HPLMN). If no corresponding slice has been defined the user should be assigned to a default network slice, as defined by the VPLMN.

[PR 5.1.2.1-018] The 3GPP system shall be able to change the network slices to which UEs are connected.

[PR 5.1.2.1-019] A network slice shall support a set of end-user services as defined by the network operator.

[PR 5.1.2.1-020] The 3GPP system shall enable the operator to assign a UE to a network slice based on services provided by the network slice.

[PR 5.1.2.1-021] The 3GPP system shall support a mechanism for an operator to authorize a UE to receive service on a specific slice in response to a request from a 3rd party.

#### 5.1.2.2	Efficient User Plane

Editor’s Note: The following requirements are copied from [2], clause 5.8.3:

[PR.5.1.2.2-001] Subject to operator’s policy and/or based on application needs, the 3GPP network shall support efficient user-plane paths between UEs attached to the same network, even if the UEs change their location during communication.

[PR.5.1.2.2-002] Subject to operator’s policy and/or based on application needs, the 3GPP network shall support efficient user-plane paths between a UE attached to the mobile network and communication peers outside of the mobile network (e.g. Internet hosts).

Editor’s Note: The following requirements are copied from [2], clause 5.37.2:

[PR.5.1.2.2-003] Subject to the service agreement between the operator and the service provider, the 3GPP network shall enable hosting of services (including both MNO provided services and 3rd party provided services) closer to the end user to improve user experience and save backhaul resources.

[PR.5.1.2.2-004] The 3GPP network shall be able to support routing of data traffic to the entity hosting services closer to the end user for specific services of a UE.

[PR.5.1.2.2-005] The 3GPP network shall support efficient user-plane paths between a UE and the entity hosting the service closer to the end user even if the UE changes its location during communication.

[PR.5.1.2.2-006] The 3GPP network shall be able to support charging, QoS, and Lawful Interception (LI) for services hosted closer to the end user.

[PR.5.1.2.2-007] The 3GPP System shall be efficient and flexible for both low throughput short data bursts and high throughput data transmissions (e.g., streaming video) from the same device.

[PR.5.1.2.2-008] The 3GPP network shall support efficient user-plane paths between a UE and the entity hosting the service closer to the end user.

[PR.5.1.2.2-009] Subject to the service agreement between the operator and the service provider, the 3GPP network shall support changing user-plane paths between a UE and entities hosting a service during an active communication (e.g., when a UE’s location changes).

[PR.5.1.2.2-010] The 3GPP network shall maintain user experience when the UE changes the connection to a different entity hosting the service.

[PR.5.1.2.2-011] The 3GPP network shall be able to interact with the entity hosting the service for network optimization.

#### 5.1.2.3	Network Capability Exposure

[PR.5.1.2.3-001] The 3GPP system shall be able to support the existing network capability exposure.

[PR.5.1.2.3-002] The 3GPP System shall be able to support exposure of 3GPP network information and capabilities to 3rd parties, such as the network slicing capability, network status information.

NOTE:	The specific network slicing capability which is needed to be exposed should be defined after the slicing definition is completed.

#### 5.1.2.4	Flexible Broadcast Service

[PR.5.1.2.4-001] The 3GPP system shall enable deployment of a stand-alone 3GPP based broadcast/multicast system over a wide geographic area in a spectrally efficient manner with no interruption in delivery of the content to the mobile users.

[PR.5.1.2.4-002] The 3GPP system shall enable the operator to have the flexibility to allocate 0% to 100% of radio resources of a radio carrier for the delivery of broadcast/multicast content.

[PR.5.1.2.4-003] Depending on the capability of the terminal and the services subscribed, the 3GPP system shall allow the user to receive the broadcast/multicast content via the broadcast/multicast radio carrier while a concurrent data session is going on over another radio carrier.

[PR.5.1.2.4-004] The 3GPP based broadcast/multicast system shall efficiently utilize the available resources to maximize the amount of content that can be delivered over a stand-alone multicast/broadcast network with individual cell coverage radius of up to 50 km.

Editor’s Note: The following requirements are copied from [2], clause 5.56.3:

[PR.5.1.2.4-005] The 3GPP system shall provide a mechanism to expose broadcasting capabilities to external Broadcasters’ management systems.

[PR.5.1.2.4-006] The 3GPP system shall be able to reserve groups of resources for Broadcast channels

[PR.5.1.2.4-007] The 3GPP system shall be able to support broadcast of video streams such as 4k/8k UHD.

[PR.5.1.2.4-008] The 3GPP device shall be able elect to receive a reduced quality version of the broadcast for display on their device screen (typically less than 12”) or a full quality version of the same channel for presentation  to a video presentation monitor (typically much larger than their device , ie:32” to 72”screen size)

[PR.5.1.2.4-009] The 3GPP System shall allow the UE to receive broadcasts selected by the user (from the Broadcaster’s management system) in accordance with any appropriate authorisations by the Broadcaster.

NOTE:	4k UHD: 3840 x 2160, 50FPS, HEVC, 20~30Mbit/s (Medium quality), 4k UHD: 3840 x 2160, 50FPS, HEVC, ~75Mbit/s (High quality), 4kUHD: 3840 x 2160, 50FPS, AVC ~ 150Mbit/s (High quality), 8K 7680*4320, 50FPS, HEVC, ~300Mbps (High quality).

Editor’s Note: The following requirements are copied from [2], clause 5.57.3:

[PR.5.1.2.4-010] The 3GPP system shall provide a mechanism to expose broadcasting capabilities to an external Ad-Hoc Broadcast management system that manages broadcast requests.

[PR.5.1.2.4-011] The 3GPP system shall be able to reserve groups of resources temporarily for scheduled Ad-Hoc Broadcasts.

[PR.5.1.2.4-012] The 3GPP System shall allow the UE to receive broadcasts selected by the user (from the Ad-Hoc Broadcast management system) in accordance with any appropriate authorisations.

[PR.5.1.2.4-013] The 3GPP system shall allow the MNO to implement a network supporting downlink only broadcasting based on a set of 5G functionalities and entities which are required for content delivery.

#### 5.1.2.5	Multi-network Connectivity and Service Delivery across Operators

[PR.5.1.2.5-001] The 3GPP system shall enable the operators to deploy networks for a specific set of services.

[PR.5.1.2.5-002] The 3GPP system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

[PR.5.1.2.5-003] For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of a single operator, the home operator.

[PR.5.1.2.5-004] In the event of the same service being offered by multiple operators, unless directed by the home 3GPP system, the user shall be prioritized to receive subscribed services from the home operator.

NOTE:	If the service is unavailable (e.g., due to lack of network coverage) from the home network, the user shall be able to receive the service from another operator’s network.

#### 5.1.2.6	Markets requiring minimal service levels

[PR.5.1.2.6-001] The following requirements apply for markets requiring minimal service levels.

[PR.5.1.2.6-002] The 3GPP system shall support very efficient use of the control plane (e.g., minimizing signalling overhead, limiting mobility support).

[PR.5.1.2.6-003] The 3GPP system shall support efficient use of the data plane (e.g., minimizing power consumption, consolidating data transmissions to 1 larger rather than many smaller).

[PR.5.1.2.6-004] The 3GPP system shall be able to expose network status information to applications (e.g., to allow applications to use network resources efficiently).

[PR.5.1.2.6-005] The 3GPP system shall minimise the amount of wireless backhaul traffic (e.g., consolidating data transmissions to 1 larger rather than many smaller, when applicable).

[PR.5.1.2.6-006] The 3GPP system shall be optimised to provide a minimal user experience (e.g. user experienced data rate of 100kbps, E2E latency of 50 ms).

[PR.5.1.2.6-007] The 3GPP system shall be able to support very large coverage areas (e.g., relaxed timing on random access and other procedures to enable very long range beyond 100km).

[PR.5.1.2.6-008] The base station shall support an energy saving mode with the following characteristics:

- The energy saving mode can be activated/deactivated either manually or automatically.

- Service can be restricted to a pre-defined set of users (e.g., public safety, emergency callers).

NOTE:	when in energy saving mode the transmit power may be reduced or turned off (deep sleep mode), latency and jitter may be increased.

[PR.5.1.2.6-009] The 3GPP system shall be able to adapt power consumption to fluctuations in power supply.

[PR.5.1.2.6-010] The 3GPP system shall support operations that minimize UE power consumption (e.g., minimize push information, adjust transmission power).

[PR.5.1.2.6-011] The 3GPP system shall support centralized network automation and management in order to reduce local management tasks.

[PR.5.1.2.6-012] The data rate transfer should be reduced at the cell edge for very large coverage area (e.g., 100 kbps for more than 100km cell coverage, 1Mbs for 100km cell coverage).

[PR 5.1.2.6-013] The 3GPP system shall be able to give priority to critical services (e.g. e-health) when resources are limited.

#### 5.1.2.7	Priority, QoS and Policy Control

[PR 5.1.2.7-001] The 3GPP system shall allow flexible means to make and enforce relative priority decisions among the different application services (e.g., MPS, Emergency, medical).

NOTE 1:	Relative priority between different application services is subject to regional or national regulatory and operator policies.

[PR 5.1.2.7-002] The 3GPP system shall be able to provide the required QoS (e.g., reliability, latency, bandwidth) for an application and support prioritization of resources when necessary for that application.

[PR 5.1.2.7-003] The 3GPP system shall be able to support a QoS and policy framework for multiple accesses.

[PR 5.1.2.7-004] The 3GPP system shall be able to support E2E (e.g., UE to UE) QoS for a service.

NOTE 2:	E2E QoS needs to consider QoS in RAN, backhaul, core network, and backbone.

### 5.2.2	Potential Requirements

Editor’s Note: The following requirements are taken from sections 5.51, 5.7, 5.9, 5.35 respectively of TR 22.891 with updates/clarifications.

System elasticity

[PR.5.2.2-001] The 3GPP system shall be able to adjust the network capacities dynamically based on the variation of demand.

Editor’s Note: the requirement in section 5.9 of TR 22.891 “The 3GPP system shall be scalable to ensure that different levels of signalling and user plane demand can be handled” is contained.

[PR.5.2.2-002] The 3GPP system shall support dynamic utilization of resources (compute, network and storage resources) in more than one geographic area in order to serve the differing needs of the users in each geographic area, subject to operator policy.

[PR.5.2.2-003] Use of resources (compute, network and storage resources) in more than one geographic area by the system shall be supported without requiring manual re-configuration of neighbouring nodes, without service disruption, and while avoiding additional signalling due to unnecessary UE’s re-attachments (e.g. due to loss of call state information in the network).

[PR.5.2.2-004] The 3GPP system shall be able to provide an increase in signalling and user plane capacity within 5 minutes of the need to do so being detected.

NOTE:	The lead time of 5 minutes stems from the most severe and unplanned use case i.e. the disaster use case, where the average time until call attempts surge after a disaster occurs is considered 5 minutes.

[PR.5.2.2-005] The 3GPP system shall be able to maintain service when performing a network scaling and automation operation.

[PR.5.2.2-006] The 3GPP system shall be able to support the enhancement on the existing mechanisms, e.g. load balancing, network function selection, which are highly related to the network scalability and automation operation.

System information collection

[PR.5.2.2-007] The 3GPP system shall support elastic configuration of the network based on system information, including:

- Instantaneous network conditions, such as serving RATs (macro cell, small cell, WiFi), network load information and congestion levels;

- Application’s user characteristics, such as mobility type (high mobility, low mobility, no mobility), expected traffic over time, location;

- When allowed by a user, UE context information, such as sensor-level information (e.g. direction, speed, power status, display status, other sensor information installed in the UE), application-level information (e.g. foreground applications, running background application, application data, user settings, etc.)

[PR.5.2.2-008] The 3GPP system shall support a secure mechanism to collect system information while ensuring end-user and application privacy e.g. application level information such as application usage information is not to be related to an individual application user identity or subscriber identity and UE level information such as UE location is not be related to an individual subscriber identity.

[PR.5.2.2-009] The 3GPP system shall support a mechanism to collect the information in a timely manner to enable optimized network elasticity regarding resource use based on accurate context information.

### 5.3.2	Potential Requirements

[PR.5.3.2-001] The 3GPP system shall enable operators to define different levels of mobility support for different UEs.

[PR.5.3.2-002] Mobility support consists of providing none, any one or some combination of the following (subject to further consideration regarding complexity by Stage 2):

-	minimizing packet loss during inter- and/or intra-RAT cell changes for some or all packet data connections (e.g. APNs) of a UE,

-	maintaining the same IP address assigned to a UE across different cells and RATs for some or all packet data connections (e.g. APNs) of a UE,

-	minimizing impact to the user experience (e.g. minimization of interruption time) when changing the IP address and IP anchoring point for some or all packet data connections (e.g. APNs) of a UE

[PR.5.3.2-003] The 3GPP system shall support service continuity when changing the IP address and/or IP anchoring point for some or all packet data connections associated with a UE.

[PR.5.3.2-004] The 3GPP system shall enable operators to update the level of mobility support provided for a UE, e.g. during packet data connection establishment (subject to further consideration regarding complexity by Stage 2).

[PR.5.3.2-005] The 3GPP system shall minimize the signaling needed for mobility management of stationary UEs.

[PR.5.3.2-006] The 3GPP system shall minimize the signaling needed for mobility management of UEs that participate in mobile-originated communication scenarios only.

### 5.4.2	Potential Requirements

Editor’s Note: The following requirements are from [2], clause 5.36.2:

[PR.5.4.2-001] The 3GPP system shall enable efficient delivery of content from an appropriate content caching entity under the control of the operator, e.g. a cache located close to the user.

[PR.5.4.2-002] The 3GPP system shall provide charging, Lawful Interception (LI) and QoS differentiation for content delivered from an in-network caching entity.

[PR.5.4.2-003] The 3GPP system shall enable a flexible deployment of content caching entity located at multiple locations within the network (e.g. at various radio sites and local aggregation points).

[PR.5.4.2-004] The 3GPP system shall support a content caching entity that is capable of being integrated within a device under the control of the operator.

[PR.5.4.2-005] The authorized UE shall be able to receive cached content broadcasted by content caching entity.

Editor’s Note: The following requirements are agreed in meeting #72:

[PR.5.4.2-006] The 3GPP system shall provide the means for caching of encrypted contents (e.g. using HTTPS) with at least similar performance benefits to those obtained for unencrypted contents.

Editor’s Note: The following requirements are from [2], clause 5.39.3:

[PR.5.4.2-007] The 3GPP system shall support the ability to securely store the personal data /files of a user in such a way that they are retrievable with no perceptible delay to the user.

Editor’s Note: The above requirement needs to be clarified.

[PR.5.4.2-008] The 3GPP System shall support a mechanism to control the upload and download of personal data/files between the 3GPP device and a appropriate storage entity in the network.

Editor’s Note: The above requirement needs to be clarified.

Editor’s Note: The following requirements are from [2], clause 5.38.3:

[PR.5.4.2-009] The 3GPP system shall efficiently support protocols for in-network caching and storage.

### 5.5.2	Potential Requirements

[PR.5.5.2-001] Radio Interface Technology (RIT) of the 3GPP system shall be designed with features and optimizations to provide a backhaul function.

[PR.5.5.2-002] The 3GPP system shall support flexible and efficient backhaul for both indoor and outdoor access nodes.

[PR.5.5.2-003] The 3GPP system shall support flexible partitioning of resources between access and backhaul functions when supported in a common band.

[PR.5.5.2-004] The 3GPP system shall support dynamic, autonomous configuration of integrated access and backhaul nodes.

[PR.5.5.2-005] The 3GPP system shall support dynamic adaptation on wireless backhaul network topologies to minimize service disruptions.

[PR.5.5.2-006] The 3GPP system shall allow a network operator to provide fronthaul/backhaul network sharing information and capabilities to other network operators.

[PR.5.5.2-007] The 3GPP system shall allow network operators to be able to share fronthaul/backhaul network resources.

### 5.6.2	Potential Requirements

Editor’s Note: The following requirements are taken from sections 5.28, 5.26, 5.27, 5.23, 5.31, & 5.58 respectively of TR 22.891 with minor updates/clarification (e.g. replaced “future network” with “5G system”).

- Multiple Access Support

Editor’s Note: The following requirements are from [2], clause 5.28.3:

[PR.5.6.2-001] The 5G system shall be able to provide data transmission by using both the 5G RATs and E-UTRA RAT simultaneously, i.e. in this case dual radio UE capability is required.

[PR.5.6.2-002] When a UE is using both the 5G RAT and E-UTRA RAT simultaneously, the 5G system shall be able to select a radio access (either a 5G RAT or E-UTRA RAT) to assign each data flow, taking into account e.g. service, traffic characteristics, radio characteristics, and UE’s moving speed.

[PR.5.6.2-003] The 5G system shall support UEs with dual radio capability as well as UEs with single radio capability, i.e. a UE that cannot transmit on the 5G RAT and the E-UTRA RAT simultaneously.

- Optimized Connection per Traffic Type

Editor’s Note: The following requirements are copied from [2], clause 5.26.3:

[PR.5.6.2-004] Based on operator policy, the 3GPP network shall provide a mechanism such that a specific traffic type (from a specific application or service) to/from a UE can be routed via specific RAN nodes, and traffic in one RAN node can be offloaded towards a defined IP network close to the UE's point of attachment to the access network, while other traffic type to/from that same UE is not offloaded.

- 3GPP/Non-3GPP Access Integration

Editor’s Note: The following requirements are copied from [2], clause 5.27.2:

[PR.5.6.2-005] The 5G system shall be able to provide 3GPP services to the UE using various 3GPP and non-3GPP access networks (e.g. WLAN, Fixed broadband access, Bluetooth, etc).

[PR.5.6.2-006]  The 5G system shall be able to support:

- Inter-system mobility between 3GPP and non-3GPP access networks,

- Capability for the UE to select the 3GPP or non-3GPP access based on operator policy,

- Simultaneous connection to different accesses, & capability for the UE to access the 3GPP services provided by the 5G network using non-3GPP access e.g. FMSS…,

- Authentication to access 5 G network through a non-3GPP access using 3GPP credentials.

Editor’s Note: this list is not exhaustive.

[PR.5.6.2-007] The 5G system shall enable the UE to simultaneously connect to the network via different accesses, both non-3GPP and 3GPP.

[PR.5.6.2-008] The 5G system shall be able to manage the addition or dropping of the various accesses dynamically during the session according to the quality of the individual connections.

[PR.5.6.2-009] For UEs simultaneously connected to the network via different access (both non-3GPP and 3GPP), the network shall support data transmissions that leverage these accesses.

[PR.5.6.2-010] The 5G system shall be able to aggregate the UE data transfer via one access or a combination of accesses.

[PR.5.6.2-011] The 5G system shall be able to aggregate the UE data transfer via one access or a combination of accesses to provide the best user experience during an ongoing data transmission.

[PR.5.6.2-012] The 5G system shall be able to support the interworking with fixed broadband network.

NOTE:	The specification of fixed broadband access network is outside the scope of 3GPP.Which evolution of fixed broadband access network architecture needs to be considered in stage 2.

[PR.5.6.2-013] The 5G system shall be able to differentiate charging a subscriber for the same 3GPP service if it is provided to a UE over different 3GPP and non-3GPP accesses.

[PR.5.6.2-014] Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic from 3GPP RAT to non-3GPP RATs and vice versa, taking into account traffic load.

Editor’s Note: The following requirements are copied from [2], clause 5.27.3:

[PR.5.6.2-015] The 3GPP network shall be able to integrate fixed and wireless access management and provide an efficient provision of services over 3GPP and non-3GPP accesses.

- Access from less trusted networks

Editor’s Note: The following requirements are copied from [2], clause 5.23.5:

[PR.5.6.2-016] Subject to regulatory requirements, the UE may use a temporary identifier for initial attach that hides its long-term identity. The home network shall be able to associate this temporary identifier to the long-term identity.

- Enhanced access control

Editor’s Note: The following requirements are covered from [2], clause 5.3.3 and clause 5.31.6:

[PR.5.6.2-017] 3GPP system shall be able to support enhanced access control mechanism with e.g. the following conditions:

- target subscribers (e.g. per PLMN),

- subscriber class (e.g. access class),

- device type (e.g. Smart phone or IoT device),

- service type (e.g. Voice, SMS and/or specific data applications),

- communication type (e.g. emergency call, signalling and/or service origination).

[PR.5.6.2-018] The enhanced access control mechanism shall be able to provide access to a limited set of services determined by an operator’s policy.

- Temporary Service for Users of Other Operators

Editor’s Note: The following requirements are covered from [2], clause 5.31.6:

[PR.5.6.2-019] Subject to regulatory requirements, 3GPP system shall be able to support temporary service in emergency case which has the following features:

- service authorization for users of other than home operators by serving operator policy,

- access security for temporary service in emergency case (see section 5.8.1.4).

- Energy Efficiency

Editor’s Note: The following requirements are copied from [2], clause 5.58.2:

[PR.5.6.2-020] The 3GPP system shall be capable of achieving [1000] times energy efficiency compared to legacy system.

- Satellite Access

[PR.5.6.2-021] The 3GPP system shall be able to provide services to the UE using satellite access.

[PR 5.6.2-022]The 3GPP system shall support an air-interface with one-way latency of up to 275 ms when satellite connection is involved.

[PR.5.6.2-023] The 3GPP system shall support service continuity between land based and satellite based networks.

- Residential Access

Editor’s Note: The following requirements are adapted from [2], clause 5.71:

[PR.5.6.2-024] The 3GPP system shall support residential deployment with a service limited to a pre-defined geographic area.

[PR.5.6.2-025] The 3GPP system shall support residential deployment with peak data rates of [10s of Gbps] and experienced data rates of up to [1 Gbps].

[PR.5.6.2-026] The 3GPP system shall support residential deployment with a latency of [10 ms].

#### 5.7.2.1	Interworking between <5G> systems

[PR.5.7.2.1-001] The <5G> System shall provide support for a UE with a <5G> subscription roaming into a <5G> Visited Mobile Network which has a roaming agreement with the UE's Home Mobile Network. A <5G> Visited Mobile Network shall provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

[PR.5.7.2.1-002] The <5G system> shall provide a mechanism for a network operator to limit access to its services for a roaming subscriber with a <5G> UE and subscription, e.g., based on a specific service logic agreement in a roaming agreement.

#### 5.7.2.2	Interworking between 5G and 4G

[PR.5.7.2.2-001] The <5G system> shall be able to support seamless handover and Inter System Mobility between<5G> RAT(s) and E-UTRA.

#### 5.7.2.3	Migration aspects

[PR.5.7.2.3-001] A <5G> System shall support all services supported in a 4G system, with the following exceptions:

CS voice service continuity and/or fallback to GERAN or UTRAN

Seamless handover between the <5G> RAT(s) and GERAN or UTRAN

Access by a GERAN or UTRAN RAT to a 5G core network

...

Editor's Note: this list may not be complete.

#### 5.8.2.1	System Security

[PR.5.8.2.1-001] Subject to regulatory requirements, the <5G system> shall support a secure mechanism allowing an authorized entity to disable from normal operation of a device reported as stolen.

[PR.5.8.2.1-002] Subject to regulatory requirements, the <5G system> shall support a secure mechanism allowing an authorized entity to re-enable a recovered stolen device to normal operation.

[PR.5.8.2.1-003] The 3GPP system shall be able to protect subscriber identity and other user identifying information from attacks by a passive listener.

[PR.5.8.2.1-004] The 3GPP system shall be able to protect subscriber identity and other user identifying information from active attacks.

[PR.5.8.2.1-005] The 3GPP system shall be able to protect user location information from attacks by a passive listener.

[PR.5.8.2.1-006] The 3GPP system shall be able to protect user location information from active attacks.

[PR.5.8.2.1-007] Subject to regulations, the 3GPP system shall be able to protect the user identity from an attack by a roamed-to network.

[PR.5.8.2.1-008] The 3GPP system shall minimize signalling overhead for security.

[PR.5.8.2.1-009] Subject to regulatory requirements, 3GPP system shall be able to provide temporary service in an operator’s network in emergency case for users from other operators without access to their home network (e.g. IOPS for commercial use, emergency services enhanced for temporary service in emergency case and so on).

[PR.5.8.2.1-010] Subject to national regulatory requirements, the 5G 3GPP System shall support a mechanism to provide Lawful Interception. Existing Lawful Interception are applicable without restrictions.

[PR.5.8.2.1-011] Subject to national requirements, the 5G 3GPP System shall comply with Lawful Interception Requirements.

[PR.5.8.2.1-012] The <5G System> shall provide appropriate mechanisms to enable Lawful Interception.

NOTE: The appropriate solution will be defined in 3GPP SA3_LI.

### 5.8.3	Potential Operational Requirements

[PR.5.8.3-001] 5G devices shall store device identities in a secure and tamper resistant way in order to prevent device identification tampering, when required.

[PR.5.8.3-002] 5G devices shall display the device identity in a consistent manner, not requiring detailed knowledge of the device's user interface, when required.

[PR.5.8.3-003] The 3GPP system shall be able to support identification of subscriptions independently of identification of devices.

Second Proposed Text Change:

# 7	Consolidation of Potential Requirements

## 7.1	Network Slicing

[PR 5.1.2.1-001] The 3GPP system shall allow the operator to compose and manage network slices. A network slice consists of a set of network functions (e.g., potentially from different vendors), the resources to run these network functions as well as policies and configurations.

[PR 5.1.2.1-002] The 3GPP system shall allow the operator to dynamically create a network slice to form a complete, autonomous and fully operational network customised for different market scenarios.

[PR 5.1.2.1-003] The 3GPP system shall be able to associate specific services, devices, UEs, and subscribers with a particular network slice.

[PR 5.1.2.1-004] The 3GPP system shall enable a UE to simultaneously access services from one or more network slices of one operator.

[PR 5.1.2.1-006] The 3GPP system shall allow the operator to simultaneously operate network slices in a manner that prevents a service in one slice from negatively impacting services offered by other slices.

[PR 5.1.2.1-010] The 3GPP system shall support the elasticity of a network slice in term of capacity with minimal impact on the services of this slice or other slices.

[PR 5.1.2.1-010b] The 3GPP system shall support modifications to network slices (e.g., adding, deleting, modifying network slices) with minimal impact to active subscriber services.

[PR 5.1.2.1-012] The 3GPP system shall be able to support E2E (e.g. RAN, core network) resource management for a network slice.

[PR 5.1.2.1-016] The 3GPP system shall support the inclusion of 3GPP defined functions as well as proprietary 3rd party or operator provided functions in a network slice, including

- Hosting multiple 3rd parties (e.g., enterprises) or MVNOs.

- Serving home and roaming user

- Supporting diverse market scenarios.

NOTE:	The specific functional areas for which the system should support proprietary or operator provided functions should be identified.

[PR 5.1.2.1-017] The 3GPP system shall support a mechanism for the VPLMN to assign the UE to a network slice with the needed functionality, or to a default network slice.

[PR 5.1.2.1-018] The 3GPP system shall be able to change the network slices to which UEs are connected.

[PR 5.1.2.1-019] A network slice shall support a set of end-user services as defined by the network operator.

[PR 5.1.2.1-020] The 3GPP system shall enable the operator to assign a UE to a network slice based on services provided by the network slice.

[PR 5.1.2.1-021] The 3GPP system shall support a mechanism for an operator to authorize a UE to receive service on a specific slice in response to a request from a 3rd party.

## 7.2	Efficient User Plane

[PR 5.1.2.2-001] Subject to operator’s policy and/or based on application needs, the 3GPP system shall support efficient user-plane paths between UEs attached to the same network, even if the UEs change their location during an active communication.

[PR 5.1.2.2-002] Subject to operator’s policy and/or based on application needs, the 3GPP system shall support efficient user-plane paths between a UE attached to the mobile network and communication peers outside of the mobile network (e.g., Internet hosts).

[PR 5.1.2.2-003] 3GPP network shall enable hosting of services (including both MNO provided services and 3rd party provided services) closer to the end user.

[PR 5.1.2.2-004] The 3GPP network shall be able to support routing of data traffic to the entity hosting services closer to the end user for specific services.

[PR 5.1.2.2-006] The 3GPP core network shall be able to support QoS for services hosted closer to the end user.

[PR 5.1.2.2-007] The 3GPP system shall be efficient and flexible for both low throughput short data bursts and high throughput data transmissions (e.g., streaming video) from the same device.

[PR.5.1.2.2-008] The 3GPP network shall support efficient user-plane paths between a UE and the entity hosting the service closer to the end user.

[PR.5.1.2.2-009] Subject to the service agreement between the operator and the service provider, the 3GPP network shall support changing user-plane paths between a UE and entities hosting a service during an active communication (e.g., when a UE’s location changes).

[PR.5.1.2.2-010] The 3GPP network shall maintain user experience when the UE changes the connection to a different entity hosting the service.

[PR.5.1.2.2-011] The 3GPP network shall be able to interact with the entity hosting the service for network optimization.

## 7.3	Network Capability Exposure

[PR 5.1.2.6-003] The 3GPP system shall be able to expose network status information to applications (e.g., to allow applications to use network resources efficiently).

NOTE:	Applications may be frugal at periods when resources are rare and should compensate and anticipate as much as they can otherwise. Applications may be able to handle disconnections in case of user mobility.

[PR 5.1.2.1-009] The 3GPP system shall allow a 3rd party to create, manage a network slice configuration (e.g. scale slices) via suitable APIs, within the limits set by the network operator.

[PR 5.1.2.4-005] The 3GPP system shall provide a mechanism to expose broadcasting capabilities to 3rd party broadcasters’ management systems.

## 7.4	Flexible Broadcast Service

[PR 5.1.2.4-001] The 3GPP system shall enable deployment of a stand-alone 3GPP-based broadcast/multicast system over a wide geographic area in a spectrally efficient manner.

[PR 5.1.2.4-002] The 3GPP system shall have the flexibility to enable the operator to reserve 0% to 100% of radio resources of a radio carrier for the delivery of broadcast/multicast content.

[PR.5.1.2.4-006] The 3GPP system shall be able to manage groups of resources for broadcast channels.

[PR 5.1.2.4-003] Depending on the capability of the device or UE and the services subscribed, the 3GPP system shall allow the user to receive content via a broadcast/multicast radio carrier while a concurrent data session is ongoing over another radio carrier.

[PR 5.1.2.4-007] The 3GPP system shall be able to support broadcast of streaming video (e.g., 4k/8k UHD).

NOTE:	taking into account the bandwidth needs for different streaming video resolution.

[PR 5.1.2.4-008] The 3GPP device shall be able to request a specific video quality for a received broadcast (e.g., reduced quality for display on a small screen or high quality for display on a larger screen).

[PR 5.1.2.4-004a] The 3GPP system shall allow the operator to deploy a stand-alone 3GPP based broadcast/multicast system with minimal interruption in the delivery of the content to the device or UE.

## 7.5	Multi-network Connectivity and Service Delivery across Operators

[PR 5.1.2.5-001] The 3GPP system shall enable the operators to deploy networks for a specific set of services.

[PR 5.1.2.5-002] The 3GPP system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

[PR 5.1.2.5-003] For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

[PR 5.1.2.5-004] In the event of the same service being offered by multiple operators, unless directed by the HPLMN, the device or UE shall be prioritized to receive subscribed services from the home operator.

NOTE:	If the service is unavailable (e.g., due to lack of network coverage) from the home network, the device or UE could be able to receive the service from another operator’s network.

## 7.6	Markets requiring minimal service levels

[PR 5.1.2.6-001] The 3GPP system shall support very efficient use of the control plane (e.g., minimizing signalling overhead, limiting mobility support).

[PR 5.1.2.6-002] The 3GPP system shall support efficient use of the data plane (e.g., minimizing power consumption, consolidating data transmissions to 1 larger rather than many smaller, adaptation of resource consumption based on the availability of limited resources (e.g. use of the appropriate layer/interface, or use several layers, possibly based on disjoint multiple paths, if more resources are available), local services).

[PR 5.1.2.6-004] The 3GPP system shall minimise the amount of wireless backhaul traffic (e.g., consolidating data transmissions to 1 larger rather than many smaller, when applicable).

[PR.5.1.2.6-006] The 3GPP system shall be able to support a minimal user experience for all (e.g., user experienced data rate of 100kbps, E2E latency of 50 ms, lower availability of 0,95).

[PR 5.1.2.6-008] The base station shall support an energy saving mode with the following characteristics:

- The energy saving mode can be activated/deactivated either manually or automatically.

- Service can be restricted to a pre-defined set of users (e.g., public safety, emergency callers).

NOTE:	when in energy saving mode the transmit power may be reduced or turned off (deep sleep mode), latency and jitter may be increased.

[PR 5.1.2.6-010] The 3GPP system shall support operations that minimize UE power consumption (e.g., minimize push information, adjust transmission power).

[PR 5.1.2.6-011] The 3GPP system shall support centralized network automation and management in order to reduce local management tasks.

[PR 5.1.2.6-013] The 3GPP system shall be able to give priority to critical services (e.g. e-health) when resources are limited.

## 7.7	System elasticity

[PR 5.2.2-001] The 3GPP system shall be able to adjust the network capacities dynamically based on variations in demand.

[PR 5.2.2-002] Subject to operator policy, the 3GPP system shall support dynamic utilization of resources (compute, network and storage resources) in more than one geographic area in order to serve the differing needs of the users in each geographic area.

[PR 5.2.2-004] The 3GPP system shall be able to provide an increase in signalling and user plane capacity within 5 minutes of the need to do so being detected.

NOTE:	The lead time of 5 minutes stems from the most severe and unplanned use case i.e., the disaster use case, where the average time until call attempts surge after a disaster occurs is considered 5 minutes.

[PR 5.2.2-005] The 3GPP system shall be able to maintain service when performing a network scaling and automation operation.

## 7.8	System information collection

[PR 5.2.2-007] The 3GPP system shall support elastic configuration of the network based on system information, including:

- Instantaneous network conditions, such as serving RATs (e.g., 5G, E-UTRA, WLAN), cell type (e.g., macro cell, small cell), network load information and congestion levels;

- Application’s user characteristics, such as mobility type (high mobility, low mobility, no mobility), expected traffic over time, location;

- When allowed by a user, UE context information, such as sensor-level information (e.g., direction, speed, power status, display status, other sensor information installed in the UE), application-level information (e.g., foreground applications, running background application, application data, user settings)

[PR 5.2.2-009] The 3GPP system shall support a mechanism to collect system information in a timely manner (e.g., network operation may be optimized based on contextual information).

## 7.9	Mobility support

[PR 5.3.2-001] The 3GPP system shall enable operators to define different levels of mobility support for different UEs.

[PR 5.3.2-002] An operator can choose to support none, any one, or some combination of the following:

•	The 3GPP system shall enable mobility support where packet loss is minimized during inter- and/or intra-RAT cell changes for some or all packet data connections (e.g. APNs) associated with a UE.

•	The 3GPP system shall enable mobility support where IP address assigned to a UE is maintained across different cells and RATs for some or all packet data connections (e.g. APNs) associated with a UE.

•	The 3GPP system shall enable mobility support where  the impact to the user experience is minimized (e.g. minimization of interruption time) when changing the IP address and IP anchoring point for some or all packet data connections (e.g. APNs) associated with a UE.

[PR 5.3.2-003] The 3GPP system shall support service continuity when changing the IP address and/or IP anchoring point for some or all packet data connections associated with a device or UE.

[PR 5.3.2-004] The 3GPP system shall enable operators to update the level of mobility support provided for a UE, (e.g., during packet data connection establishment).

[PR 5.3.2-005] The 3GPP system shall minimize the signaling needed for mobility management of stationary UEs.

[PR 5.3.2-006] The 3GPP system shall minimize the signaling needed for mobility management of UEs that participate in mobile-originated communication scenarios only.

[PR 5.3.2-007] The 3GPP system shall minimize the signaling needed for mobility management of UEs with known patterns of mobility (e.g., movement only within a limited geographic area).

## 7.10	Efficient content delivery

[PR 5.4.2-001] The 3GPP system shall enable efficient delivery of content from an appropriate content caching entity under the control of the operator, e.g., a cache located close to the device or UE.

[PR 5.4.2-002] The 3GPP system shall provide QoS differentiation for content delivered from an in-network caching entity.

[PR 5.4.2-003] The 3GPP system shall enable a flexible deployment of content caching entities located at multiple locations within the network (e.g., at various radio sites and local aggregation points).

[PR 5.4.2-004] The 3GPP system shall support a content caching entity that is capable of being integrated within a device/UE under the control of the operator.

[PR 5.4.2-005] A device or UE shall be able to receive cached content broadcasted by a content caching entity.

[PR 5.4.2-006] The 3GPP system shall provide the means for caching of encrypted contents (e.g., using HTTPS) with at least similar performance benefits to those obtained for unencrypted contents (e.g., in latency reduction, bandwidth savings, flexible caching).

## 7.11	Self backhaul

[PR 5.5.2-001] Radio Interface Technology (RIT) of the 3GPP system shall be designed with features and optimizations to provide a backhaul function.

[PR 5.5.2-002] The 3GPP system shall support flexible and efficient backhaul for both indoor and outdoor access nodes.

[PR 5.5.2-003] The 3GPP system shall support flexible partitioning of resources between access and backhaul functions when supported in a common band.

[PR 5.5.2-004] The 3GPP system shall support dynamic, autonomous configuration of integrated access and backhaul nodes.

[PR 5.5.2-005] The 3GPP system shall support dynamic adaptation on wireless backhaul network topologies to minimize service disruptions.

### 7.13	Support for multiple access technologies

### 7.13.1	LTE access support

[PR 5.6.2-001] The 3GPP system shall be able to provide data transmission by using both a 5G RAT and E-UTRA RAT simultaneously, i.e., in this case dual radio UE capability is required.

[PR 5.6.2-002] When a UE is using both the 5G RAT and E-UTRA RAT simultaneously, the 3GPP system shall be able to select a radio access (either a 5G RAT or E-UTRA RAT) to assign each data flow, taking into account e.g., service, traffic characteristics, radio characteristics, and UE’s moving speed.

[PR 5.6.2-003] The 3GPP system shall support UEs with dual radio capability as well as UEs with single radio capability, i.e., a UE that cannot transmit on the 5G RAT and the E-UTRA RAT simultaneously.

[PR 5.6.2-017] The 3GPP system shall be able to support an enhanced service access control mechanism with the following conditions:

- target subscribers (e.g., per PLMN),

- subscriber class (e.g., access class),

- device type (e.g., Smart phone or IoT device),

- service type (e.g., Voice, SMS and/or specific data applications),

- communication type (e.g., emergency call, signalling and/or service origination).

[PR 5.6.2-018] The enhanced service access control mechanism shall be able to provide access to a limited set of services determined by an operator’s policy.

### 7.13.2	3GPP/non-3GPP access integration

[PR 5.6.2-005] The 3GPP system shall be able to provide 3GPP services to the UE using various 3GPP and non-3GPP access networks (e.g., WLAN, fixed broadband access, Bluetooth).

[PR 5.6.2-006]  The3GPP system shall be able to support at least the following 3GPP/non-3GPP access interworking capabilities:

- Inter-system mobility between 3GPP and non-3GPP access networks,

- Capability for the UE to select the 3GPP or non-3GPP access based on operator policy,

- Simultaneous connection to different accesses, and capability for the UE to access the 3GPP services provided by the core network using non-3GPP access (e.g., FMSS).

[PR 5.6.2-007] The 3GPP system shall enable the UE to simultaneously connect to the network via different accesses, both non-3GPP and 3GPP, for one or more applications.

[PR 5.6.2-008] The 3GPP system shall be able to manage the addition or dropping of the various accesses dynamically during a session according to the quality of the individual connections.

[PR 5.6.2-009] The 3GPP system shall be able to support data transmissions optimized for different access technologies (e.g., 3GPP, non-3GPP) for UEs that are simultaneously connected to the network via different accesses (both non-3GPP and 3GPP).

[PR 5.6.2-012] The 3GPP system shall be able to support interworking with a fixed broadband network.

NOTE:	The specification of fixed broadband access network is outside the scope of 3GPP.

[PR 5.6.2-014] Based on operator policy, the 3GPP system shall be able to dynamically offload part of the traffic from 3GPP RAT to non-3GPP RATs and vice versa, taking into account traffic load and traffic type.

[PR.5.6.2-015] The 3GPP network shall be able to integrate fixed and wireless service access management and provide an efficient provision of services over 3GPP and non-3GPP accesses.

[PR.5.6.2-004] The 3GPP system shall be able to dynamically offload part of the traffic to a specific RAN node, taking into account traffic load and traffic type.

### 7.13.3	Satellite access

[PR 5.6.2-021] The 3GPP system shall be able to provide services to the UE using satellite access.

[PR 5.6.2-023] The 3GPP system shall support service continuity between land based and satellite based access networks owned by the same operator or by an agreement between operators.

### 7.13.4	Residential access

[PR 5.6.2-024] The 3GPP system shall support residential deployment with a service limited to a pre-defined geographic area.

### 7.13.5	Access from less trusted networks

[PR 5.6.2-016] Subject to regulatory requirements, the device or UE may use a temporary identifier for initial attach that hides its long-term identity. The home network shall be able to associate this temporary identifier to the long-term identity.

### 7.13.6	Energy Efficiency

[PR 5.6.2-020] The 3GPP system shall be capable of achieving [1000] times energy efficiency compared to legacy system.

### 7.13.7	Performance

These performance related requirements can apply to different scenarios including eMBB, mIOT and CriC.

[PR 5.1.2.4-004][CPR-PM-2] The 3GPP system shall be able to support a stand-alone multicast/broadcast network comprising of individual cells whose coverage radius spans up to 50km.

[PR 5.1.2.6-007] The 3GPP system shall be able to support very large cells (e.g., relaxed timing on random access and other procedures to enable very long range beyond 100km).

[PR 5.6.2-022] The 3GPP system shall support an air-interface with one-way latency of up to 275 ms when satellite connection is involved.

[PR 5.6.2-025] The 3GPP system shall support residential deployment with peak data rates of [10s of Gbps] and experienced data rates of up to [1 Gbps].

[PR 5.6.2-026] The 3GPP system shall support residential deployment with a latency of [10 ms].

## 7.14	Migration and interworking

### 7.14.1	Interworking between 5G systems

[PR 5.7.2.1-001] The 3GPP system shall provide support for a device or UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the device or UE's Home Mobile Network.

[PR 5.7.2.1-001a] A Visited Mobile Network shall provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

[PR 5.7.2.1-002] The 3GPP system shall provide a mechanism for a network operator to limit access to its services for a roaming subscriber with a 5G device or UE and subscription, e.g., based on a specific service logic agreement in a roaming agreement.

### 7.14.2	Interworking between 5G and 4G

[PR 5.7.2.2-001] The 3GPP system shall be able to support seamless handover and Inter System Mobility between 5G RAT(s) and E-UTRA.

### 7.14.3	Migration aspects

[PR 5.7.2.3-001] The 3GPP system shall support all services supported in a 4G system, with the following exceptions:

CS voice service continuity and/or fallback to GERAN or UTRAN,

Seamless handover between the 5G RAT(s) and GERAN or UTRAN, and

Access by a GERAN or UTRAN RAT to a 5G core network.