3GPP TSG-SA WG1 Meeting #75	S1-162093

San Francisco, CA, 22-26 August 2016	(revision of S1-16xxxx)

Title:	SMARTER – Mobility support

Agenda Item:	8.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes text for the SMARTER TS mobility support aspects. The requirements are drawn from the BB TRs.

| ### CPR | ### Requirement | ### Comment |
| --- | --- | --- |
| ### From MIOT |  |  |
| ### [PR.5.3.3.2-007] | ### The 3GPP system shall support a resource efficient mechanism to communicate with a stationary device (e.g., simplified mobility management, lower signalling to user data resource usage ratio). | ### Covered in resource efficiency issues rather than as a diverse mobility management issue. |
| ### [PR.5.3.3.3-001] | ### The 3GPP system shall provide efficient support for devices with restricted range of mobility (e.g., within a warehouse). | ### Merged with [PR 5.3.3.3-002], [CPR-7-003], [PR 5.3.2-004] and from NEO the unnumbered requirement and [PR 5.6.2-029]. The 3GPP system shall support optimized mobility management procedures for devices with varying levels of mobility (e.g., stationary, nomadic, spatially restricted mobility, full mobility). |
| ### [PR.5.3.3.3-002] | ### The 3GPP system shall provide resource efficient support for stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). | ### Merged with [PR 5.3.3.3-001] |
| ### From CRIC - none |  |  |
| ### From eMBB |  |  |
| ### [CPR-7-003] | The 3GPP system shall support service limited to a pre-defined geographic area for e.g. residential deployment. | ### Merged with MIOT [PR 5.3.3.3-001] |
| ### From NEO |  |  |
| ### [PR 5.3.2-001a] | The 3GPP system shall enable operators to define different levels of mobility support for different UEs. | ### Clarified the wording The 3GPP system shall enable operators to specify a level of mobility support (e.g., stationary, nomadic, spatially restricted mobility, full mobility) for a UE or group of UEs. |
| ### [PR 5.3.2-001b] | An operator can choose to support none, any one or some combination of the following: • The 3GPP system shall enable mobility support where packet loss is minimized during inter- and/or intra-RAT cell changes for some or all packet data connections (e.g., APNs) associated with a UE. • The 3GPP system shall enable mobility support where IP address assigned to a UE is maintained across different cells and RATs for some or all packet data connections (e.g., APNs) associated with a UE. • The 3GPP system shall enable mobility support where the impact to the user experience is minimized (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all packet data connections (e.g., APNs) associated with a UE. | ### Separated into 3 individual requirements. The leading sentence is redundant. The 3GPP system shall support different levels of mobility enable packet loss to be minimized during inter- and/or intra-RAT cell changes for some or all connections (e.g., APNs) associated with a device. The 3GPP system shall enable maintaining the IP address assigned to a UE when moving across different cells and RATs for some or all connections (e.g., APNs) associated with a device. ### The 3GPP system shall enable minimizing impact to the user experience (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections (e.g., APNs) associated with a device. |
| ### [PR 5.3.2-002] | The 3GPP system shall support service continuity when changing the IP address and/or IP anchoring point for some or all packet data connections associated with a device or UE. | ### Clarified the wording as The 3GPP system shall enable minimizing impact to the user experience (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections (e.g., APNs) associated with a device. |
| ### [PR 5.3.2-003] | The 3GPP system shall enable operators to update the level of mobility support provided for a UE, (e.g., during packet data connection establishment). | ### Clarified the wording. The 3GPP system shall enable operators to modify the level of mobility support provided for a device, (e.g., during packet data connection establishment). |
| ### [PR 5.3.2-004] | The 3GPP system shall minimize the signaling needed for mobility management of stationary UEs. | ### Merged with MOT [PR 5.3.3.3-001] |
| ### [PR 5.3.2-005] | The 3GPP system shall minimize the signaling needed for mobility management of UEs that participate in mobile-originated communication scenarios only. | ### Clarified the wording. The 3GPP system shall optimize mobility management support for devices that use only mobile originated communications (e.g., paging is not needed). |
|  | The 3GPP system shall minimize the signaling needed for mobility management of UEs with known patterns of mobility (e.g., movement only within a limited geographic area). | ### Merged with MIOT [PR 5.3.3.3-001] |
| ### [PR 5.6.2-029] | The 3GPP system shall support residential deployment with a service limited to a pre-defined geographic area. | ### Merged with MIOT [PR 5.3.3.3-001] |

First Proposed Text Change:

# 6	Basic capabilities

## 6.x	Diverse mobility management

### 6.x.1 Description

A key feature of 5G is support for devices with different mobility management needs.  5G will support devices with a range of mobility management needs, including devices that are

- stationary during their entire usable life (e.g., sensors embedded in infrastructure),

- stationary during active periods, but nomadic between activations (e.g., fixed access),

- mobile within a constrained and well-defined space (e.g., in a factory), and

- completely mobile.

The flexible nature of a 5G system will support different mobility management methods that minimize signaling overhead and optimize access for these different types of devices.

### 6.x.2	Requirements

The 3GPP system shall enable operators to specify a level of mobility support (e.g., stationary, nomadic, spatially restricted mobility, full mobility) for a device or group of devices.

The 3GPP system shall enable operators to modify the level of mobility support provided for a device, (e.g., during packet data connection establishment).

The 3GPP system shall support optimized mobility management procedures for devices with varying levels of mobility (e.g., stationary, nomadic, spatially restricted mobility, full mobility).

The 3GPP system shall optimize mobility management support for devices that use only mobile originated communications (e.g., paging is not needed).

The 3GPP system shall support resource efficient communication with a stationary device (e.g., simplified mobility management, lower signalling to user plane usage ratio).

The 3GPP system shall enable packet loss to be minimized during inter- and/or intra-RAT cell changes for some or all connections (e.g., APNs) associated with a device.

The 3GPP system shall enable maintaining the IP address assigned to a device when moving across different cells and RATs for some or all connections (e.g., APNs) associated with a device.

The 3GPP system shall enable minimizing impact to the user experience (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections (e.g., APNs) associated with a device.