3GPP TSG-SA WG1 Meeting #75	S1-162095

San Francisco, CA, 22-26 August 2016	(revision of S1-16xxxx)

Title:	SMARTER - Migration

Agenda Item:	8.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes text for the SMARTER TS Migration aspects. The requirements are drawn from the NEO TR.

First Proposed Text Change:

# 5	High-level requirements

## 5.x	Migration to 5G

### 5.x.1 Description

The 5G system will support most of the existing 4G capabilities, in addition to many new services.  The EPS service  requirements that will continue to be supported can be found in the documents specified in TS 21.xxx [x].  The few capabilities that will not continue to be supported are identified in the migration requirements.

### 5.x.2	Requirements

#### 5.x.2.1	Interworking between 5G systems

The 5G 3GPP system shall provide support for a device with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the device's Home Mobile Network.

A 5G Visited Mobile Network shall provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G 3GPP system shall provide a mechanism for a network operator to limit access to its services for a roaming device, (e.g., based on service agreement).

The 5G 3GPP system shall provide a mechanism for a network operator to direct a device onto a partner's Mobile Network for the purposes of data offload.

#### 5.x.2.2	Migration aspects

The evolved 5G system shall support all services supported in a EPS, with the following exceptions:

CS voice service continuity and/or fallback to GERAN or UTRAN,

Seamless handover between the 5G RAT(s) and GERAN or UTRAN, and

Access to a 5G core network via GERAN or UTRAN.