3GPP TSG-SA WG1 Meeting #75	S1-162145

San Francisco, CA, USA, 22-26 August 2016	(revision of S1-16xxxx)

Title:	FRMCS use case for interworking with GSM-R

Agenda Item:	9.3 (FS_FRMCS)

Source:	China Academy of Railway Sciences (CARS), Huawei

Contact:	LiuJianning (liujianning.liu@huawei.com)

Laurence Meriau (laurence.meriau@huawei.com)

Abstract: This document proposes a new section to describie a use case of interworking to GSM-R

Discussion

This document lists some use cases supported by both MCX Service and GSM-R.

Interworking use cases between the common services are proposed with potential requirements.

| GSM-R Use case | MCX Service Use case | Interworking use case proposed in this doc |
| --- | --- | --- |
| Point to point call | Private call | Point to Point communication interworking |
| Multi-user communications | Group call Broadcast call | Group communication interworking |
| Location services | Location service | Location service interworking |
| Public emergency call; Railway emergency communication | Emergency group communication; Imminent peril group communication | Emergency communication interworking |
| User presence | MCX UE presence | Presence management for users of the 2 “domains” |
| UE to Server Communication for train control over CS or PS call?(ETCS and CTCS) | UE to Server Communication over LTE | Seamless Handover between the 2 systems. |

Proposal

The following text proposes a use case explaining changes needed to LTE system to support this interworking.

Note: following text is for fully new use cases. Nevertheless, no revision marks have been provided, to allow an easy visibility of changes done during the SA1 meeting week.

---------------------------------------------------- 1ST CHANGE ----------------------------

X Use cases for interworking with GSM-R users

X.1	Group Communication interworking between GSM-R and FRMCS users

X.1.1	Description

This use case supports a group call between MCX Service User(s) and GSM-R User(s).

X.1.2 	Pre-condition

MCX User A and GSM-R User B and C are authorized Users to perform group calls together by the MCPTT administrator in MCPTT Server, in 3GPP scope.

User A and User B and C are in the same group call area defined by the MCPTT Server.

X.1.3	Service flow

User A initiates a new group call (based on location, presence etc, of User B and C).

User A broadcasts group call invitations to all Users in the same area.

User B and C receive the group call invitations.

User B and C accept the invitation and join the group call. Or User B and C UEs automatically join this group call.

X.1.4	Post-condition

User A communicates with User B and User C in a group call.

X.1.5	Potential requirements

[XX.01-001] 3GPP System shall provide a means for a MCPTT Server to define group calls between MCX Users and GSM-R Users.

[XX.01-002] 3GPP System shall provide a means to authorize a MCX User to initiate a group call with GSM-R User(s).

[XX.01-003] 3GPP System shall provide a means for an authorized MCX User to initiate a group call with GSM-R User(s).

X.2	Location Service interworking between GSM-R and FRMCS users

X.2.1	Description

This use case allows MCX Server to obtain and share with MCX Users the location information of GSM-R User(s).

X.2.2 	Pre-condition

User A is an authorized GSM-R User.

User A is affiliated to a group that consists of MCX user(s) and GSM-R User(s).

X.2.3	Service flow

MCX server sends a location request to the specific User A.

User A accepts and return his location information as requested.

X.2.4	Post-condition

The GSM-R User’s location information is sent to MCX Server.

X.2.5	Potential requirements

[XX.2-001] MCX System shall provide a means for MCX Server to obtain and share with MCX Users the location information of GSM-R User(s).

X.3	Emergency group Communication between GSM-R and FRMCS users

X.3.1	Description

This use case allows authorized MCX User(s) to inform other user(s) subscribed to GSM-R of an emergency situation, and vice versa.

X.3.2 	Pre-condition

User A is an authorized MCX Service User.

User B and C are authorized to GSM-R Users.

User A detects an emergency situation that may threaten his life.

X.3.3	Service flow

User A initiates a group communication with GSM-R User B and C for emergency data.

User A sends the emergency information to User B and C.

User B and C receive the emergency information prior to other communication.

X.3.4	Post-condition

The emergency information is sent to User B and C.

X.3.5	Potential requirements

[XX.03-001] 3GPP System shall be able to transport MCX data from an authorized MCX User to a group of GSM-R User(s) with prioritisation of emergency MCX data compared to other data.

X.4	Presence interworking between GSM-R and FRMCS users

X.4.1	Description

This use case allows MCX user(s) to see the status of GSM-R user(s), and vice versa. For example, MCX user(s) can see the status of a GSM-R user, including user ID, states (available, busy, etc.)

X.4.2 	Pre-condition

User A is an authorized MCX Service User.

User B is an authorized GSM-R Users.

GSM-R detects the change status of User B

X.4.3	Service flow

MCX Service receives the User B’s changed status.

MCX service sends the changed status to User A.

X.4.4	Post-condition

The changed status of User B is presented in User A.

X.4.5	Potential requirements

[XX.04-001] MCX service shall provide a means for an MCX user to present the dynamic status of GSM-R User, including user ID, states (e.g., available, busy, etc.), etc.

X.5	Point to Point communication between GSM-R and FRMCS users

X.5.1	Description

This use case allows a MCX user to communicate with a GSM-R User.

X.5.2 	Pre-condition

User A is an authorized MCX User.

User B is an authorized GSM-R Users.

X.5.3	Service flow

User A initiates a point to point call to User B.

User B accepts the call and joins the call.

X.5.4	Post-condition

User B and User A are communicating with each other.

X.5.5	Potential requirements

[XX.05-001] 3GPP System shall provide transport of point to point data transfer for an authorized MCX user to a GSM-R User.

X.6	Seamless handover between GSMR and LTE

Editor’s Note: this is FFS

----------------------------------------------------- END OF CHANGES ---------------------