3GPP TSG-SA WG1 Meeting #75	S1-162155

# San Francisco, CA, USA, 22-26 August 2016 									(revision of S1-16xxxx)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.864 | CR | 0003 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interworking between 5G and non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SMARTER-NEO |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In the current TR, the requirements of interworking include interworking between 5G systems and interworking between 5G RAT(s) and E-UTRA. Another important requirment of interworking between 5G RAT(s) and non-3GPP access(e.g, WLAN access is steered towards the EPC) haven’t been considered in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the descriptions and potential requirements for interworking between 5G RAT(s) and non-3GPP access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of interworking are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1, 5.7.2.X, 7.14.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************************************First change************************************************

5.7	Migration and interworking

5.7.1	Description

A 5G system is expected to support many new types of UEs and services. In order to efficiently support these new UEs and services, it is expected that the 5G system will include one or more RATs optimized for different purposes (e.g., support for high reliability/low latency vs support for low mobility/higher latency). In the 5G timeframe of 2020 and beyond, it is also anticipated there will be a need to support co-existence of 5G RAT(s) and E-UTRAN, co-existence of 5G RAT(s) and certain non-3GPP access(s) (e.g, WLAN access is steered towards the EPC).

In addition to new services derived from the new use cases envisioned for the 5G system, there exist already today a large number of services in the cellular networks of the current generations. It is envisioned that most of these existing services could be deemed as required for support in a 5G system while others are not.

5.7.2	Potential Requirements

5.7.2.1	Interworking between 5G systems

[PR 5.7.2.1-001] The 3GPP system shall provide support for a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE's Home Mobile Network. A 5G Visited Mobile Network shall provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

[PR 5.7.2.1-002] The3GPP system shall provide a mechanism for a network operator to limit access to its services for a roaming subscriber with a 5G UE and subscription, e.g., based on a specific service logic agreement in a roaming agreement.

[PR 5.7.2.1-003] The 3GPP system shall provide a mechanism for a network operator to direct a UE to onto a partnership Mobile Network for the purposes of data offload.

5.7.2.2	Interworking between 5G RAT(s)and E-UTRA

[PR 5.7.2.2-001] The 3GPP system shall be able to support seamless handover and Inter System Mobility between5G RAT(s) and E-UTRA.

5.7.2.X	Interworking between 5G RAT(s) and non-3GPP access

[PR 5.7.2.X-001] The 3GPP system shall be able to support handover and Inter System Mobility between 5G RAT(s) and certain non-3GPP access(s) (e.g, WLAN access is steered towards the EPC).

5.7.2.3	Migration aspects

[PR 5.7.2.3-001] The 3GPP system shall support all services supported in a 4G system, with the following exceptions:

-	CS voice service continuity and/or fallback to GERAN or UTRAN,

-	Seamless handover between the 5G RAT(s) and GERAN or UTRAN, and

-	Access by a GERAN or UTRAN RAT to a 5G core network.

***********************************End of First Change********************************************

***********************************Second Change***********************************************

7.14	Migration and interworking

7.14.1	Interworking between 5G systems

[PR 5.7.2.1-001] The 3GPP system shall provide support for a device or UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the device or UE's Home Mobile Network.

[PR 5.7.2.1-001a] A Visited Mobile Network shall provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

[PR 5.7.2.1-002] The 3GPP system shall provide a mechanism for a network operator to limit access to its services for a roaming subscriber with a 5G device or UE and subscription, e.g., based on a specific service logic agreement in a roaming agreement.

7.14.2	Interworking between 5G RAT(s) and E-UTRA

[PR 5.7.2.2-001] The 3GPP system shall be able to support seamless handover and Inter System Mobility between 5G RAT(s) and E-UTRA.

7.14.X	Interworking between 5G RAT(s) and non-3GPP access

[PR 5.7.2.X-001] The 3GPP system shall be able to support handover and Inter System Mobility between 5G RAT(s) and certain non-3GPP access(s) (e.g, WLAN access is steered towards the EPC).

7.14.3	Migration aspects

[PR 5.7.2.3-001] The 3GPP system shall support all services supported in a 4G system, with the following exceptions:

CS voice service continuity and/or fallback to GERAN or UTRAN,

Seamless handover between the 5G RAT(s) and GERAN or UTRAN, and

Access by a GERAN or UTRAN RAT to a 5G core network.

************************************End of Second change*****************************************

************************************End of changes**********************************************