3GPP TSG-SA WG1 Meeting #75	S1-162312

San Francisco, CA, USA, 22-26 August 2016	(revision of S1-162111)

Title:	Normative requirements for eMBB

Agenda Item:	8.1 SMARTER

Source:	Huawei

Contact:	Laurence Meriau (laurence dot meriau at huawei dot com)

Abstract: This document updates SMARTER TS 22.xxx for eMBB scenarios

Summary

The performance requirements in eMBB TR 22.863 are mostly about high data rates and high traffic densities.

The potential performances values are issued from the eMBB TR, with the following comments:

Analysis of consolidated potential requirements and needed updated

| [CPR-7-001] | For wide area coverage, the system shall support a minimum experienced data rate per user for mobile broadband services anytime and anywhere, 100Mbps, for pedestrian users (up to [10 km/h]) and users in vehicle (up to 120 km/h), except for very high connection density from150000/km2 (e.g. stadium) | This is the wide-area scenario Needed updates are - Add uplink data rate requirement: NGMN always proposes 50% less for UL than for DL, i.e. DL 50 Mbits/s and UL 25 Mbits/s - This scenario is expected for wide area/country side, NGMS’s connections density is 400/km2 in suburban area and 100/km2 in rural area - Use NGMN values for Traffic density: - for sub-urban: 20Gbps/km2 DL and 10 Gbps/km2 UL - for rural: 5Gbps/km2 DL and 2.5 Gbps/km2 UL For UEs in vehicles, the UE can be connected to the network directly, or via an on-board moving base station. Proposal - do the above updates and a Note 1 about moving base stations. |
| --- | --- | --- |
| [CPR-7-002] | The 3GPP system shall enable support up to 100% geographic area as part of wide area coverage (one way shall be via satellite access). Note that the 100% geographic area refers to the entire area allowed to be covered by the spectrum license and could be regional or global. | Not relevant for a table on bit rate or traffic density KPIs that is proposed here. Proposal - address elsewhere |
| [CPR-7-003] | The 3GPP system shall support service limited to a pre-defined geographic area for e.g. residential deployment. | Not relevant for the table on bit rate or traffic density KPIs but the scenario needs to be included in the office scenario and its performance KPIs. Proposal - include in the indoor scenario [CPR-7-005], and address the pre-defined geographic area aspect elsewhere |
| [CPR-7-004] | For pedestrian users in area with connection density lower than [200/km2] (e.g. in office, in residential, streets), the 3GPP system shall support the peak data rate up to 10 Gbps DL. | Proposal - ignore |
| [CPR-7-005] | For indoor users, in high connection density scenario (75000/km2) (e.g. in hotspot), the 3GPP system shall support user experienced data rate up to 1 Gbps DL and 500 Mbps UL, resulting into traffic density of 112,5 Tbps/km2 (UL+DL). | This is the classical office indoor scenario, but it applies also to home offices and residential deployments even if the connection densities are lower for those scenarios. NB! The data rates are minimum data rates, to allow the user to get the service in line with NGMN definition of experienced data rate (see Note 2). The traffic density proposed by NGMN is different and is not a product of data rate with connection/kms as NGMN consider different UEs activity factors (different UEs use different services, with different data rates, Note 3). NGMN values are: 15Tbps/km2 for DL + 2 Tbps/km2 for UL. Proposal - replace with NGMN values, and add the home and residential aspects |
| [CPR-7-006] | For pedestrian users in a crowded area with very high density ([150000/km2] connection density, like in stadium with [30000] users per stadium) the 3GPP system shall provide user experienced data downlink rate of 25 Mbps and uplink rate of 50Mbps, resulting into traffic density of [12 Tbps/km2] (UL+DL). | This is crowded area (stadium and concert scenario) Traffic density proposed by NGMN is instead 3.75Tbps/km2 DL and 7.5 Tbps/km2 UL Proposal - use NGMN values |  |
| [CPR-7-007] | The 3GPP system shall support the traffic density in the area at least the level of Tbps/ km2 to support case of traffic for pedestrian users and users in urban vehicle (up to 60 km/h), in 200-2500 /km2 connection density. | Urban Broadband scenario NGMN proposes traffic density requirement for this scenario (cf. NGMN 5G White Paper) and traffic density to be 750Gbps/km2 DL and 125 Gbps/km2. and user experienced data rate as 300 Mbps DL and 50 Mbps UL. For UEs in vehicles, the UE can be connected to the network directly, or via an on-board moving base station. Proposal - add the potential requirements and a Note 1 about moving base stations. |
| [CPR-7-008] | For pedestrian users in area with connection density lower than 200/km2 (e.g. in office, residential, streets), the 3GPP system shall be able to support experienced broadcast DL data rate up to 300 Mbps (e.g. video streams such as 4k UHD or 8k UHD). | This is the broadcast scenario. The requirement is not aligned with the NGMN requirement and contains some obvious errors. The following updates are needed - The DL bit rate requirements is vague, and not aligned with the NGMN value of up to 200 Mbps per channel (so this is the maximum expected) - UL rate is expected modest (e.g. 500 kbps) - The user speed excludes many valid users (in-car and in-vehicle users). NGMN suggests up to 500 km/h - Connection or traffic density is N/A for broadcast mode Proposal - do the necessary clarifications as above |
| [CPR-7-009] | For pedestrian users in area with connection density lower than 200/km2 (e.g. in office, streets), the 3GPP system shall be able to support at least [15] broadcast channels of [20Mbps] each simultaneously over the same carrier. The current speed limits the service to pedestrians.] NOTE: 4k UHD: 3840 x 2160, 50FPS, HEVC, 20~30 Mbit/s (Medium quality), 4k UHD: 3840 x 2160, 50FPS, HEVC, ~75Mbit/s (High quality), 4k UHD: 3840 x 2160, 50FPS, AVC ~ 150Mbit/s (High quality), 8K 7680*4320, 50FPS, HEVC, ~300Mbps (High quality) | This is complementary for Broadcast scenario to increase quality with improvement compared to current 4G capabilities (10 channels FullHD of 12 Mbps) Proposal: update Connection density column with these numbers |
| [CPR-7-010] | For pedestrian users in area with connection density lower than 200/km2 (e.g. in office, residential, streets), the 3GPP system shall be able to provide UE to UE (via network) roundtrip latency of 2-4ms on the radio layer between: - the low data rate in UL and corresponding high data rate video stream in DL. - the low data rate in DL and corresponding high data rate video stream in UL. The requirement for the video steam shall be capable of running an 8k 3D DL video streaming [300Mbps] for uplink (unicast) and downlink, for e.g. virtual meeting usage.. | This is irrelevant for Broadcast scenario. It is unicast. |
| [CPR-7-011] | The 3GPP system shall support mobile broadband via direct connection or via on-board base station relaying, to users in fast moving road vehicles up to 200 km/h and trains 500 km/h and user experienced data rate up to 50 Mbps per user at DL and 25 Mbps per user at UL, whatever user access directly to the network or via a base stations relaying data transfer for all users in the fast moving vehicles. | This is high speed scenario to cover cars and trains needs everywhere. The NGMN requirements are for car at 250 km/h and 500 km/h for trains. NGMN proposes connectivity density 2000 km2 and traffic density 100 Gbps/km2 DL and 50 Gbps/km2 for this scenario. Proposals: - change vehicle's speed to 250 km/h - Add a Note 1 about moving base stations - It is expected to have this everywhere as 5G V2X needs 100% network coverage - Add the connectivity density and traffic density values |
| [CPR-7-012] | The 3GPP system shall support mobile broadband via direct connection or via on-board base station relaying, to users in airplanes (up to 1000 km/h) and user experienced data rate up to 15 Mbps per user at DL and 7.5 Mbps per user at UL), whatever user access directly to the network or via a base stations relaying data transfer for all users in the airplane. | This is airplane scenario. NGMN proposes connectivity density 80 per plane, 60 airplanes/18000 km2 and traffic density 1.2 Gbps/plane and 600 Mbps/plane for this scenario. This is well aligned with the NGMN requirements Proposal - add a Note 1 about moving base stations Add the connectivity density and traffic density values |
| [CPR-7-013] | The 3GPP system shall be able to provide the mobile broadband in fast moving road vehicles and trains (traffic density of up to 100 Gbps / km2 at DL, and 50 Gbps / km2 at UL). | This is well aligned with the NGMN requirements Proposal - merge with [CPR-7-011] |
| [CPR-7-014] | The 3GPP system shall be able to provide fast moving airplanes connectivity (traffic of up to 1.2 Gbps / airplane at DL and 600 Mbps / plane at UL). | This is well aligned with the NGMN requirements Proposal - merge with [CPR-7-011] |
| [CPR-7-015] | The 3GPP System shall be efficient and flexible for transmission of both small and large amounts of data (e.g., streaming video) from the same device. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-016] | The 3GPP system shall minimize signalling overhead needed for transmission of small amounts of data. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-017] | The 3GPP system shall be able to deliver the aggregate throughput with any traffic type, of the FTTx/xDSL and Next Generation Radio access in both UL and DL directions to all traffic types and 3rd party applications, with very high reliability, very high availability. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-018] | The 3GPP system shall support the use of common user equipment (e.g. router hub at customer premises) that support Next Generation Radio and fixed broadband access. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-019] | The 3GPP system shall support dynamic and static network address allocation to the common user equipment over Next Generation Radio and fixed broadband access. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-020] | with re-wording: The 3GPP system shall support the option for Operators to provide the same level of security and support regulatory requirements and accounting over the FTTx/xDSL link as is provided over the Next Generation Radio access link. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-021] | with changes: The 3GPP system shall minimize overhead for data transfer. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-022] | The 3GPP system shall support use of common user equipment (e.g. Femtocell at customer premises) that supports Next Generation Radio, WiFi and fixed broadband access. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-023] | The 3GPP system shall support convergence of fixed broadband access and mobile network (e.g. 3GPP access (Femtocell, Microcell & Macrocell) and other access technologies including Wifi), with support of service specific policies (e.g. QoS and traffic management policies), unified set of identities for a single user. This allows to grant that user access to a single set of services (e.g. VAS, parental controls, content, content optimisation and access to local services when appropriate) when using cellular, fixed or cellular over fixed broadband access. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-024] | The 3GPP system shall support seamless service continuity during mobility between Macrocell to Femtocell, Femtocell to Macrocell and Femtocell to Femtocell. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |
| [CPR-7-025] | The 3GPP system shall support the end user accessing local area network services (e.g. peripherals, content servers etc.) when connected to the Small Cell at the Customer Premises. | Not relevant for the table on bit rate or traffic density KPIs. Proposal - address elsewhere |

Proposal

Propose normative requirements for SMARTER TS in line with the above table.

------------------------- 1ST CHANGE ----------------------------

# 7	Performance requirements

## 7.x	High bit Data rates and traffic densities

Several scenarios require transport of very high bit rates or traffic densities by the 3GPP system. In some cases a minimum data rate must be specified, in other cases the targeted maximum is set. The scenarios address different operating coverage: full network or wide-area, hotspots (e.g., office), city-wide, massive gatherings (e.g., stadium), and special deployments (e.g., residential). The scenarios and their performance requirements can be found in table 7.x-1.

-	Wide-area - The wide-area scenario covers the overall requirement for the full network, excluding extreme scenarios that are covered separately

-	Office and home - The office and home scenario covers office and home hot spots, and residential deployments

-	Pedestrian crowd - The pedestrian crowd scenario covers very dense crowds, for example, at stadiums or concerts. In addition to a very high connection density the users want to share what they see and hear, putting a also requirement on the uplink

-	Urban broadband - The urban broadband scenario covers stationary and slowly moving users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas The passengers in vehicles can be connected either directly or via an onboard base station to the network.

-	Broadcast - The broadcast scenario covers stationary and users in vehicle, also in fast trains, watching broadcast content The passengers in vehicles can be connected either directly or via an onboard base station to the network.

-	High-speed vehicle - The high-speed vehicle scenario covers road vehicles and trains. The passengers can be connected either directly or via an onboard base station to the network.

-	Airplane - The airplane scenario covers airplane passengers. The passengers can be connected either directly or via an onboard base station to the network.

Table 7.x-1 Performance requirements in high-bit rate and high-traffic density scenarios.

| Scenario | User experienced bit data rate (DL) (Note2) | User experienced bit data rate (UL) (Note2) | Traffic density (bits/s) | Connection density | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- |
| Wide-area | 50 Mbps | - Mbps | For sub-urban: 20 Gbps/km2 DL and 10 Gbps/km2 UL For rural: 5Gbps/km2 DL and 2.5 Gbps/km2 UL | 400/kms for suburban area; 100/km2 in rural area | Pedetrian or users in Vehicule (up to 120 km/h | Full network (NOTE1) |
| Office and home | 1 Gbps | 500 Mbps | Expected 15Tbps/km2 for DL + 2 Tbps/km2 for UL (Note 3) | 75.000/km2 | Pedestrian | Indoor, and residential |
| Pedestrian crowd | 25 Mbps | 50 Mbps | 3.75Tbps/km2 DL and 7.5 Tbps/km2 U | 150.000/km2 | Pedestrian | Hotspot (crowded area) |
| Urban broadband | 300 Mbps | 50 Mbps | 750Gbps/km2 DL and 125 Gbps/km2 | 200-2.500/km2 | Pedestrian and users in vehicle (up to 60 km/h) | City-center NOTE 1 |
| Broadcast | Max 200 Mbps (per channel) | N/A (expected modest (e.g. 500kbps per user)) | N/A | [15] broadcast channels of [20Mbps] each simultaneously over the same carrier with | Pedestrian and users in vehicle (up to 500 km/h) | Full network (NOTE1) |
| High-speed vehicle | 50 Mbps | 25 Mbps | 100 Gbps (DL)/km2 50 Gbps (UL)/km2 | 2000/km2 | Users in cars (up to 250 km/h) In trains (up to 500 km/h) | Along roads everywhere (full network) and railways (NOTE 1) |
| Airplane | 15 Mbps | 7,5 Mbps | 1,2 Gbps/plane (DL) 600 Mbps/plane (UL) | ## 80 per plane, ## 60 airplane/ 18000 km2 | Users in airplanes. Up to 1000 km/h | NOTE 1 |

NOTE 1: For UEs in vehicles, the UE can be connected to the network directly, or via an on-board moving base station. NOTE2 From NGMN: The user experienced data rate requirement depends on the targeted application/use case. It is set as the minimum user experienced data rate required for the user to get a quality experience of the targeted application/use case. NOTE3: NGMN proposed that Traffic Density measured in bit/s/m2 is defined as the total amount of traffic exchanged by all devices over the considered area and is given by the product: [required user experienced data rate] x [required connection density]. But NGMN values are not in line with this product as NGMN for office and home scenario that considers different UEs activity factors (different UEs use different services, with different data rates)

-------------------- END OF CHANGES ---------------------