3GPP TSG-SA WG1 Meeting #75	S1-162335

San Francisco, CA, 22-26 August 2016	(revision of S1-162099)

Title:	SMARTER - Charging

Agenda Item:	8.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes text for the SMARTER TS Charging aspects. The requirements are drawn from NEO clause 6. One additional requirement is introduced to support charging based on service KPIs from CRIC and eMBB.

First Proposed Text Change:

# 9	Charging aspects

The following requirements apply for both home and roaming cases.

The 3GPP core network shall support collection of charging information for alternative authentication mechanisms.

The 3GPP core network shall support collection of charging information associated with each serving MNO when multi-network connectivity is used under the control of the home operator.

The 3GPP core network shall support charging for services in an operator's hosted service environment.

The 3GPP core network shall support charging for content delivered from a data cache.

The 3GPP core network shall support collection of charging information based on the access type (e.g., 3GPP, non-3GPP).

The 3GPP core network shall support charging based on service KPIs (e.g., latency, reliability, availability, bandwidth).