3GPP TSG-SA WG1 Meeting #75	S1-162351

San Francisco, CA, USA, 22-26 August 2016	(revision of S1-162102)

Title:	Clarification split of eV2X requirements between LTE and NR

Agenda Item:	9.2

Source:	Huawei, Telecom Italia

Contact:	Holger Rosier holger.rosier@huawei.com

Guanglin HAN hanguanglin@huawei.com

Abstract: This document proposes a metric to enable SA1 to distinguish performances KPIs for LTE from performances KPIs for NR.

Discussion

According to eV2X SID, eV2X will define performance requirements expected to cover LTE RAT and/or new 3GPP RAT e.g. NR [1].

- Distinction between LTE and NR requirements is important to be discussed in order to determine in which specification the normative requirements have to be pushed in.

- In addition, we consider that it is important to be able to clarify to Stage2/3 WGs when a service can be fulfilled with lower, less stringent KPIs and higher, more stringent KPIs.

In our view, latency and reliability are the key and most essential requirements for V2X communication, to be considered as most important parameters to cover critical and safety relevant message exchange between vehicles. Industrial consortia [2] [3] and research projects [4] [5] confirm our assumption.

So we propose in this document to use these two parameters namely latency and reliability as main differentiator of use cases to identify the RAT supporting each use case.

Note: Other KPIs considered in SA1 use cases are also important but look to be less essential, e.g. message size will be adapted in the application layer to reduce throughput requirements while still achieving the minimum application safety level. Furthermore, the communication range is an important parameter too, but the application will notice the number of vehicles in communication range and adapts the velocity consequently to ensure safety requirements.

Table 1 below summarizes current V2X use cases covered in eV2X study item TR 22.abc [1].

Table 1: Use Cases and Requirements

|  |  | Use Cases |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5.1 eV2X support for Vehicle Platooning | 5.2 Information exchange within platoon | 5.3 Automotive: Sensor and State Map Sharing | 5.4 eV2X support for Remote Driving | 5.5 Automated Cooperative Driving for Short distance Grouping |  | 5.6 Collective Perception of Environment |  |  |
|  |  |  |  |  |  | Set I | Set II | Set I | Set II |  |
| Requirements | Latency | 10ms (E2E) | TBD | 10ms (E2E) | 5ms (E2E) | 25ms | 10ms (E2E) | 100ms (E2E) | 3ms |  |
|  | Reliability | N/A | N/A | 95% | 99.999% | 99% | 99.99% | 99% | 99.999% |  |
|  | Range | TBD | N/A | N/A | N/A | N/A | N/A | N/A | 1km | 50m |
|  | Throughput | TBD | TBD | TBD | 5Mbps DL, 50 Mbps UL | N/A | N/A | N/A | 50 Mbps | 1Gbps |
|  | Mobility | N/A | N/A | N/A | 250 km/h | N/A | N/A | N/A | N/A |  |
|  | Message Size | 50B – 1200B | TBD | N/A | N/A | 300B-400B | TBD | 1600B | N/A |  |
|  | Density | 5 UEs/Group | N/A | 15840 vehicles/ mile | N/A | N/A | N/A | N/A | N/A |  |
| LTE or NR |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |

Table 2 below correlates latency and reliability values to show what is expected to be possible with LTE RAT and NR RAT.

- SA1 debated but could not conclude where LTE RAT reaches its limit either by 90% reliability with 10ms latency or 99% reliability with 5ms latency

- 3GPP NR would be needed in our view for more stringent latency and reliability requirements.

|  | NR 5.6 Set II |  |  | 5.3 LTE |  | 5.6 Set I 5.5 Set I |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5.5 Set II 5.4 |  |  |  |

Table 2: Metric for Use Case split

| Latency /Reliability | 1ms | 2ms | 3ms | 5ms | 10ms | 25ms | 100ms |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90% | NR 5.6 Set II |  |  | LTE 5.3 |  |  |  |
| 99% |  |  |  |  |  | 5.5 Set I 5.6 Set I |  |
| 99.9% |  |  |  | 5.5 Set II 5.4 |  |  |  |
| 99.99% |  |  |  |  |  |  |  |
| 99.999% |  |  |  |  |  |  |  |

NOTE: 	Reliability requirement in eV2X use cases relates to the application layer. Translation in to radio layer reliability remains for further study. Targeted RAT shown in Table 2 might change to LTE and/or NR dependant on the radio layer reliability.

Conclusion

The proposed metric enables SA1 to distinguish KPIs for LTE from KPIs for NR.

Nevertheless, we consider that the discussion cannot be completed in SA1, so the proposed metric should be checked by RAN WG as well.

It is proposed to:

- Clarify for each agreeable Release 15 requirement whether the content belongs to set 1 KPIs for LTE or set 2 KPIs for NR based on latency and reliability.

- Double check SA1 KPIs and sets with RAN WG.

Proposal for the eV2X TR

The text below is proposed for the “Consideration” section of eV2X TR. The decision is open to SA1 floor.

----------------------------- START OF CHANGE -----------------------

6.1	Considerations on performances for LTE or NR

It is important to distinguish sets of performances with lower, less stringent KPIs from higher, more stringent KPIs for each use case if possible. It is important to define which set of KPIs are for Release 15 LTE or NR respectively.

Two sets of KPIs should be proposed in the consolidation of requirements for a use case to enable RAN to decide which RAT is targeted withSet 1 and Set 2.

Latency and reliability are differentiators to identify the RAT supporting the use cases’ KPIs.

Some of the KPIs defined are end-to-end requirements, e.g. reliability on the application layer etc., RAN WGs have to evaluate to which RAT each set belongs.

|  | NR |  |  | LTE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Table 6.1-X: Metric to distinguish LTE and NR KPIs

| Latency /Reliability | 1ms | 2ms | 3ms | 5ms | 10ms | 25ms | 100ms |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90% | NR |  |  | LTE |  |  |  |
| 99% |  |  |  |  |  |  |  |
| 99.9% |  |  |  |  |  |  |  |
| 99.99% |  |  |  |  |  |  |  |
| 99.999% |  |  |  |  |  |  |  |

----------------------------- END OF CHANGE -----------------------

Reference

[1] TR 22.abc, (FS_eV2X) "Study on Enhancement of 3GPP support for V2X services" [Rel-15]

[2] NGMN Alliance, NGMN 5G White Paper, March 2015

[3] GSMA Network 2020, "Unlocking Commercial Opportunities From 4G Evolution to 5G", 
[available online], GSMA, http://www.gsma.com/network2020, [accessed July, 2016]

[4] P. Popovski, "Ultra-reliable communication in 5G wireless systems," 5G for Ubiquitous Connectivity (5GU), 2014 1st International Conference on, Akaslompolo, 2014, pp. 146-151

[5] B. Holfeld et al., "Wireless Communication for Factory Automation: an opportunity for LTE and 5G systems," in IEEE Communications Magazine, vol. 54, no. 6, pp. 36-43, June 2016.

[6] METIS Project Deliverable D1.1. (April 2013). Scenarios, Requirements and KPIs for 5G Mobile and Wireless System. [available online],  https://www.metis2020.com/ [accessed July, 2016]

[7] NGMN Alliance White Paper, “Perspectives on Vertical Industries and Implications for 5G”, [online], Available: https://www.ngmn.org/uploads/media/160610_NGMN_Perspectives_on_Vertical_Industries_and_Implications_for_5G_v1_0.pdf , [Accessed July, 2016]

[8] H2020, FANTASTIC-5G project, “Deliverable D2.1. Air interface framework and specification of system level simulations”, http://fantastic5g.eu, April 2016