3GPP TSG-SA WG1 Meeting #75	S1-162393

San Francisco, US, 22-26 August 2016

(revision of S1-162079, S1-162371)

Title: 	eV2X use case for Teleoperated Support (TeSo)

Agenda Item:	9.2 (FS_eV2X)

Source:	Huawei

Contact:	Holger Rosier holger.rosier@huawei.com

Laurence Meriau laurence.meriau@huawei.com

Abstract: This use case describes 3GPP impact to support centralised Teleoperated Support (TeSo) enabling remote control of  automated vehicles.

# References

[1] A. Kwoczek (Volkswagen AG), “Automotive Requirements for Future Networks”, Information Technology Society (ITG), Communication Technology, May, 2016

[2] NGMN Alliance, White Paper: “Perspectives on Vertical Industries and Implications for 5G”, [online], available: https://www.ngmn.org/uploads/media/160610_NGMN_Perspectives_on_Vertical_Industries_and_Implications_for_5G_v1_0.pdf , [accessed July, 2016]

[3] C. Rousseau (Renault), “5G Connectivity Requirements for Automotive Sector”, 3rd Annual 5G Huddle, Wireless World Research Forum, London, April, 2016

# Proposed additions to the eV2X TR

The following changes are proposed for eV2X TR.

Note: this is a fully new use case but no revision marks have been used, with the goal to get a better visibility of changes done during SA1 week.

************************************* Start of change *****************************

## 5.x	Teleoperated Support (TeSo)

### 5.x.1	Description

While traffic safety as well as accident-free driving is the task of each connected autonomous vehicle, Teleoperated Support (TeSo) enables a single human operator to remotely control autonomous vehicles for a short period of time. TeSo enables efficient road construction (control of multiple autonomous vehicles from a single human operator), snow plowing e.g.

### Remote control of vehicle (TeSo) has the following requirements on 3GPP network for V2X communication:

### End-to-End latency less than 20ms for fast vehicle control and feedback [1], [2]

### Uplink and downlink data rate 15Mbps for exchanging application related control and command messages [1], [3] via 3GPP V2X communication service

### Reliability of 99.999%, necessary to avoid application malfunctions [1], [2]

### 5.x.1.1	Pre-conditions

- Vehicle A is able to drive autonomously and coordinate driving manoeuvres via 3GPP V2X communication service.

- Vehicle A supports TeSo on the application layer (out of 3GPP), it can be in a far end server or in a server hosted by another UE close or not close to the first UE, see Figure 1.

Figure 1: TeSo Concept Description

### 5.x.1.2	Service Flows

- Vehicle A sends TeSo application messages (camera data, sensor data, status data, confirmation etc.) via 3GPP network to the remote operator.

- Remote operator sends TeSo application command messages via 3GPP network to vehicle A.

### 5.x.1.3	Post-conditions

### TeSo terminates 3GPP V2X communication service, if no further support from remote operator is needed

### 5.x.2	Potential Requirements

### [PR-5.x.2-001]	The 3GPP network shall enable communication between a UE and a server (e.g. far end server or a server hosted by another UE), to exchange messages to support [15Mbps] in uplink and downlink direction with less than [20ms] end-to-end latency and [99.999%] reliability.

************************************* End of change *****************************