3GPP TSG-SA WG1 Meeting #75	S1-162289

San Francisco, CA, USA, 22-26 August 2016	(revision of S1-162145)

Title:	FRMCS use case for interworking with GSM-R

Agenda Item:	9.3 (FS_FRMCS)

Source:	China Academy of Railway Sciences (CARS), Huawei,Kapsch CarrierCom,Nokia, KRRI

Contact:	LiuJianning (liujianning.liu@huawei.com)

Laurence Meriau (laurence.meriau@huawei.com)

Abstract: This document proposes a new section to describie a use case of interworking to GSM-R

Discussion

This document lists some use cases supported by both FRMCS System (3GPP MCX service) and GSM-R.

Interworking use cases between the common services are proposed with potential requirements.

| GSM-R Use case | MCX Service Use case | Interworking use case proposed in this doc |
| --- | --- | --- |
| Point to point call | Private call | Point to Point communication interworking |
| Multi-user communications | Group call Broadcast call | Group communication interworking |
| Location services | Location service | Location service interworking |
| Public emergency call; Railway emergency communication | Emergency group communication; Imminent peril group communication | Emergency communication interworking Note: it is FFS |
| User presence | MCX UE presence | Presence management for users of the 2 “domains” |
| UE to Server Communication for train control over CS or PS call?(ETCS and CTCS) | UE to Server Communication over LTE | Seamless Handover between the 2 systems. Note: it is FFS |

Proposal

The following text proposes a use case explaining changes needed to FRMCS system to support this interworking.

Note: following text is for fully new use cases. Nevertheless, no revision marks have been provided, to allow an easy visibility of changes done during the SA1 meeting week.

---------------------------------------------------- 1ST CHANGE ----------------------------

X Use cases for interworking with GSM-R users

X.1	Area  Broadcast Group Communication interworking between GSM-R and FRMCS users

X.1.1	Description

This use case describes an area broadcast group communication between FRMCS User(s) and GSM-R User(s), and vice versa.

X.1.2 	Pre-condition

FRMCS User A and GSM-R User B and User C are authorized Users to perform group communication together by the FRMCS system.

User A and User B and User C are in the same group communication area defined by the FRMCS system.

X.1.3	Service flow

FRMCS system to GSM-R

FRMCS User A initiates a new area broadcast group communication (based on location, presence etc, of  GSM-R User B and C).

FRMCS User A broadcasts group communication invitations to all Users in the same area.

GSM-R User B and User C receive the group communication invitations.

GSM-R User B and User C accept the invitation and join the group communication. Or GSM-R User B and User C automatically join this group communication.

GSM-R to FRMCS

GSM-R User B initiates a new group call (based on location, presence etc, of FRMCS User A and GSM-R User C).

GSM-R User B broadcasts group call invitations to all Users in the same area.

FRMCS User A and GSM-R User C receive the group call invitations.

FRMCS User A and GSM-R User C accept the invitation and join the group call. Or FRMCS User A and GSM-R User C automatically join this group call.

X.1.4	Post-condition

FRMCS User A, GSM-R User B and User C can communicates among each other in a group call.

X.1.5	Potential requirements

[XX.01-001] FRMCS System shall provide a means to initiate group communication between FRMCS Users and GSM-R Users.

[XX.01-002] FRMCS system shall provide means to FRMCS users to join the group calls of GSM-R.

- These two requirements are not covered by 3GPP existing specification.

X.2	Location Service interworking between GSM-R and FRMCS users

X.2.1	Description

This use case allows FRMCS system and GSM-R system to obtain and share the location information of their users..

X.2.2 	Pre-condition

User A is an authorized FRMCS User.

User B is an authorized GSM-R User.

User A and User B are affiliated to a group that consists of FRMCS User(s) and GSM-R User(s).

X.2.3	Service flow

FRMCS system to GSM-R

FRMCS User A sends a location request to the specific GSM-R User B.

GSM-R User B accepts and return his location information as requested.

GSM-R to FRMCS system

GSM-R User B sends a location request to the specific FRMCS User A.

FRMCS User A accepts and returns his location information as requested.

X.2.4	Post-condition

FRMCS User A’s location information is sent to GSM-R User B.

GSM-R User B’s location information is sent to FRMCS User A.

X.2.5	Potential requirements

[XX.2-001] FRMCS System shall provide a means to obtain and share with FRMCS User the location information of GSM-R User(s).

[XX.2-002] FRMCS system shall provide means to provide location information of FRMCS user to GSM-R.

- These two requirements are not covered by 3GPP existing specification.

X.3	Emergency group Communication between GSM-R and FRMCS users

Editor’s Note: this is FFS

X.4	Presence interworking between GSM-R and FRMCS users

X.4.1	Description

This use case allows FRMCS User(s) to see the status of GSM-R user(s), and vice versa. For example, FRMCS User(s) can see the status of a GSM-R user, including user ID, states (available, busy, etc.), vice versa.

X.4.2 	Pre-condition

User A is an authorized FRMCS User.

User B is an authorized GSM-R Users.

GSM-R system detects the change status of GSM-R User B

FRMCS system detects the change status of FRMCS User A

X.4.3	Service flow

FRMCS system to GSM-R

FRMCS System receives the GSM-R User B’s changed status.

FRMCS System sends the changed status to FRMCS User A.

GSM-R to FRMCS system

GSM-R receives the User A’s changed status.

GSM-R sends the changed status to User B

X.4.4	Post-condition

The changed status of GSM-R User B is presented in FRMCS User A.

The changed status of FRMCS User A is presented in GSM-R User B.

X.4.5	Potential requirements

[XX.04-001] FRMCS System shall provide a means for an FRMCS User to present the presence status of GSM-R User, including user ID, states (e.g., available, busy, etc.), etc.

[XX.04-002] FRMCS System shall provide a means to share the presence status of FRMCS User(s) to GSM-R, including user ID, states (e.g., available, busy, etc.), etc.

- These two requirements are not covered by 3GPP existing specification.

X.5	Point to Point communication between GSM-R and FRMCS users

X.5.1	Description

This use case allows a FRMCS User to communicate with a GSM-R User, vice versa.

X.5.2 	Pre-condition

User A is an authorized FRMCS User.

User B is an authorized GSM-R Users.

X.5.3	Service flow

FRMCS system to GSM-R

FRMCS User A initiates a point to point communication to GSM-R User B.

GSM-R User B accepts the communication and joins the communication.

GSM-R to FRMCS system

GSM-R User B initiates a point to point call to FRMCS User A.

FRMCS User A accepts the call and joins the call.

X.5.4	Post-condition

GSM-R User B and FRMCS User A are communicating with each other.

X.5.5	Potential requirements

[XX.05-001] FRMCS System shall provide a means for point to point communication between an authorized FRMCS User and a GSM-R User,

- This requirement is not covered by 3GPP existing specification.

X.6	Seamless handover between GSM-R and FRMCS system

Editor’s Note: this is FFS

----------------------------------------------------- END OF CHANGES ---------------------