3GPP TSG-SA WG1 Meeting #75	S1-162481

San Francisco, CA, 22-26 August 2016	(revision of S1-162346)

(revision of S1-162310)

(revision of S1-162095)

Title:	SMARTER - Migration

Agenda Item:	8.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell, KPN

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes text for the SMARTER TS Migration aspects. The requirements are drawn from the NEO TR.

First Proposed Text Change:

# 5	High-level requirements

## 5.x	Migration to 5G

### 5.x.1 Description

The 5G system supports most of the existing EPS services, in addition to many new services. The existing EPS services may be accessed using the new 5G access technologies even where the EPS specifications might indicate E-UTRA(N) only. Only new or changed service requirements for new or changed services are specified in this TS. The few EPS capabilities that are not supported by the 5G system are identified in clause 5.x.2.2 below.

### 5.x.2	Requirements

#### 5.x.2.1	Interworking between 5G systems

The 5G system shall provide support for a device with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the device's Home Mobile Network.

The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming device, (e.g., based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane traffic over the partnership network, subject to a bilateral agreement between the operators being in place.

#### 5.x.2.2	Legacy Service Support

The 5G system shall support all EPS capabilities (e.g., from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173) with the following exceptions:

- CS voice service continuity and/or fallback to GERAN or UTRAN,

- Seamless handover between the 5G RAN and GERAN or UTRAN, and

- Access to a 5G core network via GERAN or UTRAN.

Editor's Note: 5G will be replaced with the appropriate brand name when it becomes available.

Editor's note: device can be changed to UE if we can agree on a common definition